3GPP TSG-WG SA2 Meeting #169	 S2-2505352

19 - 23 May, 2025, Fukuoka, Japan

Source:	HONOR

Title:	Update to clause 5.7 Identifiers

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Update to clause 5.7: Identifiers and add the general description about the AIoT device temporary identifier.

1. Discussion

SA WG3 has concluded to support the requirement on AIoT Device privacy and solution principle is using “Temporary ID”. The TR 33.713-0.8.0 clause 7.3 Conclusion on KI #3 shows:

The following aspects and principles are agreed for the conclusion on KI#3

-	A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported.

-	Mechanism shall allow unambiguous identification of the AIoT device.

-	A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

-	The network and the device shall support a mechanism for the use of temporary ID.

Hence, SA WG2 needs to support the AIoT device temporary ID based solution from architectural perspective in the normative phase.

Generally, the following principles should be considered in SA WG2 during the normative phase:

- The network and the AIoT device shall support a mechanism for the use of AIoT device temporary ID.

- The AIoT Device privacy principle concluded by SA3 is that AIoT device ID is protected based on the use of AIoT device temporary ID. When the AIoT device temporary ID is available both in the AIoT device side and the network side, the AIOTF shall use the AIoT device temporary ID instead of the AIoT device permanent identifier to trigger the AIoT Reader to perform AIoT service operations towards the AIoT Device(s).

- According to TR 33.713, the AIoT device temporary ID is a privacy preserving identifier containing the concealed the AIoT device permanent identifier. The AIoT device temporary ID is generated in the AIoT device and the CN locally based on the security mechanism or assigned by the CN. Regardless of how the temporary ID is generated, the AIoT device temporary ID is used to uniquely identify an AIoT device in the MNO domain.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change (all new texts) * * * *

5.7.x	AIoT Device Temporary Identifier

The 5G system supports allocation of the AIoT device temporary identifier in order to support the AIoT device permanent identifier privacy protection. The AIoT device temporary identifier is a privacy preserving identifier containing the concealed the AIoT device permanent identifier. An AIoT device temporary identifier is used to uniquely identify an AIoT device in the MNO domain.

Editor’s Note:	How the AIoT device permanent identifier is concealed will be determined by SA WG3.

If the AF provides the AIoT device permanent identifier(s) in the AIoT service operation request, the AIOTF should use the AIoT device temporary identifier(s) to trigger the AIoT Reader to perform AIoT service operations towards the AIoT Device(s).

Editor’s Note:	How the AIoT device temporary identifier is allocated/reallocated needs to align with SA WG3.

# * * * * End of changes * * * *