3GPP TSG-WG Meeting #169	S2-2505577

# Fukuoka, Japan, May 19 - 23, 2025											revision of S2-2504609

Source:	Rakuten Mobile, NIST

Title:	Pseudo-CR on New Solution Key Issue #1: System Architecture to Support Sensing

Document for:	Approval

Agenda item:	20.2.1

Spec:	3GPP TR 23.700-14

Version:	0.1.0

Work Item:	FS_Sensing_ARC / Rel-20

# 1. Introduction/Discussion

This paper proposes a solution to Key Issue #1, System Architecture to Support Sensing. We propose a new Network Function “Sensing Function” for sensing services.

# Text Proposal

It is proposed to agree the following key issue(s) into TR 23.700-14.

* * * First Change * * * *

## 6.X 	Solution #1: Architecture enhancement to support Sensing          Services

### 6.X.0	High-level solution principles

A new NF called “Sensing Function” is introduced for service operations and procedures to support Integrated Sensing and Communication.

Integrated Sensing and Communications as a new feature requires 5GS to support authorization and revocation for a particular sensing service, Sensing entity(ies) discovery and (re-)selection, collect and transfer sensing data, sensing result exposure, parameter provisioning of sensing entities etc. To meet these requirements, the current 5G System (5GS) as specified in TS 23.501 [X] must be further enhanced.

### 6.x.1	Description

This solution addresses Key Issue # 1 and provides detailed AF service request on top of call flow as described in [x]

This solution proposes:

- that a new NF is introduced in 5G core network to address the following functionalities:

- register itself to NRF

- authorize the sensing request from Consumer NF, AF

- sensing entities discovery and selection

- sensing measurement data collection and sensing result calculation

- sensing result exposure

- NEF is enhanced to receive sensing request and expose results to authorized 3rd parties.

Figure 6.x.1-1 describes the architecture of this solution.

Figure 6.x.1-1: Architecture assumption to support Sensing Services

### 6.x.2	Procedures

Figure 6.x.2-1: Procedure for Sensing related information collection and exposure

1. 		The AF triggers subscription of NEF on sensing function related information with Nnef_EventExposure_Subscribe Request, in the request, AF provides the subscription information including sensing related information such as purpose of sensing, target area granularity (e.g. geographical areaetc), Sensing QoS requirement, reporting mode (defines whether the report is to be provided on event basis or periodically), reporting threshold (if the reporting mode is per event basis) and Time period (if the reporting mode is periodic).

2.	Upon receiving the request, the NEF authorizes the AF for usage of Sensing Service.

3. 	The NEF discovers and selects the Sensing Function(s)

4. 	The NEF sends the subscription request to Sensing Function containing information received in Step 1. with Nsf_EventExposure_Subscribe Request

5. 	The Sensing Function may perform further authorisation for the sensing request based on parameters provided by AF.

6.	 If the Sensing Function performs authorisation, Sensing Function confirms the request to NEF with Nsf_EventExposure_Subscribe Response

7. 	The NEF notifies the AF whether the subscription request is authorised or rejected with Nnef_EventExposure_Subscribe Response

8. 		The Sensing Function performs the Sensing Entity discovery and selection according to the requirement in the sensing service description, optionally the AMF selection.

Editor note:	Whether AMF is selected is FFS

9.	The Sensing Function exchanges any relevant configurations/adjustments and coordinates with the Sensing Entity (e.g., parameters such as sensing area, sensing operation period and sensing operation time window etc.)

10. 	The Sensing Entity initiates the sensing and measurements and the Sensing data collection.

11. 	The Sensing Entity reports the sensing measurement data according to the configuration to the Sensing Function

12.  	The Sensing Function receives the sensing measurement data and derives the Sensing result.

13.  	The Sensing Function reports the Sensing result to NEF with Nsf_EventExposure_Subscribe_Notify

14.	  The NEF reports the sensing result from the Sensing Function to the AF with Nnef_EventExposure_Notify.

### 6.x.3	Impacts on services, entities and interfaces

Sensing Function: A new NF is defined and the functionalities are captured as description of clause 6.x.1.

Sensing Entity: Perform the sensing operation based on received parameter from Sensing Function and provide the Sensing data.

NEF：

-	There is an impact on existing Nnef_EventExposure_Subscribe service.

-	NEF is enhanced to receive sensing request and expose results to authorized 3rd parties.

AF:

-	Request Sensing service

-	Receive Sensing result

* * * End of Changes * * * *