3GPP TSG-WG SA2 Meeting #169 	S2-2506056

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-2505925, S2-2505680, S2-2505669, S2-2505385)

# Source:	Apple (Moderator)

# Title:	New SID on Short Message Service to Emergency Response Centre

# Document for:	Approval

# Agenda Item:	30.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Short Message Service to Emergency Response Centre

Acronym: FS_SMS2EC_Ph2_ARC

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| SMS2EC_Ph2-REQ | SA1 | 1070052 | Stage 1 of SMS to Emergency Centre Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

3GPP specifications support Emergency Calls with voice and other media, but this does not include SMS.

Service requirements for Short Message Service to Emergency Response Centre have been specified in Rel-19 in clause 36 of TS 22.101.

# 4	Objective

The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101.

The study includes the following work tasks:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

NOTE 1:	Depending on the type of emergency service, Short Messages can be routed to different emergency response centres.

NOTE 2:	This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages.

For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item.

The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis.

SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 | 1.5 | 1 | No | WT-1 is self-contained |
| WT-2 | 0.5 | 0.5 | No | WT-2 is self-contained |
| WT-3 | 2 | 2 | No | WT-3 is self-contained |

Total TU estimates for the study phase: 4

Total TU estimates for the normative phase: 3.5

Total TU estimates: 7.5

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.abc-de | Study on Short Message Service to Emergency Response Centre | TSG#109 (2025-09) | TSG#110 (2025-12) |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Security aspects will be analysed by SA3.

# 9	Supporting Individual Members

Supporting IM name

Apple

Nokia

ViaSat

Ericsson

Qualcomm

Google

Skylo Technologies

Sateliot

Novamint

Iridium Satellite

EchoStar

Orange

Verizon

ZTE

Aalyria

SyncTechno Inc.

EDF

AT&T

BT

CATT

Erillisverkot

EENA