3GPP TSG-SA2 Meeting #169	S2-2505721

# Fukuoka, Japan, 19 – 23 May 2025		(was S2-2504800)

Source: 	Transsion Holdings, OPPO

Title: 	KI#1: New Solution – UE-side model training data collection via DCCF and AIRF in CN over UP

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 to support UE-side model training data collection via CN over UP.

# 1	Discussion

This paper proposes a solution to support UE data collection via DCCF and AIRF over UP for UE-side model training.

The DCCF and ADRF have been introduced to 5GC since R17. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection (DCCF discovery principles are defined in clause 6.3.19 of TS 23.501 [2]). And ADRF is allowed by 5G System architecture to store and retrieve the collected data, analytics and ML Model(s). The DCCF may determine the ADRF and interact directly or indirectly with the ADRF to request or store data or analytics.

Figure 4.2.1-1 in TS23.288 shows architecture with DCCF and ADRF for Analytics, Collected Data and ML Model(s) as below,

While DCCF  could be re-used for the control of UE-side model training data collection coordination and storage, a new NF named AI data Repository Function (AIRF) is introduced to store UE-side model training data

The approved SID in SP-250413 shows that WT#1 is to study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

In TR 38.843, it clearly states that “For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection. Only for model inference, when required data comes from other entities, there is a latency requirement for data collection.” . Online model training is not mentioned in TR38.843. Actually, online trained model usually cannot be used due to no model validation and model testing and even not needed.

The nature of AI model training is to extract patterns from huge amount of data over a certain probability distribution through mathematical algorithms such as neural networks and decision trees, forming generalized model(s). In order to get the right pattern(s) to form correct generalized model(s), the collected data for UE-side model training should has the following characteristics,

- No latency requirement, offline collection is applicable

- Huge amount

- From UEs, but non UE specific

Note: Data collected for model inference is out of scope of this paper and latency requirement need to be considered for model inference.

# 2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

# Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #x> | <Key Issue #y> |
| #x | x |  |

Next Change (all new text)

## 6.X	Solution #X: UE-Side model training data collection via DCCF and AIRF in CN over UP

### 6.X.1	High level principles

The following principles for this solution are proposed as below:

- New AIRF (AI data Repository Function) is introduced to store UE-side model training data.

- The 5GC-centric variant is used:

-	AF requests UE-side model training data from DCCF

-	DCCF checks whether the requested UE collected data is already stored in AIRF or not. If the requested UE collected data is available in AIRF, DCCF exposes the data from AIRF to AF. If the requested UE collected data is not available in AIRF, DCCF sends requests to UEs to collect data.

-	DCCF informs UE(s) the AIRF address. UEs establish the UP connection via UPF to AIRF.

-	UE(s) collects UE-side model training data and sends them to AIRF.

- 	AIRF sends the received data(together with historical collected data if available) to DCCF, DCCF further sends to AF.

### 6.X.2	Description

This solution is for Key Issue#1 for UE data collection over UP.

The DCCF has been introduced to 5GC since R17. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection (DCCF discovery principles are defined in clause 6.3.19 of TS 23.501 [2]).

In this release, a new functionality AIRF is introduced, together with DCCF to collect and store UE-side model training data from UE according to data consumer’s request, wherein the data consumer could be entity in CN or AF (e.g. OTT server) to train UE-side ML model. Data consumer requests UE-side model training data from DCCF, if the AF (e.g. OTT server) is an untrusted AF, the data collection request should be transferred to the DCCF via NEF. The DCCF check whether the UE-side model training data is available in AIRF, if the UE-side model training data is available in AIRF, DCCF retrieves the data from AIRF and sends the data to the data consumer. If the UE-side model training data is not available in AIRF, then DCCF starts to collect data from UEs, UEs collect and send collected data to AIRF through user plane, AIRF may send the data to DCCF after the required data is ready, DCCF purges and sends the UE-side model training data to the data consumer.

Editor’s note: What is the different between the AIRF and ADRF and whether and how the AIRF can be used for the UE data collection is FFS.

### 6.X.3	Procedures

#### 6.x.2.1 UE-side mode training data collection via DCCF and AIRF in CN over UP

Figure 6.X.2.1-X: UE-side model training data collection via DCCF and AIRF in CN over UP

- The Data consumer (AF) requests UE training data from DCCF, if the AF (e.g. OTT server) is an untrusted AF, the data collection request should be transferred to the DCCF via NEF. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection. The data consumer requests data via the DCCF, using Ndccf_DataManagement_Subscribe service operation.

- The DCCF sends a request to the AIRF, using Nairf_DataManagement_RetrievalSubscribe (UE-side model training data Specification, Notification Target Address=DCCF) service operation.

- The AIRF responds to the DCCF with an Nairf_DataManagement_RetrievalNotify response indicating whether the AIRF can supply the UE-side model training data. If the UE-side model training data can be provided, the procedure continues with step 11, otherwise step 4 to 10 is performed.

- The DCCF sends request to NG-RAN(s) via AMF to collect UE-side mode training data.

- NG-RAN(s) sends the requests to a group of UEs.

Editor’s Note: How the group of UEs are determined is FFS.

- UEs collect UE-side model training data.

- UEs establish PDU session(s).

- UEs utilize user plane with the established PDU session(s) to transfer UE-side model training data to AIRF.

- The AIRF merges and restructures and stores UE-side model training data together with historically collected UE training data.

- The AIRF sends the DCCF with an Nairf_DataManagement_RetrievalNotify response indicating the AIRF can supply the UE-side model training data is ready.

- The DCCF fetches the UE-side model training data from AIRF and perform purging to clean the UE label and other sensitive information.

- The DCCF sends the purged UE training data to notification endpoint (data consumer) via Ndccf_DataManagement_Notify.

### 6.X.4	Impacts on services, entities and interfaces

DCCF:

-	Sends data collection request to NG-RAN via AMF to collect UEs data;

-	Allocates and sends UP information for data collection to NG-RAN via AMF.

-  DCCF purges the UE training data to clean the UE label and other sensitive information

-  DCCF sends the data to data consumer.

.

AIRF:

-	Receives data collected by the UEs via UP connection from the UEs and stores the collected data in AIRF;

-  AIRF merges, restructures and stores UE training data together with historically collected UE training data.

.

AMF:

-  Receives request(s) including data type, UP information from DCCF and send the request(s) to NG-RAN

.

NG-RAN:

-  Receives request(s) including data type, UP information from AMF and sends the requests to a group of UEs or all UEs in the cell

.

UE:

-	Receives UP information for data reporting originated from DCCF, and establishes UP connection with the DCCF to report data;

-	Receives data collection request originated from DCCF, and collects corresponding data;

- 	Sends the collected data via the UP connection to the DCCF.

End of Changes