3GPP TSG-SA2 Meeting #170	S2-2506357

Goteborg, Sweden, 25th Aug 2025 – 29th Aug 2025	is a revision of S2-2505134

Source:				CEWiT

Title:			Solution for KI#5 on Sensing Result Exposure for 5GA

Document for:				Approval

Agenda Item:				20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The contribution proposes a solution for Key Issue #5: Sensing Result Exposure

- Introduction

This paper presents a solution that enables the 5G System (5GS) to support the exposure of the computed Sensing Results.

- Text Proposal

It is proposed to agree the following changes in 3GPP TR 23.700-14.

# 6	Solutions

## 6.0			Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| #X |  |  |  |  | X |  |

* * * * Start of Changes * * * *

- 6.X	Solution #X: Sensing Result Exposure for 5GA

- 6.X.0	High-level Solution Principles

For the Sensing feature to be supported in the network, a NF namely "SeMF (Sensing Management Function)" is introduced to control and manage sensing service operations.

The SeMF can receive requests and expose the computed Sensing Results to,

-	AF(s) authorized by NEF, allowing the AF(s) to leverage real-time sensing information.

-	AF(s), NF(s) in Core network

- 6.X.1	Description

The proposed solution addresses Key Issue #5 on Sensing Result Exposure. It describes the procedure for Sensing Result Exposure along with the key parameters present in the sensing exposure request and response.

### 6.X.1.x	Sensing Result Exposure

The following attributes may be present in the request of the sensing service consumer:

-	AF ID

-	Target Sensing Area

-	Cell ID(s), Tracking Area

Editor's Note:	It is FFS if the Tracking area definition for sensing would be redefined.

-	Reference Co-ordinates

-	Azimuth / Horizontal

-	Zenith / Vertical

-	Reference Co-ordinate system (Universal, Cartesian)

-	Distance

-	Angle

-	Sensing Service Type(s) (Object Detection, Object Localization, Object Tracking, Environment Monitoring etc.,)

-	Sensing Time Window(s)

-	Object(s) dimensions

-	Object(s) status i.e. Stationary, Moving

-	Object Characteristic Type

-	Object Properties (e.g. RCS value of interest, metallic object, aerial object etc.)

-	Object Filter Type (e.g. Human, metallic object (small/large), vehicle (small/large), Aerial etc.)

-	Object(s) Velocity

-	Azimuth / Horizontal

-	Zenith / Vertical

Editor’s Note: The sub-categorization of object velocity is FFS and is subject to RAN inputs.

-	Type of Event Reporting

-	One time, Periodic, Event Triggered

-	Maximum number of Event Reports

-	Duration of Event Reporting

-	Quality Indication of Sensing Result

-	Expected Confidence of Sensing Result

-	Expected Accuracy of Sensing

-	Sensing Result Response Time i.e., expected sensing response time

The following attributes may be present in the sensing service response:

-	Object Presence Detection i.e., Yes or No

-	Sensed object count

-	Object Characteristic Type

-	Object Properties (e.g. RCS value of interest, metallic object, aerial object etc.)

-	Object Filter Type (e.g. Human, metallic object (small/large), vehicle (small/large), Aerial etc.)

-	Object(s) status i.e. Stationary, Moving

-	Dimensions of sensed object(s)

-	Sensed Object(s) Location

-	Cell ID(s), Tracking Area

Editor's Note:	It is FFS if the Tracking area definition for sensing would be redefined.

-	Reference Co-ordinates

-	Azimuth / Horizontal

-	Zenith / Vertical

-	Reference Co-ordinate system (Universal, Cartesian)

-	Distance

-	Angle

-	Quality Indication of Sensing Result

-	Confidence of Sensing Result

-	Accuracy of Sensing

-	Velocity of sensed object(s)

-	Azimuth / Horizontal

-	Zenith / Vertical

Editor’s Note: The sub-categorization of object velocity is FFS and is subject to RAN inputs.

-	Time Stamp of Sensing measurement information

### 6.X.2	Procedures

The following procedure as in Figure 6.X.2-1 provide architecture support for sensing result exposure.

Figure 6.X.2-1: Procedure to support Sensing Result Exposure

1.	The request for a sensing service may come from an AF authorized by NEF, AF or any other NF.

1a-1.	An AF sends a request to the NEF by invoking the Nnef_SensingExposure_Subscribe service. The request may include the AF ID and other parameters as listed above.

1a-2.	The NEF authorizes the request from the AF and performs SeMF selection and discovery either based on local configuration or by querying the NRF based on the parameters in AF request. To the selected SeMF, the NEF invokes the Nsemf_Sensing request service carrying the AF request.

1b.		A AF/NF performs SeMF selection and discovery either based on local configuration or by querying the NRF. To the selected SeMF, the AF/NF invokes the Nsemf_Sensing request service. The request may include the parameters as listed above.

2.	Based on the sensing area and other parameters in the request, the SeMF authorizes the sensing request.

After successful authorization, the sensing entity selection is performed by the SeMF and the sensing measurement is collected by interacting with sensing entities. Using the collected data, the sensing result is computed.

3.	Based on the request received in step 1, the SeMF reports the sensing results to the requesting entity. The report contains parameters as listed above.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts may be present in the following entities for sensing result exposure,

SeMF:

-	Support for sensing request processing and sensing result exposure to AF/NF.

NEF:

- Support sensing request processing and result exposure to authorized AF(s).

* * * * End of Changes * * * *