SA WG2 Meeting #170	S2-2506505

25 – 29 Aug, 2025, Goteborg, SE

Source:	MediaTek Inc.

Title:	[WT#1.1, all topics] Control Signalling Design for 6G

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution addresses scoping of WT#1.1 on Control Signalling Design for 6G covering Arch. Motivation, Gap analyses and expected Technical Impact and suggests some potential scope for relevant Key Issue(s).

# 1. Justifications

The control signalling in 5G (incl. NAS) is responsible for signalling and control functions between UE and the core network (5GC). The control signalling layer manages several procedures

- Registration Management (RM)

- Connection Management (CM)

- Mobility Management (MM)

- Session Management (SM)

- Reachability Management and Paging

- Authentication, Security Management, and Integrity Protection

- Transport of UE Policy, SMS, LCS

To enable above procedures, the NAS relies on several network functions, e.g., AMF, SMF, PCF, AUSF, UDM…

Out of above, AMF has a focal role in 5GC NAS Design:

- Terminates N1 interface (UE < > AMF)

- Terminates N2 interface (RAN < > AMF)

- Handles RM, CM, MM, Reachability Management and Paging

- Handles authentication, security management & integrity protection (in coordination with AUSF, UDM, etc.)

- Handles routing of NAS control signalling messages incl. MM, SM, UE Policy, SMS.

- Handles routing of signalling for some beyond connectivity services (e.g., LCS signalling or service authorization for other services)

Architectural Motivation:

Existing 5GS control signalling and NAS design has been optimized and perfected over several releases of 5G from Rel-15 to Rel-20.

For connectivity / legacy services, leveraging the existing principles as far as possible can ensure simplest path to 6G, also providing long term viability for several vertical sectors.

For beyond connectivity / new services, different possibilities for exchanging control messages between UE and CN can be explored.

Gap Analysis:

Having said above, some possible enhancements and simplifications are possible:

Based on existing 5GS design, AMF (in addition to several roles it plays) handles routing of all NAS messages between UE and other NFs in 5GC. As a result, addition of any new information element (IE) or NAS functionality within other NFs (e.g., SMF or PCF) may require updating AMF IEs or procedures.

So, a potential direction to pursue is whether and how to decouple routing of NAS control signalling messages from AMF while keeping the design principles as close as possible to 5GS.

Based on existing 5GS design, AMF (in addition to several roles it plays) handles routing of some control messages for beyond connectivity services (e.g., LCS signalling or service authorisation for other services). As a result, addition of any new IE or feature update for such non-connectivity services (e.g., for LCS) may require updating AMF IEs or procedures.

So, a potential direction to pursue is whether and how to define mechanism(s) for UE-Network interactions to support such beyond connectivity services that is not necessarily routed via AMF. Note such mechanisms(s) needs to be coordinated with any Data Management Framework that will be developed as part of WT#5 concerning services like AI or Sensing or WT#6 concerning compute framework.

Technical Impact:

Assuming the 5GS design as the baseline, possible impact expected on

- How to route control messages for connectivity, legacy services (i.e., NAS) between UE and CN

- How to route control messages for beyond connectivity, new services between UE and CN

- Termination point of interfaces between UE and CN (e.g., N1 and N2)

- NAS architecture and system procedures (e.g., registration, connection, mobility, session management, etc.)

- Where and how to handle authentication, security management and integrity protection

**** First Change ****

# Annex A.X. WT#1.1 Scope

Extracted below from SP-250806:

1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

This WT focuses on studying the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

In particular, this WT addresses at least the below versus 5GS design as the baseline:

- How to route control messages for connectivity, legacy services (i.e., NAS) between UE and CN

- How to route control messages for beyond connectivity, new services between UE and CN

- Impact on termination point of interfaces between UE and CN

- Define NAS architecture and system procedures (e.g., registration, connection, mobility, session management, etc.)

- Where and how to handle authentication, security management and integrity protection

NOTE 1: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 2: UE-CN interactions for beyond connectivity services to be coordinated with KIs related to WT#5, WT#6.

NOTE 3: Potential security and privacy impact to be coordinated by SA3.

**** Second Change ****

# 5.X. Key Issue #X: Control Signalling Design for 6G

This Key Issue focuses on studying the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

In particular, this key issue addresses at least the below versus 5GS design as the baseline:

- How to route control messages for connectivity, legacy services (i.e., NAS) between UE and CN

- How to route control messages for beyond connectivity, new services between UE and CN

- Impact on termination point of interfaces between UE and CN

- Define NAS architecture and system procedures (e.g., registration, connection, mobility, session management, etc.)

- Where and how to handle authentication, security management and integrity protection

NOTE 1: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 2: UE-CN interactions for beyond connectivity services to be coordinated with KIs related to WT#5, WT#6.

NOTE 3: Potential security and privacy impact to be coordinated by SA3.

**** End of Changes ****