SA WG2 Meeting #170	S2-2506625

25 – 29 August, 2025, Goteborg, SE

Source:	LG Electronics

Title:	[WT#1.2, Exposure Framework] Scope and New Key Issue for Exposure framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes scope refinement and new KI for network exposure framework in WT#1.2.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Regarding the following WT#1.2 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01. Specifically, this contribution focuses on network exposure framework aspects.

1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation: As 5G network does, 6G network can support network exposure services to provide various information such as UE related events, network events, analytics, etc to exposure service consumers as well as to enable external functions to provision various parameters such as UE related parameters (e.g. UE level, PDU Session level, etc), network configuration parameters, service specific parameters, etc. Various information can be exposed externally (i.e. towards trusted 3rd party and untrusted 3rd party via NEF or directly) and internally (i.e. towards core network function).

Gap Analysis: In 5G, the network exposure features and mechanisms have been defined and continuously evolving (e.g. as specified in TS 23.502 clause 4.15). So, over several releases, more information that can be exposed has been defined and new procedures/mechanisms have been defined.

For 6G, whether any enhancement is needed to existing procedures/mechanisms specified for 5G, and if any, how to enhance them can be investigated and studied.

Technical Impact: This WT/KI would influence the following to support network exposure framework:

- System architecture to support network exposure framework including indirect interaction with AF (e.g. via NEF) and direct interaction with AF

- Interaction between core network functions, between core network function and RAN node, between core network function and AF

# * * * * Start of 1st Change * * * *

# !! All New Texts !!

A.X	WT#1.2 Scope: Network Exposure Framework

WT#1.2.x:	Study whether and how to support and/or enhance network exposure framework including providing various information such as UE related events, network events, analytics, etc to exposure service consumers (i.e. externally to 3rd party, internally to CN functions) as well as enabling external functions to provision various parameters such as UE related parameters (e.g. UE level, PDU Session level, etc), network configuration parameters, service specific parameters, etc.

NOTE:	This WT requires coordination with SA3.

# * * * * Start of Next Change * * * *

# !! All New Texts !!

5.X	Key Issue #X: Network Exposure Framework

As 5G network does, 6G network can support network exposure services.

This Key Issue aims to study whether and how to support and/or enhance network exposure framework, considering the following:

-	providing various information such as UE related events, network events, analytics, etc to exposure service consumers. Various information can be exposed externally (i.e. towards trusted 3rd party and untrusted 3rd party via NEF or directly) and internally (i.e. towards core network functions).

-	enabling external functions to provision various parameters such as UE related parameters (e.g. UE level, PDU Session level, etc), network configuration parameters, service specific parameters, etc. Various information can be exposed externally (i.e. towards trusted 3rd party and untrusted 3rd party via NEF or directly) and internally (i.e. towards core network functions).

NOTE:	This KI requires coordination with SA3.

# * * * * End of Changes * * * *