SA WG2 Meeting #170	S2-2506973

25th – 29th August, 2025, Goteborg, Sweden

Source:	ETRI

Title:	New Solution: Conveying Emergency Service Type in SMS over NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for WT#2 of FS_SMS2EC_ARC.

1. Introduction

This contribution proposes a solution for supporting emergency service type for SMS over NAS.

Although the contents are entirely new for this solution, they are presented using track-change notation against the baseline procedures of TS 23.502 for clarity and ease of review.

2. Proposal

It is proposed to agree the following updates to TR 23.700-65.

*** Start of Changes (all new text) ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[r1]	3GPP TS 23.040: “Technical realization of the Short Message Service (SMS)”

[r2]	3GPP TS 23.540: “5G System: Technical realization of Service Based Short Message Service; Stage 2”

*** Next Change ***

6	Solutions

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# |  |  |  |  |

*** Next Change ***

6.X	Solution #X: Conveying Emergency Service Type in SMS over NAS

6.X.0	High-level solution principles

The solution is based on existing SMS over NAS architecture in TS 23.501 and TS 23.502 and emergency service principles in TS 23.167.

The UE includes the emergency service type information as a new Information Element (IE) in the SMS.

6.X.1	Description

The UE detects the emergency service type information as a new Information Element in the TPDU of SMS. Depending on the emergency service type information, Short Messages can be routed to different emergency response centres.

The following shows the structure of a Mobile Originated (MO) Short Message, consisting of CP-DATA / RP-DATA / TPDU/SMS-SUBMIT), with reference to TS 24.011 and TS 23.040, and illustrates the new IE in the TPDU.

CP-DATA: SM-Control Protocol User Data

- RP-DATA:	SM-Relay Protocol User Data

- RP-OA: RP-Originator Address

- RP-DA: RP-Destination Address

- RP-UD: RP-User Data

- TPDU: TP Data Unit (SMS-SUBMIT)

- TP-DA: TP-Destination-Address

- TP-EST: the new IE for TP-Emergency-Service-Type

- TP-UD: TP-User-Data

Editor's note:	Name and Position of new IE in the TPDU, are FFS.

6.X.2	Procedures

Figure 4.13.3.3-1: MO SMS over NAS

NOTE 1:	In the case of emergency SMS, the SC represents the PSAP (Public Safety Answering Point).

1.	The UE performs domain selection for UE originating SMS as defined in clause 5.16.3.8 of TS 23.501 [2] if SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send uplink SMS message, then UE and network perform the UE Triggered Service Request procedure firstly as defined in clause 4.2.3.2 to establish a NAS signalling connection to AMF.

2a.	The UE builds the SMS message to be sent as defined in TS 23.040 [r1] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an indication indicating that the NAS message is for SMS transporting. The UE may include service type information for the emergency SMS (e.g. police, ambulance, fire brigade) as a new Information Element (IE) in the TPDU.

2b.	The AMF forwards the SMS message and SUPI to the SMSF serving the UE over N20 message by invoking Nsmsf_SMService_UplinkSMS service operation. In order to permit the SMSF to create an accurate charging record, the AMF adds the IMEISV, the current UE Location Information (ULI) of the UE as defined in clause 5.6.2 of TS 23.501 [2] and if the UE has sent the SMS via 3GPP access, the local time zone.

2c.	The SMSF invokes Namf_Communication_N1N2MessageTransfer service operation to forward SMS ack message to AMF.

2d.	The AMF forwards the SMS ack message from the SMSF to the UE using downlink unit data message.

3-5.	The SMSF checks the SMS management subscription data. If SMS delivery is allowed, the procedure defined in TS 23.040 [r1] or TS 23.540 [r2] applies.

6a-6b.	The SMSF forwards the submit report to AMF by invoking Namf_Communication_N1N2MessageTransfer service operation which is forwarded to UE via Downlink NAS transport. If the SMSF knows the submit report is the last message to be transferred for UE, the SMSF shall include a last message indication in the Namf_Communication_N1N2MessageTransfer service operation so that the AMF knows no more SMS data is to be forwarded to UE.

NOTE:	The behaviour of AMF based on the "last message indication" is implementation specific.

If the UE has more than one SMS message to send, the AMF and SMSF forwards SMS /SMS ack/submit report the same way as described in step 2a-6b.

6c-6d.	When no more SMS is to be sent, UE returns a CP-ack as defined in TS 23.040 [r1] to SMSF. The AMF forwards the SMS ack message by invoking Nsmsf_SMService_UplinkSMS service operation to SMSF.

6.X.3	Impacts on services, entities and interfaces

UE:

-	indicates the service type information as a new IE in payload container in NAS.

*** End of Changes ***