3GPP SA WG2 Meeting #170	S2-2507093

# Goteborg, Sweden, 25 – 29 August 2025                                           (revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0042 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Reader Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the RAN3 LS (S2-25xxxxx/R3-253802), RAN3 provides the following agreements related to A-IoT Area: The new A-IoT Area is encoded as an A-IoT Area ID. A-IoT Area ID = PLMN ID +NID(optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) One reader only belongs to one gNB. One reader can map to one or multiple A-IoT Area ID(s). One A-IoT Area may include readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas. OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. In the RAN3 LS (S2-25xxxxx/R3-253803), RAN3 provides the following agreements related to Reader selection: - The AIOTF is aware of the supported “A-IoT Areas” of a gNB via OAM. - A new A-IoT Area is represented by an A-IoT Area ID - A-IoT Area ID = PLMN ID +NID (optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) - The AIOTF is also aware of the served Reader list of a gNB via OAM. - Each Reader is uniquely identified globally by “Global gNB ID + Reader Index”. - The Reader Index is defined as INTEGER (1..65536, …). - The AIOTF may be aware of the location of reader via OAM configuration. - The details of the Reader Location are out the scope of RAN3. - OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. In the RAN3 LS (S2-25xxxxx/R3-253941), RAN3 provides the following agreements related to Reader selection: - No NGAP signalling is introduced for providing the A-IoT RAN Information (served reader list, supported A-IoT areas, reader location) from the gNB to the AIOTF. - About Reader Selection: - Upon only receiving the area in Inventory Request, the gNB selects readers within the indicated area. - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. - WA: Upon only receiving the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment of RAN Reader Selection with RAN3 agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN3 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN node selection information (NG-RAN node(s), AIoT Area list, RAN Reader list, and, optionally, the location of each RAN Reader) via OAM or local configuration.

NOTE 1:	No NGAP signalling is introduced for providing the RAN node selection information from the NG-RAN node to the AIOTF.

NOTE 2:	The identification of AIoT Area and RAN Reader, and the mapping relationships among NG-RAN nodes, RAN Readers and AIoT Areas are defined in TS 38.300 [5].

When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN node selection information, the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node. If the RAN Readers within an AIoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s).

The Requested Service Area Information provided to each NG-RAN node can be empty, a list of AIoT Areas, or a list of RAN Readers.

NOTE 3:	The Target Area information in a received request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node.

The AIOTF sends the AIoT service request to each selected NG-RAN node, either directly or through the selected AMF, the AIOTF sends the AIoT service request to each selected NG-RAN node along with the corresponding Requested Service Area Information for the NG-RAN node.

If the AIOTF provides an empty Requested Service Area Information in the AIoT service request to the NG-RAN node, then the NG-RAN node selects all the served RAN Readers.

If the AIOTF provides a list of AIoT Areas in the AIoT service request to the NG-RAN node, then the NG-RAN node selects the RAN Readers within the indicated area.

If the AIOTF provides a list of RAN Readers in the AIoT service request to the NG-RAN node, then the NG-RAN node selects the RAN Readers indicated by the RAN Reader list.

If an AIoT service request includes information about  individual target AIoT Device(s), the AIOTF may consider the last known serving RAN Reader(s) from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request.

NOTE 4:	The RAN Reader ID is not exposed to the AF.

NOTE 5:	An AIOTF receives the Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

NOTE 6:	The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN reader information in the local AIoT Device context.

# * * * * End of changes * * * *