3GPP TSG-SA2 Meeting #170	S2- 2507451

# 25 - 29 August 2025, Goteborg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | Draft | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE power saving when processing packets for closed IP flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_UEPS |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been experienced in the field that "straggler packets" from closed or partially closed IP flows continue to be sent to the UE, even when they hold no value to the UE anymore. When the UE is in idle mode, the UE shall be paged, initiate a Service Request procedure, enter CM-CONNECTED state, in order to receive these packets. After UE processing, the UE will eventually drop these packets as they belong to closed or partially closed IP flows. This procedure results in waste of energy and unnecessary signaling messages. An optimized procedure is sought that enables the UE to inform the Core Network not page the UE for MT traffic that corresponds to certain port numbers or port number ranges. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two new Paging Restriction Information is proposed that enable the UE to request the AMF: - to restrict paging for certain UDP or TCP port number(s) or port number range(s) - to restrict all paging except for certain UDP or TCP port numbers(s) or port number range(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Waste of energy and unnecessary signaling messages when the UE is in idle mode and receives MT packets from closed or partially closed IP flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.38.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.38.5	Paging Restriction

A UE and the network may support Paging Restriction.

NOTE 1: In previous releases of this specification (Rel-19 and earlier), this feature was restricted to MUSIM UE.

A UE, if the AMF indicates that the network supports Paging Restriction feature, may indicate Paging Restriction Information in the Service Request or Registration Request message (including the case where the Registration Request is sent due to mobility outside the Registration Area, i.e. before detecting whether the network supports the feature in the new Tracking Area, provided that the network has already indicated support for Paging Restriction feature in the current stored Registration Area) as specified in clauses 5.38.2 and 5.38.4.

Based on operator policy the AMF may accept or reject the Paging Restriction Information requested by the UE. If the AMF accepts the Paging Restriction Information from the UE, the AMF stores the Paging Restriction Information from the UE in the UE context. If the AMF rejects the Paging Restriction Information, the AMF removes any stored Paging Restriction Information from the UE context and discards the UEs requested Paging Restriction Information. The AMF informs the UE about the acceptance/rejection of the requested Paging Restriction Information in the Registration Accept or Service Accept message.

If the UE does not provide any Paging Restriction Information in the Service Request over 3GPP access or the Registration Request over 3GPP access, the AMF removes any stored Paging Restriction Information from the UE context.

The Paging Restriction Information may indicate any of the following:

a)	all paging is restricted; or

b)	all paging is restricted, except paging for voice service (IMS voice); or

c)	all paging is restricted, except for certain PDU Session(s); or

d)	all paging is restricted, except paging for voice service (IMS voice) and certain PDU session(s); or

e)	all paging is restricted, except for certain UDP or TCP port numbers(s) or port number range(s); or

f)	paging is restricted for certain UDP or TCP port number(s) or port number range(s).

NOTE 1:	The UE expects not to be paged for any purpose in case a). The UE expects to be paged only for voice service in case b). The UE expects to be paged only for certain PDU Session(s) in case c). The UE expects to be paged for voice service and certain PDU session(s) in case d). The UE expects to be paged only for MT traffic that is matching the UE-defined UDP or TCP port number(s) or port number range(s) in case of e). The UE expects no to be paged for MT traffic that is matching UE-defined UDP or TCP port number(s) or port number range(s) in case of f). The AMF can page the UE for mobile terminated signalling based on local policy considering the stored Paging Restriction Information, except for case a). In this case, to comply with the UE provided Paging Restriction Information, the AMF can trigger AN release procedure as soon as possible after the mobile terminated signalling procedure is executed.

NOTE 2:	In the case of roaming, the Paging Restrictions for voice service implied by bullet b) and d) depends on the existence of an agreement with the HPLMN to support voice service via IMS. Hence the support of Paging Restrictions in bullets b) and d) takes the IMS voice service agreement into consideration.

NOTE 3:	When there is no PLMN-wide support for the Paging Restriction feature, it can occur that upon Mobility Registration Update with Paging Restriction Information the UE detects the network does not support the feature. If so, the UE assumes that no Paging Restriction Information is applied.

**** End of Change ****