# 3GPP TSG-SA WG2 Meeting #170	S2-2506454

25 - 29 August 2025, Goteborg, Sweden

Title:	[Draft] LS on NF idle energy consumption

Response to:	-

Release:	Rel-20

Work Item:	FS_EnergySys_Ph2

Source:	NTT DOCOMO (to be SA2)

To:	SA5

Cc:	-

Contact person:	Bahador Bakhshi

bakhshi (at) docomolab-euro (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

In Rel-19, SA2 introduced EIF (Energy Information Function), which provides estimation of energy consumption (EC) at granularities finer than the NF level (which is already provided by OAM) such as PDU session level EC. The functionalities of EIF are specified in clause 5.51 of TS 23.501. The EIF calculates the ECs at these levels based on the EC of UPF(s) obtained from OAM (and also the information on the data volume), according to the procedures specified in clause 4.29 of TS 23.502 and using the estimation formula documented in Annex T of TS 23.501.

SA2 understands that the “NF Energy Consumption” specified in clause 6.7.3.1 of TS 28.554 is the total amount of energy consumed by the NF including both idle EC for powering up the NF and the EC of providing services by the NF. Using the total EC of UPF to estimate the EC at finer granular levels, which may be exposed to the AF, could be misleading for the consumer because for transmission of the same amount of data, the EIF may report different EC values due to the inclusion of idle EC in the calculation (illustrative examples of the issue are documented in clause 6.11 of TR 23.700-67).

To address this issue, it was proposed to exclude the idle EC of UPFs from the calculation by EIF, which would require further NF-level idle EC information of UPFs from OAM (to be deducted by EIF from the total EC); or alternatively, OAM may provide only the EC of serving user traffic. SA2 understands that for VNFs, the NF-level idle EC depends on the scaling level of the VNF and may vary over time.

SA2 kindly asks SA5 to provide answers to the following questions:

Question 1: Is it feasible that OAM, with some potential enhancements, would be capable of providing the idle EC of UPFs (i.e., the amount of energy consumed only to keep the UPFs up and running) per scaling level of the UPF?

Question 2: Is it feasible that OAM, with some potential enhancements, would be capable of providing the EC of UPF only for handling user traffic (i.e., excluding the idle EC of UPF)?

# 2	Actions

To SA5

ACTION 1: SA2 kindly asks SA5 to answer the questions above.

# 3	Dates of next TSG SA WG-2 meetings

SA2#171		2025-10-13 – 2025-10-17			Wuhan, China

SA2#172		2025-11-17 – 2025-11-21			Dallas, US