SA WG2 Meeting #170	S2-2506878

25 – 29 Aug, 2025, Goteborg , SE

Source:	Google

Title:	[WT#3] Scope and Key Issues proposal to support Agentic AI in 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers the WT#3 for enabling the use of AI in 6G study, which provide an overview of current architecture limitations, a scope and related Key Issues.

# 1. Justifications

This proposal is to address related scopes and key issues for WT#3 in 6G study SID: SP-250806.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

For seamless interaction between applications and the underlying network infrastructure, 3GPP has standardized the Network Exposure Function (NEF) within the 5G Core (5GC) to securely expose network capabilities to authorized Application Functions (AFs) and third-party applications.

The inherent complexity of 3GPP specifications and the underlying network architecture directly translates into usability issues for external third-party service providers and application developers. This highlights the critical need for simpler solutions.

CAMARA [x1] and GSMA Open Gateway [x2] initiatives represent a concerted effort to simplify application developer access to network capabilities through standardized, open APIs by creating a higher-level abstraction layer, directly addressing the practical challenge. However, the fragmented landscape of network APIs across different operators and regions remains a significant hurdle to achieving scale deployments. Application developers require a consistent, globally interoperable set of APIs to ensure their applications function seamlessly across various networks.

## 1.X	Current architecture limitation for supporting network capability exposure

The current 5G architecture presents significant complexity barriers for third-party application developers seeking to leverage network capabilities. The inherent complexity of 3GPP specifications and underlying network architecture directly translates into usability issues for external service providers and application developers. This complexity serves as a significant barrier to the full realization of network capabilities in 5G/6G networks and their monetization potential.

Key limitations include:

- Technical depth requirements: Current APIs are primarily designed by telecom engineers for telecom engineers, not for the broader application developer community.

- Fragmented API landscape: Different operators and regions maintain inconsistent network APIs, hindering scale deployments.

- Complex 3GPP API integration: Application developers must invest significant effort in understanding and implementing complex network interfaces.

- Limited abstraction layers: Lack of intelligent intermediary functions to translate high-level application requirements into specific 3GPP service requirements.

- NEF scalability constraints: Current NEF implementations face scalability challenges when managing hundreds of AF request sessions with diverse application requirements

- Static service parameter mapping: Existing architecture lacks dynamic adaptation mechanisms to translate application-level requirements into network-specific service parameters.

- Limited network analytics integration: Insufficient utilization of network automation analytics for intelligent service provisioning decisions.

Figure 1.x-1 shows current API based network capability exposure with complex, static 3GPP APIs.

Figure 1.x-1: Current API based network capability exposure

This scope is linkage to WT#1.2 for the aspect of network capability exposure. The solutions are expected to demonstrate the optimization and benefits that can be achieved by Agentic AI:

- Reduced Development Effort: Application developers can focus on core logic rather than complex 3GPP API integration

- Accelerated Innovation: Simplified network access enables faster deployment of 6G-enabled services

- Enhanced User Experience: Dynamic AI-driven QoS allocation improves application performance consistency

- New Monetization Opportunities: Operators can better monetize network capabilities through simplified, AI-enhanced service offerings

- Proactive Network Optimization: Agentic AI architecture enables anticipatory resource allocation and service adaptation.

Figure 1.x-2 shows the example of Agentic AI based abstraction architecture, including abstraction protocol e.g. Model Context Protocol (MCP) in [x3], for network capability exposure with simplified and common intelligent work flows.

Figure 1.x-2: Agentic AI based abstraction for network capability exposure

### 1.X.1 Summary

Architectural Motivation: The solutions of this key issue are fundamental evolution toward intelligent, autonomous network capability exposure that addresses current complexity barriers while enabling innovative 6G use cases and business models.

Gap Analysis: Current Rel-15 to Rel-20 architecture lacks intelligent abstraction mechanisms that can:

- Dynamically translate application needs into optimal network service configurations

- Provide proactive resource allocation based on predictive analytics

- Simplify complex 3GPP API interactions for external developers

- Enable autonomous network capability management

Technical Impact: This WT would influence overall system architecture in the following aspects:

- Introduction of Agentic AI architecture to act as intelligent intermediaries.

- Exposing network capabilities for provisioning 5GS/6GS services to the third parties through AI-driven decision making and automated service composition.

- Considering interworking or migration from 5GS with APIs based network capability exposure to 6GS with Agentic AI architecture for network capability exposure.

* * * Start of Change (All New Texts) * * * *

# Annex A.3.X. WT#3.X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT 3 covers the study of AI Native Framework for Network Capability Exposure, including at least the following:

- 3.1: Whether and how to integrate Agentic AI architecture within 6G system architecture as intelligent intermediaries for application-network interaction.

- 3.2: Whether and how to design AI-driven abstraction layer that translate high-level application requirements into specific 3GPP service parameters.

- 3.3: Whether and how to support security and privacy when using Agentic AI architecture for autonomous network capability exposure to proactively manage resource and provision services.

This topic is scoped by the WT#3.X descriptions in Table A.3.X-1.

Table A.3.X-1: Work tasks for Native AI Framework

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| WT_#3.1 | Agentic AI Architecture Integration | 1.1: Study integration approach for Agentic AI architecture within 6G system architecture for network capability exposure to third parties. 1.2: Define Agentic AI architecture interfaces and interaction models (NOTE 1) |
| WT_#3.2 | Intelligent Abstraction Layer Design | 2.1: Study AI-driven translation mechanisms from application requirements to network services parameters 2.2: Define adaptive service composition (NOTE 2) |
| WT_#3.3 | Security and Privacy Framework | 3.1: Study authentication and authorization mechanisms for using Agentic AI in network capability exposure 3.2: Study privacy preservation mechanisms in AI-driven network operations (NOTE 3) |
| NOTE 1: Agentic AI architecture integration needs to consider existing network function architectures and ensure minimal disruption to current deployments while enabling advanced AI capabilities. NOTE 2: The abstraction layer needs to maintain compatibility with existing standards such as CAMARA [x1] and GSMA Open Gateway [x2] initiatives while providing enhanced intelligence. NOTE 3: Security and privacy framework must address expanded attack surfaces introduced by Agentic AI based architecture and API exposure while maintaining robust access controls. |  |  |

* * * Second Change (All New Texts) * * * *

# 3.X. Key Issue #X: Native AI Framework for Simplified Network Capability Exposure

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue focuses on how to enable:

- Intelligent application-network interaction through Agentic AI based architecture that comprehends both application service requirements and underlying network capabilities, facilitating seamless translation between application-level requirements and network service parameters.

- Autonomous service optimization based on real-time network conditions and predictive analytics that enable dynamic resource allocation

- AI-driven abstraction layers that encapsulate 3GPP API complexity, providing simplified, intuitive interfaces for application developers while maintaining full network capability access and preserving existing network function interactions

- Proactive resources management utilizing Agentic AI for network services adaptation, e.g. adaptive QoS provisioning.

Solutions for this key issue will describe, based on the WT scope described in Annex A.X:

- Whether and how to architect AI-native framework that integrate Agentic AI architecture within 5GS/6GS to provide intelligent mediation between third-party applications and network services for network capability exposure, including the protocol and service interfaces for Agentic AI communication.

- Whether and how to design Intelligent Abstraction mechanisms that provide AI-driven translation from high-level application requirements to specific network services parameters and generate adaptive service composition that dynamically optimize network resource allocation.

- Whether and how to establish comprehensive security and privacy framework for Agentic AI based network capability exposure to third parties that ensure secure, scalable, and interoperable Agentic AI operations across diverse 6G deployment scenarios.

* * * Third Change * * * *

Reference

[x1]: CAMERA Project: https://lf-camaraproject.atlassian.net/wiki/spaces/CAM/overview?mode=global

[x2]: GSMA Open Gateway: https://www.gsma.com/solutions-and-impact/gsma-open-gateway/what-is-gsma-open-gateway/

[x3]: Anthropic, Introducing the Model Context Protocol (MCP): https://www.anthropic.com/news/model-context-protocol

* * * End of Changes * * * *