3GPP TSG-SA2 Meeting #170	S2-2507003

Gothenburg, Sweden, 25th  – 29th August, 2025

Source:	China Mobile, Huawei

Title:	Discussion on 6G IMS Architecture Enhancement

Document for:	Discussion

Agenda Item:	30.2

Work Item / Release:	Rel-20

Abstract of the contribution: This contribution discusses the motivation of IMS architecture enhancements and the aspects to be studied in R20.

- Introduction

SP-250857, "Study on IMS Architecture Enhancement", is endorsed at SA#108. This document discusses why study on IMS architecture enhancements is needed in 6G.

- Discussion

2.1 Enhance IMS architecture for new services

Empowered by the rapid development of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs) and AI Agent, and the emerging of new types of terminals such as AI glasses and embodied robots, operators will be able to provide various personalized and enriched communication services natively in the 6G era. 3GPP SA1 has started the FS_6G_REQ study item to identify the use cases and requirements for various new services. The Use cases, including personal AI assistant, intelligent calling, intelligent immersive calling service, multi-agent collaboration etc., create new network requirements for IMS network, which derives the following study areas:

-	How to support AI capabilities natively in the IMS network and invoke 3rd party AI capabilities.

Large portion of 6G use cases defined in 3GPP TS 22.870 [1] leverages the AI capabilities provided by 6G network. Certain use cases that could be supported by the IMS network, e.g., personal AI assistant, intelligent calling, intelligent immersive calling service etc., would require it to provide corresponding AI capabilities natively. In addition, use cases like personal AI assistant service might also need to invoke the AI capabilities that are provided by the 3rd party. For example, personal AI assistant service needs to interact with 3rd party’s AI agents to book a flight and/or a hotel for its subscriber.

-	How to support the transmission, understanding, generation, synchronization, and transcoding of multi-modal data in IMS network.

Multiple use cases, e.g., intelligent immersive calling service, multi-app media synchronization, personal AI assistant, multi-agent collaboration etc., mention the processing of multi-modal data. As the operator managed multimedia service network, IMS should be enhanced to support multi-modal data processing capabilities natively, e.g., transmission, understanding, generation, synchronization and transcoding, in the 6G era.

-	How to support multi-modality network rendering in the IMS network.

Immersive calling, metaverse, XR services, etc., and all the use cases mentioned above need to be supported by lightweight terminals but require heavy computing resource. IMS network already supports AR network rendering and avatar network rendering in 3GPP Rel-18 and Rel-19 respectively. Therefore, IMS should be enhanced to support multi-modality network rendering to better support Rel-20 multi-modality use cases mentioned above.

-	How to support new terminals connecting to IMS network, e.g., wearables, AR glasses, robots, sensors etc.

Various new types of terminals have been introduced in 3GPP TR 22.870 [1] to support corresponding use cases. For examples, immersive calling would require AR glasses; AI agent collaboration involves robots; intelligent home requires multiple sensors. However, not all new terminals that would support full IMS stacks, e.g., SIP/SDP. Whether and how to support new types of terminals that do not have the full IMS stacks to connect to IMS network to provides 6G new services, e.g., immersive calling, personal AI assistant service, etc., should be studied.

2.2 Simplify IMS architecture for OPEX and CAPEX

IMS network has been existing and developing for two decades. It has been developing a heavyweight, complicated system that is hard to be maintained and deploy new services. Network simplification needs to be considered for simplifying network operation and service delivery (3GPP TR 22.870 [1], clause 5.9.5.2) and supporting on-demand rollout (e.g., within hours) of new or updated services/capabilities with minimal disruption to existing services (3GPP TR 22.870 [1], clause 5.9.6.6).

The network simplification can be considered from the following three aspects.

2.2.1 Convergence of the control plane nodes

The convergence can be implemented in the following ways.

- Converging MMTel AS and S-CSCF.

The MMTel services have been stable for multiple 3GPP release, and they are the common services subscribed by almost all the subscribers in IMS network. Therefore, the system overhead from S-CSCF to MMTel AS, i.e., iFC trigger signaling and process logic and the iFC configuration data, can be saved if converging MMTel AS and S-CSCF. In addition, Diameter signaling messages between MMTel AS and HSS are also reduced.

- Converging P-CSCF, S-CSCF and I-CSCF.

According to GSMA PRD IR.65 [2], GSMA had selected S8HR/N9HR (Home Routed) as the recommended solution of IMS roaming except for emergency service, considering that S8HR/N9HR has few requirements on the VPLMN. It implies that the interface between P-CSCF and S-CSCF is not Network-Network Interface (NNI) anymore. Then it becomes an unnecessary interface and increases the system complexity.

I-CSCF is a stateless proxy in IMS network and provides limited functionalities, i.e., querying S-CSCF addresses from HSS and assigning a S-CSCF in (re-)Registration procedure and routing SIP messages to S-CSCF in session-related or session-unrelated procedures as specified in clause 4.6.2 of 3GPP TS 23.228 [3]. Therefore, the performance overhead of SIP protocol stack accounts for a large proportion of I-CSCF system overhead. Considering that the S-CSCFs deployed in the existing IMS network are homogeneous and have no capability difference, the functionality of I-CSCF, i.e., querying S-CSCF addresses from HSS and assigning a S-CSCF in (re-)Registration procedure, can be simplified. Moreover, the interface between I-CSCF and S-CSCF leads to more failure points to IMS network.

Converging P-CSCF, S-CSCF and I-CSCF will reduce system resources overhead, promote system performance significantly and minimize the potential network failure points. For example, some parts of the functionalities implemented in the three CSCFs are same, e.g., SIP protocol stack (including SIP protocol parsing at ingress and SIP protocol encapsulation at egress), Diameter protocol stack, DNS protocol stack, session management, reliability (e.g., signaling storm mitigation and prevention) and so on. After convergence, the CPU and memory overhead of these functionalities in one converged node can be reduced to one third of the original overhead in three CSCFs. In addition, some signaling procedure becomes unnecessary and can be reduced, e.g., UAR (User-Authorization-Request)/UAA (User-Authorization-Answer) interaction between I-CSCF and HSS to query the address of S-CSCF serving the UE.

Figure 2.2.1-1 illustrates the comparison between the standardized IMS registration procedure specified in 3GPP TS 23.228 [3] and the simplified IMS registration procedure after convergence of the control plane nodes. The number of signalling messages is decreased significantly in the latter, from 26 to 10.

Figure 2.2.1-1: IMS registration procedures comparison

Figure 2.2.1-2 and Figure 2.2.1-3 illustrate the standardized IMS session procedure and the simplified IMS session procedure after convergence of the control plane nodes respectively. For brevity, the QoS related process is not included in both procedures. The number of signalling messages in the latter is decreased significantly, from 102 to 32.

Figure 2.2.1-2: the standardized IMS session procedures

Figure 2.2.1-3: the simplified IMS session procedures

It’s obviously that the convergence can help to reduce the system resource overhead, the number of signalling messages and maintenance workload, e.g., data configuration. In addition, it will lead to less E2E signalling latency and better user experience. It will also help to reduce the number of signalling interface and the used protocols.

An extra benefit is that the signalling number interacting between HSS and other nodes is also reduced to a half of the original number in IMS registration procedure shown in Figure 2.2.1-1, from 12 to 6. This is very helpful for preventing IMS network signalling storms according to the analysis in SP-250245 [4] discussed at 3GPP SA#107 plenary.

2.2.2 Convergence of the media plane nodes

In 3GPP Rel-18 TS 23.228 [3], a new Media Function (MF) is introduced and supports all new media functionalities since then, e.g., data channel related functionalities, media rendering capabilities. On the other hand, MRF handles the legacy media capabilities, e.g., play tone, play announcement, DTMF collection etc. This fragmentation not only complicates the media establishment procedure but also creates conflicts. For examples, how to play announcement in an IMS data channel session is not defined. The capabilities of MRF are logically simple and have been stable for multiple 3GPP releases. Therefore, it proposes to review the media capabilities in a holistic view and converge MRF and MF with SBI interfaces.

In addition, it is possible to further converge MF and IMS-AGW to reduce system resources overhead, promote system performance, and provide more flexible NF signalling with SBI interfaces. Almost all phone calls require the basic play tone functionality, which means almost all sessions need to anchor to MRF before they are established. Converging MF and IMS-AGW can also avoid the E2E media renegotiation procedure triggered by the called party answering the call.

2.2.3 Optimization to the iFC mechanism

IMS relies on the iFC (initial Filter Criteria) mechanism shown in Figure 2.2.3-1 for service triggering. Based on the iFC mechanism, any AS which may potentially be used during the call shall be triggered when every SIP message in the call is received by the S-CSCF, from the initial INIVTE request to the last response to BYE request. This may lead to waste of network resources since the invoked AS may not be used at all during the call if the service trigger condition is not matched.

In addition, any invoked AS may be impacted when any other SIP node on the signalling path upgrades with some changes to SIP/SDP signalling or when a new AS is introduced or an existing AS is upgraded for the new service rollout. Therefore, the Interoperability Test (IoT) has to be performed to ensure that there is no impact to all the existing services provided by all the invoked AS.

Figure 2.2.3-1: iFC trigger mechanism

Again, due to the iFC mechanism, all the ASs are triggered in a predefined order. When a new service is deployed, operators have to figure out (manually) a proper place to insert the new service on the iFC chain, then the order of the services cannot be changed, which significantly reduces the flexibility for services interaction in real time. Also, after the initial INVITE exchange, there is no way to add new AS into the call, which reduces the service flexibility further.

It’s proposed to replace the iFC mechanism with Service-Based Interface (SBI) and subscription/notification mechanism. The service AS indicates the S-CSCF the session events and information it requires through subscription procedure, and the S-CSCF only notifies the service AS of the required events and information. It’s unnecessary to put the service AS on the SIP signalling path, which can avoid that the service AS is disturbed by irrelevant SIP messages, fasten the new service rollout, and reduce the unnecessary system overhead of SIP stack in the service AS. In addition, the parallel service trigger of multiple service ASs in some cases become possible.

2.3 Summary

The target of all the enhancements discussed above is to evolve the existing complicated and heavyweight IMS into a lightweight and agile network architecture which can reduce OPEX and CAPEX of service evolution, and match new requirements of various 6G services.

- Proposal

It’s proposed to add the following objectives into FS_ARCH_enIMS SID endorsed by SA#108 plenary.

WT#1: Whether and how to simplify and enhance the IMS architecture.

- WT#1.1: How to converge the control plane nodes, e.g., converging MMTel AS and S-/I-CSCF, converging P-CSCF, S-CSCF and I-CSCF.

- WT#1.2: How to converge the media plane nodes, e.g., converging MF and MRF, converging MF and IMS-AGW.

- WT#1.3: How to optimize the iFC mechanism.

WT#2: Whether and how to enhance the IMS architecture and procedures to support new services. The output of this work task shall be independent of WT#1.

- WT#2.1: How to support AI capabilities natively in the IMS network and invoke 3rd party AI capabilities.

- WT#2.2: How to support the transmission, understanding, generation, synchronization, and transcoding of multi-modal data in the IMS network.

- WT#2.3: How to support computing offload in the IMS network.

- WT#2.4: How to support new terminals connecting to IMS network, e.g., wearables, AR glasses, robots, sensors etc.

- Reference

- [1]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements; Stage 1".

- [2]	GSMA PRD IR.65: "IMS Roaming, Interconnection and Interworking Guidelines".

- [3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

- [4]	3GPP SP-250245: "Discussion on IMS resiliency study".