SA WG2 Meeting #170	S2-250xxxx

25 – 29 Aug, 2025, Goteborg , SE

Source:	Nokia, AT&T, Ericsson, T-Mobile US

Title:	[General aspects] Architecture Assumptions and Requirements

Document for:	Agreement

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution is intended to propose architecture assumptions and requirements for 6G System.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Justifications for architectural assumptions:

1) Even in the agreed 6G SID, re-use of 5G NFs and principles where it makes sense is an underlying principle. Reference from Study item in S2-2506096:

The conclusion for each work task should:

a.          Identify functionalities, NFs etc. that use 5GC NFs as basis and any enhancements.

b.         Identify functionalities, NFs etc. that need further study and that may be redesigned

c.          Identify new functionalities, e.g. NFs to be added for supporting new features

Many companies do assume that 5G SBI (NRF based) framework will be used as baseline also for 6G Core. Thus, it is best to agree from the very beginning of the study that this as an architecture assumption avoid wasting time and effort to discuss other framework etc.

2) It is also proposed to keep NF as the granularity for defining open interfaces. Many operators have in fact indicated that there were too many NFs defined during 5GS time frame while only few of them are deployed, thus fine granular open interfaces is not required or desired as it could have performance impacts, requiring extensive inter-operability testing and complex deployments (hindering also deployment agility). Thus, it is reasonable to assume that the current 5G level of NF granularity is the baseline for defining open interfaces in 6G.

3) In order to ensure that we are able to support voice natively also for 6G, we need to ensure that the existing IMS architecture  is reused and is assumed as the baseline from the start.

4) Standalone 6G system is the baseline assumption for this 6G study item. The 6G study assumes an architecture where the 6G RAN connects to the core network for 6G System via control and user plane interfaces. The 6G UE connects to the 6G RAN and core network for 6G System via control and user plane protocols.

Justifications for architectural requirements:

1) Following is taken from agreed 6G study item. It is critical to capture the requirement also in the TR.

“All work tasks aim at supporting multi-vendor interoperable interfaces.”

2) It is important that the 6G System delivered services are superior compared to earlier generations. Furthermore, it is critical that the 6G System procedures (for connectivity and beyond connectivity services delivered by 3GPP system) result in predictable behaviour (i.e. with or without the integration of AI/ML capabilities).

3) Following is taken from agreed 6G study item. It is again critical to capture this as a key requirement also in the TR.

“This study should ensure that the 6G System fulfils requirements on cloud nativeness, sustainability, energy efficiency, robustness and resiliency.

4) It is also expected in 5G System and 6G System will need to co-exist in most deployments thus it is critical to ensure interworking with service continuity shall be supported for UE(s) that are moving between 5G and 6G networks.

**** First Change (all new text) ****

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

Following are the architecture assumptions identified for this study:

- This study assumes that the Service based interface framework as specified in TS 23.501 [2] clause 7 is the starting point for Core network for 6G System

NOTE 1: Improvements to the SBI framework (e.g. procedure optimizations, enhancing the discovery/selection models supported) for 6G are not precluded.

- This study assumes that Network Functions continue as the only granularity for defining open interfaces within Core network for 6G System.

NOTE 2: This includes the possibility to re-use, to enhance existing 5G Network Functions or to define new 6G specific Network Functions.

- This study assumes NF Set is reused.

- This study assumes leveraging IMS as defined in 3GPP TS 23.228[5] for supporting multi-media services such as voice, video (essential and regulatory services) etc.

NOTE 3: Enhancements to IMS architecture as part of IMS architecture evolution study expected in Rel-20 are not precluded.

- This study assumes that 6G RAN will only be connected to Core network for 6G System. This includes connectivity for both control and user plane interfaces.

## 4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

Following are the architectural requirements identified for this study:

- This study aims at supporting multi-vendor interoperable interfaces within 6G System.

- The 6G System procedures shall result in deterministic behaviour for connectivity and beyond connectivity services delivered by 3GPP system.

- The study should consider the following high-level principles: cloud native deployment, sustainability and energy efficiency, robustness and resiliency.

- As 5G System and 6G System are expected to co-exist in deployments, interworking with service continuity between 5G System and 6G System shall be supported.