3GPP SA WG2 Meeting #170	S2-2507092

# Goteborg, Sweden, 25 – 29 August 2025                                           (revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0041 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) In the RAN3 LS (S2-25xxxxx/R3-253800), RAN3 provides the following feedback on removal of “Service Type” from “Assistance Information”: Since RAN3 already agreed separate procedures in NGAP for Inventory and Command and also agreed to support a “follow-on Command Indication” in Inventory Request to indicate Inventory followed by Command, RAN3 does not see the need to provide such kind of “service type” information in an additional, explicit indicator. Based on RAN3 feedback, SA2 can remove the “Service Type” from “Assistance Information”. 2) In the RAN3 LS (S2-25xxxxx/R3-253940), RAN3 provides the following agreement: RAN3 has agreed to define an Expected D2R Message Size IE in Inventory Request message from A-IoT CN to NG-RAN node. RAN3 will work on the encoding of the IE based on further progress in other WGs. It is proposed SA2 aligns “Size of the Inventory Response message from the AIoT Device” with RAN3 agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Removal of “Service Type” from “Assistance Information”. 2) Alignment of “Size of the Inventory Response message from the AIoT Device” with RAN3 agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN3 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests.

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN:

a)	Optionally, approximate number of AIoT devices based on AF request.

b)	Expected Size of the Inventory Response message from the AIoT Device.

c)	Optionally, time interval for AIoT Device response aggregation used by the NG-RAN as specified in clause 5.9.

For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN:

d)	Size of the Command Response message from the AIoT Device.

Bullet b) is determined by AIOTF based on the AIoT Device ID length for the AIoT Devices that are expected to respond to Inventory.

If not provided by the AF, bullet c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

The assistance information is used by the NG-RAN for performing service operations, e.g., radio resource allocation by using bullets a), b) and d), AIoT Device responses aggregation by using bullet d).

# * * * * End of changes * * * *