3GPP TSG-WG SA2 Meeting #170	S2-2507248

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Architectural Assumptions and Requirements

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Architectural Assumptions for TR 23.700-30 for FS_AmbientIoT_Ph2_ARC.

# 1. Introduction/Discussion

The following are Architectural Assumptions and Requirements for TR 23.700-30 based on the Rel-19 TR 23-700-13 scope and SID in SP-250834.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

## 4.1	Architectural Assumptions

Common assumptions:

-	Architecture support for Ambient power-enabled Internet of Things defined in TS 23.369 [x] is the baseline architecture.

NOTE x:	Any additional security aspects for Ambient IoT requires coordination with SA WG3.

NOTE x:	Any additional charging aspects for Ambient IoT will be studied by SA WG5.

Assumptions for support of Rel-19 AIoT Devices and services in Topology 2:

-	Rel-19 Ambient IoT Devices are not aware of whether Topology 1 or Topology 2 is in use, and will operates the same in both topologies and there is no impact from the support of Topology 2 on Rel-19 Ambient IoT Devices.

-	The following topology as defined in TR 38.848 [7] is assumed:

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control

-	The UE acting as an intermediate node (Reader) is using the RRC based-option from the conclusion in TR 23.700-13 [a].

Assumptions for support of DO-A Capable AIoT Devices:

-	The following traffic types for Ambient IoT Device are assumed:

-	DT: Device-terminated, as defined in Rel-19;

-	DO-DTT: Device-originated - device-terminated triggered, as defined in Rel-19, and

-	DO-A: Device-originated – autonomous.

-	Handover-like procedures are not supported.

-	AIoT Device power saving when paging the AIoT Device needs to be considered.

-	In topology 1 both Direct and Indirect Connectivity paths will be supported.

-	The architecture for Topology 2 supporting DO-A capable devices will use the architecture concluded for the support of Rel-19 AIoT Devices and services in Topology 2 as the baseline.

# * * * * Second change (all new) * * * *

## 4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

-	Support for AIoT Services needs to adhere to the nature of the AIoT Devices (e.g. ultra-low complexity, need for power saving, cost and resource-constrained).

# * * * * End of changes * * * *