3GPP TSG-SA WG2 Meeting #171	S2-2508288

# October 13-17, 2025 Wuhan, China

Source:	Skylo Technologies, ViaSat, Boost Mobile Network, EchoStar, Verizon

Title:	Interim Conclusion for KI#1

Agenda item:	20.1.1

Document for:	Agreement

Work Item / Release: FS_5GSAT_Ph4_ARC /Rel-20

Abstract of the contribution: This paper proposes Interim conclusions for KI#1

# 1. Discussion

TS 23.700-19 v1.0.0 has documented 20 solutions relevant to KI#1 and KI#2.

Regarding KI#1, while evaluating these solutions, the following observations and proposals are identified for system enhancements to support IMS voice service over NB-IoT via a GEO satellite connection to the EPC.

- NB-IoT Non-IP Data Delivery over User Plane (UP), as specified in TS 23.401, provides the lowest protocol overhead among other UP/CP and IP/Non-IP combinations, making it especially suitable for constrained and high-latency environments such as GEO-based NTN. It is proposed to use UP NIDD to support both media packets and voice over GEO signalling delivery for voice over GEO service to accommodate more simultaneous calls using limited IoT NTN (GEO) transmission bandwidth.

- Two EPS bearers shall be used, one for signalling and one for voice media packets, due to their different transport requirements. This separation is consistent with IMS handling of control and media and enables better traffic management and differentiated satellite resource allocation.

Taking into the account of the architecture assumptions specified in TS 23.700-19, where enhancements to NB-IoT (GEO) shall be minimized and considering that NB-IoT does not support dedicated bearers, default bearers shall be used. These two default bearers shall be established within two separate non-IP PDN connections.

In summary, two non-IP PDN connections are proposed, each carrying one default bearer, with one for signalling and one for media. This approach also removes the dependency on the RAN.

- IP with RoHC adds additional complexity and CPU processing for both the UE and the network. This makes it less efficient for NB-IoT, where devices are constrained by limited processing capability, power consumption, and memory footprint. ROHC is not a fit for voice over GEO ROHC’s two-party negotiation process adds several seconds to call setup and increases latency, a significant issue in GEO. ROHC's initial state introduces a large IP overhead, which isn't suitable for the narrowband channels used in IoT NTN.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-19 V0.3.0.

* * * First Change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

To supports KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following principles are concluded:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused to trigger UE-SAT-UE communication, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: The following existing 5G VN features apply for KI#5:

-	A single SMF or a single SMF set is supported.

-	The UEs are in the same 5G VN group.

-	Local switch within UPF on-board satellite, N6 and N19 tunnel between UPFs on-board different satellites are supported.

-	Both IP PDU Session type and Ethernet PDU Session type are supported.

-	Only unicast UE-SAT-UE communication is supported.

Principle 3: The SMF determines whether to continue or to fall back the UE-SAT-UE communication due to serving satellite change, based on the information about the availability of ISL(s) between satellites serving the UEs as well as the 5G VN group supporting UE-SAT-UE communication. How SMF gets the information is per implementation, e.g. based on UPF report or from transport network via a proprietary interface.

Principle 4: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline. To support service continuity, the following apply:

-	For the UE-SAT-UE communication continuity case, the SMF reuses existing procedures for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session as described in clause 4.3.5.7 of TS 23.502 [3] for the UE.

-	For the ground fallback case the SMF coordinates existing procedures at both UEs for Removal of additional PDU Session Anchor and Branching Point or UL CL as described in clause 4.3.5.5 of TS 23.502 [3]. The SMF firstly removes UL CL/L-UPF of peer UE and configure PDR&FAR and N6/N19 routing in ground PSA UPF(s) (if two UEs are using different PSA UPFs) during the satellite handover of local UE, then removes UL CL/L-UPF of local UE after satellite handover of local UE.

Principle 5: For PSA UPF on satellite, SSC mode 2 or 3 can be used to support service continuity. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF).

### 7.1.x	Agreed principles for KI#1 on support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

The following high-level principles apply for KI#1

- Separate EPS bearers shall be established for voice-over-GEO signalling and voice media traffic.

- A default bearer shall be used for voice-over-GEO signalling. Another default bearer, established within a separate PDN connection, shall be used for the delivery of voice packets.

- Both signalling and voice data packets shall be delivered via the user plane.

- Voice data packets shall be delivered using the Non-IP Data Delivery (NIDD) mechanism within a Non-IP PDN connection.

- Voice-over-GEO signalling shall be delivered using the NIDD mechanism within a Non-IP PDN connection.

- In the case of Non-IP Data Delivery, point-to-point tunnelling by UDP/IP encapsulation shall be used as described in TS 23.401 sub-clause 4.3.17.8.3.3.2.

* * * End of Changes * * * *