3GPP TSG-SA2 Meeting #171	S2-2508366

Wuhan, CN, Oct 13 – 17, 2025                                          (was)

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 2

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#2

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions captured in the TR.

## 1.1 Discussion for KI#2:

This KI focuses on the authorization of Sensing Service requests and revocation of an on-going Sensing Service as well as the authorization and revocation for the Sensing Entities. The SF is responsible for the reception, configuration and coordination of a sensing service request. The SF has the necessary information to conduct the required sensing service authorization checks to allow, disallow or even revoke a sensing service. And specifically, the SF can use the provided sensing request information, sensing data and sensing results of an ongoing sensing session as well as profiles and policies, which can be stored at the SF to decide the allowed or not sensing services. The following types of profiles may be used:

- Location-aware sensing profile that indicates whether to allow or not to allow a sensing request in a specific area

- Object-aware sensing profile that indicates whether to allow or not to allow a sensing request for a specific object type, for example UAV.

Based on the discussion in KI#3, the SF has the necessary information to select the appropriate sensing entity (i.e. gNB) e.g., based on sensing entity’s capabilities to support a specific service request and can also decide whether a specific sensing entity is authorized to perform a sensing operation.

Proposal for KI#2:

Following content for KI#2 is proposed to be captured as principles for agreement.

- The SF conducts authorization checks for each sensing service request to decide whether to allow or disallow a sensing service request.

- The SF stores location-aware and object-aware sensing profiles and policies that are used to authorize a sensing service request and configure a sensing service session.

- The SF based on sensing data and sensing results can decide the revocation of an ongoing sensing service request.

- The authorization of the Sensing Entity takes place at the SF, based on the capabilities of a sensing entity and the stored sensing profiles and policies.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.2.1	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#2:

- The SF conducts authorization checks for each sensing service request to decide whether to allow or disallow a sensing service request.

- The SF stores location-aware and object-aware sensing profiles and policies that are used to authorize a sensing service request and configure a sensing service session.

- The SF based on sensing data and sensing results can decide the revocation of an ongoing sensing service request.

- The authorization of the Sensing Entity takes place at the SF, based on the capabilities of a sensing entity and the stored sensing profiles and policies.

# * * * * End of change * * * *