3GPP TSG-SA WG2#171	S2-2508427

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	TR 23.700-04: Update agreement for KI#1

Document for:	Discussion / Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2

Abstract of the contribution: This contribution proposed interim conclusion for KI#1 Transfer of data over UP for UE data.

# 1.	Discussion

This contribution proposed to update the agreed principles for KI#1 Transfer of data over UP for UE data.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-04:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data.

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules, or

-	The packet filter set (e.g., IP address / IPv6 prefix or port number of the new 5GC entity) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

-	The specific DCF is configured by NEF to support the UE data transfer to different AFs that need to receive data for UE side model training.

-	The FQDN / IP address of the DCF needs to be sent to UE using existing methods e.g. new PCO parameter, self-constructed FQDN. The trigger for UE establishing a UP path to the DCF is based on UE side implementation.

-	The UE data transfer configuration is sent from DCF to UE via UP signalling.

-	The event ID is defined for specific use cases which will be defined by RAN WGs.

-	The transferred parameters for each event ID are decided by RAN WGs

# >>>>NEXT CHANGE<<<<

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether and how the UE controls UE data transfer.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether and how to select UEs for UE data collection.

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<