SA WG2 Meeting #171	S2-2508501

13 - 17 Oct, 2025, Wuhan, China	(Revision of S2-2508122)

Source:	[Ericsson, Xiaomi, OPPO, Nokia, InterDigital, Google, Charter Communications], China Mobile

Title:	[WT#1.2.x, QoS] Support QoS differentiation for multiplexing traffic in the same application

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT 1.2.x QoS Framework with 5GS QoS Framework as a baseline.

- Justification

- In R19, QoS differentiation for multiplexing traffic was developed and only applies to the limited types of service, i.e., RTP-based video and audio flow. However, nowadays the various applications are changing and emerging rapidly, there are much more types of applications with multiplexing traffic existing in the real network, which means that the related mechanism can not work very well so far. For the operators, it is beneficial to develop the common mechanism to support QoS differentiation for multiplexing traffic in the same application to cover as more scenarios as possible, e.g., after obtaining more granular service information, the 6G network can implement refined differentiated guarantees of service, thereby optimizing the perceived quality of service experience.

- We proposes to add  one sub-bullet in the following bullet#2, highlighted in yellow in annex clause:

- study whether and how to enhance QoS control mechanisms to support QoS differentiation for multiplexing traffic in the same application.

- Text Proposal

It is proposed to capture the following change in TR 23.801.

**** First Change ****

Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements.

  - support QoS differentiation for multiplexing traffic in the same application.

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

  - UE provided information to the network for QoS collaboration

  - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** End of Changes ****