3GPP TSG-SA2 Meeting #170	S2-2508765

Goteborg, Sweden, August 25-29 2025	(revision of 7677)

Source:	[Nokia] NTT DOCOMO

Title:	[KI#3] proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3.

# 1.	Introduction

This paper summarizes the principles for KI#3

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

NOTE: 	In addition to the new parameters in the NF profile in NRF, the consumer's NF selection logic should also support the parameters to enable this feature.

-	For NF profile, additional parameter such as renewable energy consumption ratio may be considered is SA5 indicates support.

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF and/or energy-related events by EIF may trigger UPF reselection subject to SSC mode of the PDU sessions.

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	whether to support the following items in NF profile:

| Information | Description |
| --- | --- |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |

-	whether the NF profile needs to include explicit indication of usage of renewable energy or using energy Priority to implicitly take this into account is sufficient.

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

-	For NF profile, additional parameter such as renewable energy consumption ratio may be considered is SA5 indicates support.

# * * * * End of change * * * *