3GPP TSG-SA2 Meeting #171	S2-2508893

Wuhan, China, October 13 – 17, 2025

Source:	Ericsson

Title:	KI#3: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3.

# 1.	Introduction

The following table summarizes the principles, benefits, drawbacks or unclear points, conclusion summary of discussion, and proposed agreement of solutions present in 23.700-67 that impact Key Issue 3. The table also includes a column identifying the Section in Annex (if it exists) that discusses the solution.

| Sol. | Discuss | High level Principle | Benefits | Drawbacks or unclear points | Evaluation summary | Proposal |
| --- | --- | --- | --- | --- | --- | --- |
| 19 | A.1 | Take energy related information into account for AMF (re-)selection. | - Re-selects best AMF based on energy information available. |  | Based on clause 4.3.2 in TS 28.310 the change to energySaving state shall not cause interruption of service, so this procedure may be used without requiring the scheduling. | Considered for normative phase. |
| 20 | A.2-1 | NF profile is augmented with Energy Priority Information. | - Enables the selection of the most energy efficient NF. | - Places another parameter in NF selection increasing complexity of this process. | Instead of introducing a new parameter, NF priority can be calculated based on current value plus Energy Priority Information value using a formula such as the one provided as example in A.2-1. | Requires further consideration. |
|  | A.2-2 | NF profile is augmented with schedule of Energy Priority Information. | - NFc knows estimated Energy Priority Information of each NFp and can select NFp based on this prediction. | - Schedule Energy priority estimative in schedule can present significant error compared to reality. - Vulnerable to unusual people or network behaviour. - Vulnerable to NFp failures or NFp not leaving Energy Saving state at the scheduled moment. - Not defined how NFp obtain its own schedule when leaving EnergySaving state. - Overloads with schedules NFcs that interact many types of NFs. | The schedule principle does not bring any significant benefit, presenting significant drawbacks, so in its current proposal it cannot be considered for normative phase. | Not considered for normative phase or require further consideration. |
|  | A.2-3 | NF profile is augmented with schedule of EnergySaving/not EnergySaving state. | - NFc knows when each NFp is in EnergySaving state or not and can select the best NFp based on their future EnergySaving state. |  |  | Not considered for normative phase or require further consideration. |
|  | A.2-4 | SMF selects UPF taking into account Energy Priority information and EnergySaving state. | - Enables the selection of the most efficient UPF. | - Adds complexity into selection process. | Since this principle is based on other solution 20 principles and as in their current state other principles cannot be considered for normative phase. | Requires further consideration. |
| 21 | A.3 | Selection of DNAI based on energy performance, optimizing UP path energy performance. | - Enhances current DNAI (re)selection mechanisms with energy related information. - Enables selection most energy efficient UP path. |  | It is unclear how AF determines UP path per DNAI and how 5GC NFs can obtain and calculate the energy consumption and/or energy efficiency of alternative DNAIs that are not being used. | Requires further consideration. |
| 22 | A.4 | Support of UP path adjustments to maximize the usage of renewable energy. |  | Many parts of the principles and description of this solution is unclear. It does not explain what causes the trigger and includes information in NF profile with higher granularity than the NF. | The overall text of the solution requires better and clear explanations. | Not considered for normative phase or require further consideration. |
| 23 | – | UP path adjustment considering energy-related information. | - Enables operators to apply configurations for optimizing energy (e.g. reduce energy consumption, or increase renewable energy consumption) - Each configuration has its own activation triggers (e.g. time window, energy consumption thresholds, or manual activation), and impacted users (e.g. specific UEs, slices, or PDU sessions). |  | This solution enables operators to implement configurations that leverage existing mechanisms and metrics to optimize the usage of the different UPFs in the network. These configurations can reroute traffic based on energy consumption enabling the shutdown of several UPFs when unnecessary, optimizing network energy efficiency. | Considered for normative phase. |
| 28 | A.5 | Array of Energy priority and policy selecting element of Energy Priority array. | - Enables the deployment of different energy policies in the network. - Allows the usage of different energy policies for each UE or network slice. | - Energy policies can conflict with one another. - No clear benefit of the reason to have multiple energy policies on the same area. - Significant increase in complexity of NF selection process without any major benefit in energy consumption. | The usage of the array of Energy priorities increases complexity of NF selection process without any major benefit in energy consumption. A unique value of Energy Priority calculated based on operator defined importances of each energy metric would provide a simpler and more effective solution. This alternative solution could then be enhanced by enabling operators to define specific areas where each area can have a different importances for each energy metric. | Not considered for normative phase or require further consideration. |
| 29 | – | N3IWF/TNGF Reselection Based on Energy Related Information. | - Enables the selection of the most energy efficient N3IWF/TNGF |  | This solution is based on solution 20 principles, which require further consideration, or are not being considered for normative phase. | Requires further consideration |
| 30 | – | Avoid discovering an NF in the 5G core, which is selected by the OAM to enter an energy saving state in the near future. | - Proposes a mechanism for the transition to EnergySaving state. | - it is unclear what happens to NF and the register in NRF during the EnergySaving state. - the solution does not clarify its connection to the schedule of EnergySaving/Not EnergySaving state. - it is not clear the benefits brought by the solution. | There are many unclear points in the proposed solution, that requires further clarification. | Requires further consideration. |
|  | – | Limit the energy usage of an NF. | - Provides a mechanism to limit the energy consumption of each NF. | - Unclear how to leverage it. - Limiting usage of some NFs might result in more deployed NFs. Increasing overall energy consumption of network. | The solution does not clarify how the “CANARY_RELEASE” could be used. Additionally, by limiting the usage of the NF, it might end up forcing the network to deploy more instances of the NF, increasing the base energy consumption. | Requires further consideration |

# 3. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change  * * * *

## 7.1	Agreed Principles

### 7.1.3	Agreed Principles for KI#3

The following aspects related to KI#3 may be considered for the normative phase according to the evaluation above:

-	Take state related information into account for AMF (re-)selection.

-	UP path adjustment considering existing mechanisms and metrics information.

## 7.2	Topics for further consideration

### 7.2.3	Topics for further consideration for KI#3

The following principles related to KI#3 are for further discussions:

-	NF profile is augmented with Energy Priority Information.

-	SMF selects UPF taking into account Energy Priority information and EnergySaving state.

-	Selection of DNAI based on energy performance, optimizing UP path energy performance.

-	N3IWF/TNGF Reselection Based on Energy Related Information.

-	Limit the energy usage of an NF.

-	Avoid discovering an NF in the 5G core, which is selected by the OAM to enter an energy saving state in the near future.

-	NF profile is augmented with schedule of Energy Priority Information.

-	NF profile is augmented with schedule of EnergySaving/not EnergySaving state.

-	Support of UP path adjustments to maximize the usage of renewable energy.

-	Array of Energy priority and policy selecting element of Energy Priority array.

# * * * * End of change * * * *

Annex A:
Discussion

The annexes have the overall discussion which results in the summaries presented in the table in Introduction of this temporary document. The annex is divided into sections, where each section discusses a specific solution.

## A.1	Solution 19

This solution proposes a procedure to re-allocate AMF considering energy related information, based on the discussion in clause A.2 the scheduling is not considered for normative phase, so the trigger of this procedure cannot be based on the schedule. But considering the text in clause 4.3.2 in TS 28.310 the change from not energySaving state to energySaving state should not cause any impact on the existing services, and all messages need to be redirected to other AMF before the AMF goes into energySaving state. So, the trigger of this procedure can be the decision to change the AMF to energySaving state.

With that adjustment this principle can be considered for normative phase.

## A.2	Solution 20

Each principle proposed in solution 20 is discussed separately in each of the following bullets:

1 – Energy Priority Information parameter is mostly affected by the availability of renewable energy sources, enabling the selection of the NF with more renewable energy sources available.
Current, NF Priority can already consider hardware and software efficiency, and the current NF selection process is already complex. Adding a new priority parameter will significantly impact the NF selection process making it even more complicated. So, instead of adding a new parameter called Energy priority Information. Energy priority information can be a component of the NF priority, where the calculation of the new NF priority can be made based on some formula. For example: 

New NF priority = (NF priority) * (importance of NF priority defined by operator) + (Energy Priority Information) * (importance of Energy Priority information defined by operator)

Since a significant increase in the complexity of NF selection would also affect the final energy consumption of the network, and considering that Energy Priority Information can be easily integrated into the current NF priority, significantly reducing the final impact on existing NFs and increasing compatibility with previous releases. Therefore, the Energy Priority Information can be agreed for normative phase if it is integrated as a component of Priority parameter of NF profile. Otherwise, Energy Priority Information should be considered for further discussion.

2 – Since, Energy Priority Information will be mostly affected by availability of renewable energy sources, to make a schedule it would be necessary to be based on weather forecasts. However, current weather forecasts are for wide area, having a significant error in specific locations, this would result that even an Energy Priority Information schedule of one hour can have a significant error due to the sun, rain, or wind being different than predicted in the location of the renewable energy sources, this difference, might be caused by something blocking it (e.g. animals, clouds, dirt, cars, trucks, etc.) or by the prediction being correct in the surrounding area, but slightly different in the specific area that matters.
The rest of the discussion is presented in following bullet.

3 – The schedule is predicted and then distributed to the different NF consumers, so all NFc will use the schedule they have when selecting a NFp. This means that for the entire duration of the schedule the NFc will select NFp based on the information it currently has. This means in case of unpredicted network behaviour, for example a network overload (e.g. unusual traffic, or an event that gathered more people than usual in a certain area) the NF load would not be taken into account during the selection process, which would result in a suboptimal NF selection, having two possibilities: (i) Selection of a NF already overloaded or (ii) Not selecting an optimal NF because it is supposed to be in EnergySaving state during this unexpected event. On the other hand, if the NF load is taken into account, means that NF load is frequently forwarded to each NFc, so the values in the schedule could also be dynamically updated with a more reliable value using the messages transmitting NF load, resulting in the schedule being useless.
In case of a NFp failure or if the NFp does not leave EnergySaving state in the scheduled moment who will update the schedules of every NFc, since a failure to update the NFcs will result in NFcs selecting a NFp that is unavailable.
Current solution is not considering how the NFp obtains its own schedule when leaving EnergySaving state. Does any entity will provide the schedule to the NFp? Or the NFp should produce a new schedule and propagate to all NFcs?
Currently, an AMF needs to interact at least with eight different types of NFs (e.g. AMF, SMF, NSSF, AUSF, UDM, PCF, NRF, and NEF). In a medium network deployment, an AMF can easily interact with at least two to three NFs of each type, resulting in more than twenty NFs that a single AMF is interacting, in a more complex network a single AMF might interact with more than one hundred NFs. This would result that some NFcs would be overloaded with numerous Energy Priority and EnergySaving schedules, which they would also need to take in all NF selections, significantly impacting these critical NFs.
Considering the significant disadvantages that implementation of the schedules present, we propose that the Energy priority Information schedule and the EnergySaving/not EnergySaving schedule should not be considered for normative phase.

4 – SMF selects UPF taking into account Energy Priority information and EnergySaving state is based on the other principles of this solution, which have too many problems to being considered for normative phase at the moment. Therefore, we propose to put this principle to further consideration.

## A.3	Solution 21

This solution proposes a (re)selection of the DNAI based on the energy efficiency in the path, enabling a selection of the most energy efficient DNAI and UP path.

In the solution, it is said the following:

-	“The AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information, and provide the selected DNAI(s) with corresponding traffic steering policy identifier(s) to the NEF/PCF.”

It is not clear how the AF will get the information about UPFs and corresponding traffic steering policies that should be applied for each of the selected DNAIs.

-	“The energy related information to be taken into account for DNAI (re)selection can include the following:
…
The 5GC NF (i.e. AF, PCF or the SMF) can obtain or calculate the above information based on the notifications received from the EIF, i.e. Energy Consumption information in the CN at per UE per PDU Session and/or per UE per application (i.e. Service Data Flow) granularity, and the notifications received from the UPF(s) about the data volume.”

The 5GC NFs can obtain the energy related information from the EIF for the DNAI being used. But EIF cannot provide energy related information of alternative DNAIs not being used by UE, slice, and/or PDU sessions. So 5GC NFs would not be able to obtain the energy related information from EIF about alternative DNAIs before they are selected.

-	“Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of considering energy consumption and/or energy efficiency of only one NF.“

It is unclear how the UP path adjustment can obtain the overall energy consumption and/or energy efficiency of alternative DNAIs and UP paths that are not being used.

The proposed solution does not clarify how the 5GC NFs are able to obtain and calculate the energy consumption and/or energy efficiency of alternative DNAIs that are not being used. So, this principle requires further consideration.

## A.4	Solution 22

This solution proposes the selection of UPFs by SMF taking into account the usage of renewable energy or not.

The solution states the following:

-	“the energy usage information of renewable energy per slice or a set of slices and corresponding priority of renewable energy per slice in NF profile in NRF;”
The meaning of this sentence is not clear.
How does the energy usage information relates to renewable energy per slice? Do NF profiles in NRF have a priority of renewable energy per slice?

-	“the EIF can obtain the energy resource usage information per slice or for a set of slices from OAM.”
But, based on clause 5.51.2.2.3 in TS 23.501 EIF can only request NF level energy consumption and data volume information from OAM.

-	“The SMF can be notified by EIF whether renewable energy per slice is used and then the SMF is triggered to query NRF by considering whether to reselect a UPF or not for the slice related to the existing PDU Sessions.”
What is the reason for SMF to query NRF, just because it received notification from EIF, or because the slice should be using renewable energy information, but it is not?

-	What does the examples in the solution try to demonstrate? In both examples all UPFs have indication of renewable energy, and SMF just selects the UPF with the lowest number in NF priority. This is not different than what is done in current networks.

-	“The SMF can interact with EIF to obtain the energy related information, and the SMF can (re)select a UPF based on the obtained energy related information.”
The solution does not clarify how the SMF select the different UPFs, for then requesting energy related information from EIF for each of the selected UPFs.

-	“On the other hand, the SMF can also utilize the NRF to (re)select a UPF by considering the energy related information notified by EIF and queries NRF.”
Why does SMF utilize NRF to (re)select UPF, if it already has several queries responses from NRF?

-	“When a SMF is triggered to (re)select a UPF (i.e. is notified by EIF), the SMF queries NRF to get the list of candidates of UPFs, by considering the NF profiles in each of those candidates UPFs.
A notification from EIF is not a trigger to (re)select UPF, the information in the notification can cause that trigger. The notification might not have information to cause a trigger; this is not clarified in the text.

-	“The energy related information that can be included in NF profile can be, e.g.:

- whether renewable energy is used or not per slice per application or for a set of slices;
- energy efficiency.”
Why a NF profile contains information of whether renewable energy is used in each slice or application? NF profile is not expected to have that granularity of information.

Many parts of the principles and description of this solution is unclear, it does not explain what causes the trigger and includes information in NF profile with higher granularity than the NF. So, this principle should not be considered for normative phase.

## A.5	Solution 28

This solution proposes that each NF has an array of energy priorities, where each value can optimize a different aspect of energy, such as renewable energy usage, or energy efficiency. This solution would allow operators to associate each position of the array to a specific energy performance and rate the NF performance relative to the metric used. This would then allow operators to define policies that associate specific services, slices, or UEs to a specified energy performance.

This solution does not present any clear motive for the provision of the array of Energy priorities, the example demonstrates a possibility of using this solution, but at the same time demonstrates a consequence of it, where the two energy policies are conflicting selecting different UPFs. Using the example, the network instead of selecting a UPF with a better performance in a specific energy metric, the network ends up using both UPFs each for a specific set of UEs or slices, making it more difficult to disable one of the UPFs in low-traffic moment. This demonstrates that different energy metrics can have conflicting interests, and at the end instead of having an optimization of energy usage, the solution only increases complexity without any energy benefit or ending up increasing overall energy consumption. Besides, renewable energy requires a dynamic adjustment of the Energy priority values, to ensure that the Energy priority of each NF is up to date to its actual use of renewable energy. This would require a constant update of the complete array of Energy Priorities, which would significantly increase the overall complexity of the NF selection process.

Instead of having an array of Energy priorities that can end up with conflicting interest, a solution where the operator defines a specific importance for each energy metric, which then is used to calculate a unique Energy Priority value, would be better fit. This solution could then be enhanced by enabling operators to define specific areas where each area can have a different importance for each energy metric.