SA WG2 Meeting #171	S2-2509123

13 – 17 October, 2025, Wuhan, China	(was S2-2508043)

Source:			[Apple (penholder)], CATT

Title:			[WT#4] WT for Integrated Sensing And Communication

Document for:			Agreement

Agenda Item:			20.6.4

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution updates the baseline pCR (S2-2508043) for WT#4 which is postponed in SA2#170 meeting and to merge and update the baseline pCR (S2-2507900) for the related key issue.

**** First Change ****

Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE X1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE X2:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE X3: The scope of WT#4 will be aligned with RAN for 6G. SA2 could check and align scope with Rel-20 FS_Sensing_ARC status and with RAN decisions on the scope for 6G during Q1 2026, and to determine whether to start solution discussion in Q2 2026.

**** Second Change (revise S2-2507900) ****

5.X.	Key Issue #X: Support of Integrated Sensing And Communication

5.X.1	Description

This Key Issue focuses on the 6G architecture and procedures to support Integrated Sensing And Communication (ISAC). The following aspects will be studied:

-	Architecture and function definition to support ISAC

-	Service authorization and revocation.

-	Discovery and (re-)selection of Sensing Entity(ies) and Sensing Function(s);

-	Parameter configuration and policy provisioning of the selected Sensing Entity(ies);

-	Collection and distribution of the Sensing measurement data and the associated information for Sensing Result calculation.

-	Exposure of the Sensing Result to the Sensing service consumer.

NOTE 1:	The above-mentioned bullet may require coordination with WT#5. Data framework applicable for ISAC (e.g., Sensing measurement data collection, distribution, Sensing Result exposure, and the storage) will be addressed as part of Key Issues related to WT#5 of 6G SI on Data Framework.

-	Sensing continuity for the moving of the Object(s) and the UE(s) as the Sensing entity(ies);

-	Support for fusion of data from multiple Sensing Entities.

NOTE 2:	Functionalities related to RAN nodes will be defined together with the RAN working groups.

NOTE 3:	Privacy and security require coordination with SA3, OAM and charging aspects require coordination with SA5.

NOTE 4: The scope of WT#4 will be aligned with RAN for 6G. SA2 could check and align scope with Rel-20 FS_Sensing_ARC status and with RAN decisions on the scope for 6G during Q1 2026, and to determine whether to start solution discussion in Q2 2026.

**** End of Changes ****