3GPP TSG-WG SA2 Meeting #171	S2-2509138

Wuhan, CN, 13th – 17th Oct, 2025

Source:	CEWiT

Title:	Interim Conclusion proposal for KI#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide a basis for future discussion, interim conclusions have been incorporated into Clause 7.

# 1.	Discussion

This contribution proposes the interim agreements for KI#4. The interim agreements are drafted based on the common points across all solutions proposed for KI#4 in 3GPP TR 23.700-14 V1.0.0.

The proposed solutions #1, #3, #5, #18, #21, #22, #23, #24, #25, #26 and #34 address KI#4. The solutions are summarized as follows,

- Solution #1

The Sensing Function (SF) sends a trigger to the Sensing Entity (SE) to set up a direct interface over a direct connection with it, if the interface was not yet established. The SE(s) use(s) the direct connection to provide the sensing result to SF. The set up signalling can be realised by SF by invoking the AMF service and SE establishes a TLS connection to the IP address provided by SF. There are different options to realise direct interface between the SE and SF – (i) define a point-to-point protocol, (ii) define a new interface.

- Solution #3

The Sensing Function (SF) may be split into Sensing Control Function (SCF) and Sensing Processing Function (SPF). Here, the SPF is responsible for receiving the sensing data from the SE, and for calculating the sensing result. The SF may establish a path for reporting the 3GPP sensing data from SE(s). On receiving sensing service request from SF, the SE may trigger establishment of path for 3GPP sensing data reporting.

- Solution #5

This solution suggests the enhancement of current N1/N2 interface to support sensing data transmission. The SF forwards the sensing request to sensing entities via AMF, and the SE(s) transfer the sensing data to SF via AMF.

- Solution #18

The sensing data and associated information are transmitted between the Sensing Entity and the Sensing Function via data connection. The data connection establishment procedure is carried out on control plane signalling interaction. The Sensing Function provides its connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity. The Sensing Entity sends binding information via established data connection to sensing function for connection binding.

- Solution #21

The Sensing Function (SF) and sensing entity supports direct interface to transfer sensing message.

- Solution #22

This solution proposes the sensing data and associated information collection and transport via data tunnel. A direct data tunnel is established between the Sensing Entity and the Sensing Function via the sensing control signalling interaction. The Sensing Function and the Sensing Entity exchange their tunnel information (e.g. TEID (Tunnel Endpoint ID) and/or IP address) for data tunnel establishment. The sensing data and the associated information are transmitted from the Sensing Entity to the Sensing Function via the established data tunnel.

- Solution #23

This solution supports multiple sensing data consumer entities (e.g. Sensing Processing Function). The entities involved in the sensing procedure are Sensing Management Function (SeMF) and Sensing Processing Function(s) (SePF). The SeMF sends a processing configuration request to the SePF. The SePF sends a data request message to the sensing entity. The SePF receives the data from the sensing entity and it processes the data.

- Solution #24

The sensing data and associated information from multiple Sensing Entities and Sensing Function (SSMF) is collected and coordinated by the Sensing Data Control Function (SDCF). The configuration request to Sensing Entity includes the SDCF endpoint information as selected by the SSMF to establish a data connection to coordinate the sending sensing data collection and processing from the Sensing Entities. The SDCF performs the sensing processing on the sending data report from Sensing Entity.

- Solution #25

The Sensing Control Function (SCF) is introduced to support the management of control plane signalling for sensing services. If the SCF decides to establish direct gNB-SPF connection, a sensing session is established, which enables the transmission of sensing data and the associated information between the gNB and the Sensing Processing Function (SPF). The SCF provides the gNB with the information required for establishing the direct gNB-SPF connection. The sensing session provides an association between the gNB and the SPF for carrying sensing data. The gNB and SPF may maintain the established direct connection, which can be reused for subsequent transmissions of sensing data and related information triggered by either the gNB or the sensing function

- Solution #26

A new Network Function, i.e. Sensing Data Management Function (SDMF) is introduced for the storage and management of the Sensing result. When the Sensing Entity is a gNB, the SCF sends the sensing service request and the configuration information to the gNB via a new interface defined between the SCF and the gNB.

If the sensing result is not available in the SDMF, the SCF needs to send the Sensing Initiated Request and the corresponding parameters to the SPF. When the gNB finishes the sensing data measurement, the gNB reports the sensing data to the SPF directly via the established data tunnel. The Sensing Entities can be multiple gNBs, and the SPF performs the aggregation of the sensing data collected from multiple sources. The SPF generates the Sensing result based on the sensing data received from the gNB(s), and sends the Sensing result to the SCF and the SCF can further send the Sensing result to the SDMF for storage.

- Solution #34

The solution supports direct connection between Sensing entity and Sensing Function (SF). The data connection is used for sensing data transport. The Sensing Function send the connection address information to Sensing entity. If Control and Processing Split is applied, the SCF request the SPF to allocate the data connection address information to receive the sensing data. The Sensing entity establish the connection with SF using the address information received from SF (SCF).

Based on the above given summary of the solutions, we propose the following interim agreements on KI#4:

- On receiving a sensing request from sensing service consumer, Sensing NF decides to obtain the sensing data from Sensing Entity via control plane or data plane.

- On decision of using control plane for transmission of sensing data, Sensing NF collects the data via AMF and transfers to another dedicated NF for processing.

- If data plane is decided to be used, sensing NF configures the processing NF and Sensing Entity for the transmission of sensing data.

- The processing NF is responsible for processing the collected sensing data and generating the sensing result.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

## 7.1	Agreed Principles

### 7.1.A	KI#4

The following are the interim agreements for KI#4:

- Upon receiving a sensing service request, the Sensing Management Function (SeMF) decides whether sensing data can be obtained from the Sensing Entity via a control plane connection or a data plane connection.

- If SeMF decides to use control plane, then the sensing data is collected by SeMF via AMF or directly and is transferred to a Sensing Processing Function (SePF) for sensing data processing

NOTE 1:	The exact method of how SeMF communicates to Sensing Entity will be defined in normative phase.

- If data plane is decided to be used, then the sensing data transfer from Sensing Entity to SePF is configured by SeMF.

NOTE 2:	The exact method of how SeMF configures Sensing Entity to establish data path for sensing data transfer will be covered in the interim agreements of KI#6.

NOTE 3:	The exact information provided in the sensing configuration request will be dependent on RAN WG3 study and will be specified during the normative phase.

- When the sensing data is received from the Sensing Entity (via Control Plane or Data Plane), the SePF processes the sensing data and generates the sensing result.

# * * * * End of Changes * * * *