SA WG2 Meeting #171	S2-2509176

13 – 17 October 2025, Wuhan , China

Source:	InterDigital Inc.

Title:	[WT#1.2.x, Localized Service Access] Support for Localized Service Access

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT#1.2 Localized Service Access.

# Discussion

At SA2 #170, S2-2508125 on Localized Service Access was postponed. This paper proposes a few changes to S2-2508125.

Proposal #1: Add localized service discovery/selection aspects

UE discovering and selecting the localized services and the network providing the localized services is the foundation for localized service access. This important aspect is now missing in the WT description and we propose to add it.

Proposal #2: Clarify that “network providing localized services” can also be PLMN

Some text in S2-2508125, such as the sentence quoted below, can be misinterpreted as that “the network providing localized services” is not PLMN. We believe this is not the intention and we propose to add a Note clarifying that “the network providing localized services” can also be PLMN or a part of a PLMN.

- interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

**** First Change ****

# Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1: Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

-	Discovery and selection of localized services and the network providing the localized services.

-	authorization and authentication of UE for localized service access

- 	interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

- Whether and how to support UE mobility between PLMN network and network providing localized services, or between networks providing localized service

- Whether and how to support service continuity for localized service, including access to localized service after leaving the original network providing localized service

NOTE 2: The network providing localized services can also be a PLMN or a part of a PLMN.

**** End of Changes ****