3GPP SA WG2 Meeting #171	 S2-2509189

# Wuhan, China, 13-17 October 2025					             			    (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0562 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Layer-2 Intermediate UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In LS on topologies for L2 based multi-hop U2N relay (S2-2508186/ R2-2506569), RAN2 has clarified the following on their design for the Layer-2 based intermeidate relay UE: - One intermediate Relay UE connects with only one parent (upstream) Relay UE for L2 based multi-hop U2N relay operation. - One intermediate Relay UE has only one upstream PC5 connection towards the parent (upstream) Relay UE for L2 based multi-hop U2N relay operation. Given that the management of the PC5 connection is in the scope of SA2, such design considerations need to be also reflected in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clasue on 5G ProSe Layer-2 Intermediate UE-to-Network Relay to clarify on the topology limitation of RAN2 design. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 design principles to support stage 3 protocol design for Layer-2 based multi-hop UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 5.4.5	5G ProSe Intermediate UE-to-Network Relay

#### 5.4.5.1	General

5G ProSe Intermediate UE-to-Network Relay can operate within NG-RAN coverage or out of NG-RAN coverage. The 5G ProSe Intermediate UE-to-Network Relays, 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay may have different HPLMNs.

The maximum number of 5G ProSe Intermediate UE-to-Network Relay on the path of 5G ProSe Remote UE is controlled by the maximum number of hops value configured on the 5G ProSe Intermediate UE-to-Network Relays per RSC, i.e. less than the value of the maximum number of hops. When multiple 5G ProSe Intermediate UE-to-Network Relays are used on the path, a relay (5G ProSe Intermediate UE-to-Network Relay or 5G ProSe UE-to-Network Relay) that has less hops to the NG-RAN is the parent relay of the child relay (5G ProSe Intermediate UE-to-Network Relay) that has more hops to the NG-RAN.

For a 5G ProSe Remote UE accessing 5G ProSe UE-to-Network Relay via one or more 5G ProSe Intermediate UE-to-Network Relay, the principles defined in clauses 5.4.1 to 5.4.4 apply.

#### 5.4.5.2	5G ProSe Layer-3 Intermediate UE-to-Network Relay

The 5G ProSe Layer-3 Intermediate UE-to-Network Relay provides relay service for any IP, Ethernet or Unstructured traffic based on the corresponding RSC announced by the 5G ProSe UE-to-Network Relay. The 5G ProSe Layer-3 Intermediate UE-to-Network Relay determines the traffic type based on configuration of the mapping between PDU Session parameters and RSC, as specified in clause 5.1.4.1a.

For IP type of traffic, the 5G ProSe Layer-3 Intermediate UE-to-Network Relay needs to act as an IP router and for Ethernet type of traffic, the 5G ProSe Layer-3 Intermediate UE-to-Network Relay needs to act as an Ethernet switch.

In case of IP type of traffic or Ethernet type of traffic, the 5G ProSe Layer-3 Intermediate UE-to-Network Relay's upstream Layer-2 link (with the 5G ProSe UE-to-Network Relay or another 5G ProSe Layer-3 Intermediate UE-to-Network Relay) can be shared by more than one 5G ProSe Layer-3 Remote UEs or downstream 5G ProSe Layer-3 Intermediate UE-to-Network Relays. For Unstructured type of traffic, one upstream Layer-2 link can be used to support only one 5G ProSe Layer-3 Remote UE.

#### 5.4.5.3	5G ProSe Layer-2 Intermediate UE-to-Network Relay

The 5G ProSe Layer-2 Intermediate UE-to-Network Relay is defined in TS 38.300 [12].

For one 5G ProSe Layer-2 Intermediate UE-to-Network Relay, it connects with only one parent relay for Layer-2 based multi-hop UE-to-Network relay operation. In both Model A and Model B discovery procedures, the 5G ProSe Layer-2 Intermediate UE-to-Network Relay provides information for only one path to the network.

The 5G ProSe Layer-2 Intermediate UE-to-Network Relay only establishes one upstream Layer-2 link towards the parent relay for Layer-2 based multi-hop UE-to-Network relay operation, regardless of the number of Remote UEs it serves.

# >>>>END OF CHANGES<<<<