3GPP SA WG2 Meeting #171	 S2-2509483

# Wuhan, China, 13-17 October 2025						             			    			(was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0564 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of RRC container in Multi-hop U2N Relay Discovery Procedure with Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST, KPN, Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Among the agreements reached at RAN2 #131 and captured in RP-251941, the following two agreements assume the presence of RRC container in the discovery response message and indicate the potential usage of the info: - “Working assumption on having RRC_CONNECTED state indication for the announcing/responding UE (only) in the discovery AS container is confirmed. No SA2/CT1 impact is expected.” - “Capture in a revision of the existing note in RRC running CR (relay reselection section 5.8.15.3) that the UE may prioritise relays based on any information available in the discovery message for the applicable model. However, RRC container is not mentioned in the discovery procedure (6.3.2.5.3) for multi-hop U2N relay with model B, although the RRC container is already mentioned explicitly with model A (6.3.2.5.2 step 3). Thus the proposed updates to Model B discovery procedure that is applicable to L2 U2N multihop relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RRC container information to the multi-hop UE-to-Network Relay Discovery procedure with Model B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency with RAN2#131 agreements on Relay discovery and (re)selection for multi-hop U2N relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 24.554 CR 0841 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | C1-256194 is submitted to CT1 on TS 24.554 to reflect the agrements at RAN2 #131 cited above and to align with the updates proposed in this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 6.3.2.5.3	Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B

Figure 6.3.2.5.3-1: General Procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS requirements.

If the Hop-Limit is determined based on configuration associated with the RSC, the 5G ProSe Remote UE does not include the Hop-Limit in the Solicitation message. Otherwise, the 5G ProSe Remote UE includes the Hop-Limit in the Solicitation message.

2a.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Relay Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that Multi-hop relay is supported, Hop-Count and Hop-Limit (optional).

The Target Info may contain the User Info ID of the UE-to-Network Relay and Intermediate UE-to-Network Relay(s).

3a.	If an indication that Multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message.

The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit (if included in the received Solicitation message) or the (pre)configured maximum number of hops associated with the RSC (if no Hop-Limit is included in the received Solicitation message).

The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), e.g. based on the unicast link profiles, without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s), optionally hop count and optionally Accumulated QoS for PC5 link, as described in clause 5.8.3.1.

If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

4a.	A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message. The hop count is increased by 1. The solicitation message may include Accumulated QoS for PC5 link (e.g. sum of delay of each hop).

The Accumulated QoS for PC5 link, if present in the received solicitation message, will be updated to include the QoS of the PC5 link between the 5G ProSe Intermediate Relay and its child UE (Intermediate Relay or Remote UE).

2b.-5b.	The Solicitation message from the same Remote UE goes through a different ordered list of 5G ProSe Intermediate UE-to-Network Relays.

6-7. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of the 5G ProSe UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe UE-to-Network Relay (if any), then the 5G ProSe UE-to-Network Relay responds to the 5G ProSe Intermediate UE-to-Network Relay with a 5G ProSe UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to- Network Relay Discovery Response message additionally contains the path information compared with that in clause 6.3.2.3.3. The response message may include: the received Accumulated QoS for PC5 link, the RRC container if provided by lower layers.

The 5G ProSe UE-to-Network Relay may choose the path based on e.g. the PC5 signal strength of each message received, hops to the Remote UE, the path information, Accumulated QoS for PC5 link, RRC state information included in the RRC container, etc.

NOTE 1:	It is up to implementation how the 5G ProSe UE-to-Network Relay decides when to send the Relay Discovery Response message.

8-9.	When a 5G ProSe UE-to-Network Relay Discovery Response message is received, if the User Info ID of the receiving 5G ProSe Intermediate UE-to-Network Relay is included in the path information, the 5G ProSe Intermediate UE-to-Network Relay forwards the 5G ProSe UE-to-Network Relay Discovery Response message. The Response message additionally contains the path information which is a list of User Info IDs of 5G ProSe Intermediate UE-to-Network Relay(s) and the remote UE in the order (starting with the remote UE and ending with the last 5G ProSe Intermediate UE-to-Network Relay) of position in the path. Based on the path information, the Relay Discovery Response message is forwarded along the path, where each 5G ProSe Intermediate UE-to-Network Relay determines the next hop information (e.g. User Info ID and Layer-2 ID) to forward the Relay Discovery Response message. The response message may include: the received Accumulated QoS for PC5 link, the RRC container if provided by lower layers.

10.	If the Remote UE receives multiple Discovery Response messages within a pre-configured time window, it may perform relay path selection based on e.g. path information, the PC5 signal strength between Remote UE and its neighbour Intermediate UE-to-Network Relay, Accumulated QoS for PC5 link, RRC state information included in the RRC container, and the number of hops to the 5G ProSe UE-to-Network Relay.

NOTE 2:	If the 5G ProSe Remote UE does not receive any response after a pre-configured time, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

# >>>>END OF CHANGES<<<<