3GPP TSG-SA WG2 Meeting #171	S2-2508688

Wuhan, China, October 13 – 17, 2025

Source:	Ericsson

Title:	KI#2: Update to Solution #20: Signalling Optimization via IWK in P-CSCF

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: The contribution provides an update to Solution #20: Signalling Optimization via IWK in P-CSCF.

# 1. Introduction

This contribution proposes a solution to Key Issue #2:

-	Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

# 2. Proposal

It is proposed to update Solution #20: Signalling Optimization via IWK in P-CSCF  in  3GPP TR 23.700-19.

* * * Start of change * * * *

## 6.20	Solution #20: Signalling Optimization via IWK in P-CSCF

### 6.20.0	High-level solution Principles

This solution addresses KI#2 to support IMS voice call over NB-IoT via GEO connecting to EPC.

The following principles are applied to this solution:

-	Binary encoding of SIP over UNI using the framework defined, e.g. in TS 24.294 [14] for I1 encoding. The UNI reference point is labelled GEO Gm for the purpose of this solution.

NOTE:	The solution uses the I1-like protocol.

-	No restrictions are applied in this solution on UNI signalling in terms of what the UE negotiates, e.g. pre-conditions, 100rel, etc.

-	Only necessary information elements are included in the UNI to enable interoperability and support originating/terminating UE negotiated needs.

-	In summary, the UE, and P-CSCF support for interworking purpose a UE proxy functionality during IMS call-related procedures(i.e. it is not applied to IMS registration procedure), as depicted in Figure 6.20.1-1, that performs the following:

-	Translation from binary encoded SIP to text-based SIP encoding and vice versa.

-	Maintaining an association between the incoming and outgoing sessions in the P-CSCF to ensure compliance to GEO Gm and Mw reference points for interworking purposes. Separate session Identifiers are used for the incoming and outgoing sessions.

-	Storing and managing needed information elements, and updating stored values where applicable, to ensure compliance to the GEO Gm and Mw reference points. -	The UE proxy functionality does not hold session state machine and does not interfere with the state machine implemented in the P-CSCF.

-	IMS Registration over the GEO Gm reference point is performed without binary encoding.  When getting the unsecured Initial IMS Registration, the P-CSCF can identify that it is received over GEO Gm either by using a predefined, GEO Gm specific well-known port, or by employing implementation-specific methods (e.g. using P-Access-Network Information). After the IPSec security association is established, P-CSCF shall be able to receive both text-encoded IMS Register messages (e.g., Re-Register) and binary-encoded session related messages (e.g. INVITE) on the same IPSec IP port. It is assumed this differentiation can be achieved through implementation specific means.

-	This solution does not require any update to the remote end for interworking. Additionally, the solution assumes dedicated bearer for IMS voice.

Additional Assumptions:

-	The identity used for initiating an IMS session is the default IMPU.

-	P-CSCF removes the SIP headers/ SIP Header fields, which do not need to be sent to the UE. The P-CSCF stores the removed information should this be needed for the Mw interactions with the IMS Core.

-	The P-CSCF does not perform additional security checks in relation to IPSec during IMS session initiation or termination procedure.

Editor's note:	This assumption needs to be confirmed by SA WG3.

-	UE Location formatted according to 24.229 [16] PANI encoding is not required. Only the Cell ID is sent, when needed, using the TS 23.003 [22] ECGI encoding.

-	P-Asserted-Id coding is supported over the UNI towards the UE when needed.

-	P-CSCF does not need to send to terminating UEs Information elements whose values have not changed. Similarly, UE does not need to transmit to the P-CSCF IEs whose values have not changed. This is however not reflected in the tables below.

-	SDP negotiation is replaced by pre-configuration where pre-configuration is possible (e.g. codec to be used). For cases where negotiation is required such as UE preference (e.g. 100rel) an IE conveying the preference is used between P-CSCF and UE.

-	Support for privacy header (OIR) can be included with no impact on call session setup time.

-	AGW not impacted by binary encoding of SIP.

* * * Next change * * * *

### 6.20.1	Description

The Figure below depicts the functional architecture for this solution. In this architecture; GEO Gm is the new reference point for the UNI for binary encoding of SIP using the TS 24.294 [14] framework.

GEO P-CSCF is the P-CSCF that performs the necessary interworking between the GEO Gm and Mw, based on the high-level principles depicted above and detailed below. As depicted in the figure, the UE Proxy functionality is strictly restricted to signalling interworking between binary-encoded SIP and text-based SIP, with no SIP state machine.

Figure 6.10.1-1: Architecture to Support GEO IMS audio sessions, and Messaging

In this architecture, SIP Registration sent over GEO Gm uses text-based encoding, while session-related messages use binary-encoding.

The solution supports the two alternatives for transporting IMS voice over the dedicated bearer within an IMS IP PDN connection: as non-IP (NIDD) traffic, or as IP traffic. The difference being the SIP message sizes in both cases and well as the interaction between the P-CSCF and PCC/PCRF during dedicated bearer setup.

The session-related messages sent over the GEO Gm interface are based on modified binary-encoded I1 interface messages. The comparison and modification are illustrated below.

The initial structure has a minimum length of 7 octets. The size of any GEO Gm message varies based on the necessary information elements needed to support the specific aspects being negotiated within an IMS session.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol version number |  |  |  | Protocol identifier |  |  |  |  | 1 |
| Message type |  |  |  |  | R |  | Reason |  | 2 |
| Reason |  |  |  |  |  |  |  |  | 3 |
| Call-Identifier (Part-1) |  |  |  |  |  |  |  |  | 4 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 5 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 6 |
| Sequence-ID |  |  |  |  |  |  |  |  | 7 |
| Information element #1 |  |  |  |  |  |  |  |  |  |
| Information element #2 |  |  |  |  |  |  |  |  |  |

Figure 6.20.1-1: I1 message structure

To ensure compliance to TS 24.229 [16] the protocol over I1 reference point has been extended with:

-	new protocol Messages: ACK, PRACK and UPDATE.

-	additional IEs depicted in yellow in the table below.

The use and processing of these elements and others is described in the procedure clause and referenced tables therein. Additionally, some of the elements currently supported in I1 framework, indicated in green, are not required in this solution as indicated in the two tables below.

Table 6.20.1-1 below is intended to be used for the case of NIDD for IMS voice.

NOTE:	User plane and bearer types including content is handled in other solutions and may impact some of the IEs in the tables below.

Table 6.20.1-1: SIP proposed headers in case of NIDD of IMS Voice

|  |  | Information element | Presence | Format | Length | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol Information | M | V | 1 |  |
|  |  | Message Type | M | V | 2 |  |
| E |  | Call ID | M | V | 3 |  |
| XI |  | Message Sequence Number | M | V | 1 |  |
| S |  | Timestamp | O | TLV | 1 |  |
| TI |  | To-id | O | TLV | 3-X | If e.164 numbers are used the size is 2+7 (for 13 digits) |
| NG |  | From-id | O | TLV | 2-X | If the main IMPU is used the size is only 2 octets |
|  |  | Accept Contact | O | TLV | 5 | not needed for GEO |
|  |  | ERAccept Contact | O | TLV | 3-X | not needed for GEO |
|  |  | Reject Contact | O | TLV | 5 | not needed for GEO |
|  |  | P-asserted-Id | O | TLV | 3-X | Same encoding as for To / From with new IEC |
|  |  | Cell ID | O | TLV | 2+8 | Using TS 23.003 [22] ECGI encoding. No access-type is needed since P-CSCF already knows from Register the UE is in GEO access |
| N |  | Supported | O | TLV | 2+1 | each bit has a predefined value |
| E |  | Required | O | TLV | 2+1 | each bit has a predefined value |
| W |  | Precondition & Codec Negotiation | O | TLV | 2+2 | One octet for Codec profile and one octet for precondition negotiation aligned with SDP. Each bit has a predefined value in both octets. |
|  |  | Rsq/Rack | O | TLV | 2+1 | Rseq and Rack cannot be in the same message. |

Table 6.20.1-2 below is intended to be used for the case of IP transport for IMS voice.

Table 6.20.1-2: SIP proposed headers in case of IP of IMS Voice

|  | Information element | Presence | Format | Length | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Protocol Information | M | V | 1 |  |
|  | Message Type | M | V | 2 |  |
|  | Call ID | M | V | 3 |  |
|  | Message Sequence Number | M | V | 1 |  |
|  | Timestamp | O | TLV | 1 |  |
|  | To-id | O | TLV | 3-X | If e.164 numbers are used the size is 2+7 (for 13 digits) |
|  | From-id | O | TLV | 2-X | If the main IMPU is used the size is only 2 octets |
|  | Accept Contact | O | TLV | 5 | not needed for GEO |
|  | ERAccept Contact | O | TLV | 3-X | not needed for GEO |
|  | Reject Contact | O | TLV | 5 | not needed for GEO |
|  | P-asserted-Id | O | TLV | 3-X | Same encoding as for To / From with new IEC |
|  | Cell ID | O | TLV | 2+8 | Using TS 23.003 [22] ECGI encoding. No access-type is needed since P-CSCF already knows from Register the UE is in GEO access |
|  | Supported | O | TLV | 2+1 | each bit has a predefined value |
|  | Required | O | TLV | 2+1 | each bit has a predefined value |
|  | Precondition & Codec Negotiation | O | TLV | 2+2 | One octet for Codec profile and one octet for precondition negotiation aligned with SDP. Each bit has a predefined value in both octets. |
|  | Rsq/Rack | O | TLV | 2+1 | Rseq and Rack cannot be in the same message. |
|  | AGW-MediaIPAddress | O | TLV | 2+20/40 | IPv4 /IPV6 Address |
|  | AGW-MediaPort | O | TLV | 2+2 | Port |
|  | UE MediaPort | O | TLV | 2+2 | Port |

* * * End of changes * * * *