SA WG2 Meeting #171	S2-2508698

13th – 17th October 2025, Wuhan, China

Source:	Lenovo

Title:	[WT#6, Computing] Working Scope and Key Issues for WT#6 Computing

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the update of the Working scope and key issue description for 6G computing in TR 23.801-01, based on the postponed paper from SA2#170 meeting (S2-2508133 (Oppo) and S2-2507949 (MediaTek)).

1.1 General

During the discussion at the last SA2#170 meeting, some concerns were raised that the gap between the edge computing and the computing service in the 6G needs to be firstly identified and fully analysed before any study can be initialized. Therefore, we list some of the key gaps between the Edge Computing (that has been studied from Release 15 to Release 19) and the computing service that is expected to be supported in the 6G.

1.2 Gap Analysis

Edge Computing has been extensively studied during the past releases and the existing 5G architecture for edge computing is designed for UEs to connect to pre-deployed applications in the local data network that is closer to UE. In detail, the uplink data traffic from UE will be selectively routed to the edge application server (EAS) that resides in the local data network (DN), based on the latency requirement provided by the application function, and the potential EAS information. Normally, the EAS is dedicated for a specific kind of APP, which is as same as the one that sends the uplink traffic from UE (APP client at UE), and is supposed to provide computing service only for this kind of APP.

However, the current Edge Computing mechanism cannot be directly adopted to satisfy the use cases that are specified in the SA1 specs and meet the requirements for the 6G. Some of the key gaps are listed and analysed below:

Gap 1: The joint control of communication and computing

One of the biggest aspects for the 6G computing is to jointly consider the computing and communication, e.g., when selecting appropriate computing site, the computing capability of the computing site, and the associated latency that contains both the processing delay, and the round-trip delay, also need to be considered at the same time to fulfil the service requirements.

However, for Edge Computing, only the communication delay is considered for EAS selection (e.g., Information on user plane latency sent from AF), while the computing delay and the related computing capability of the EAS is not considered. The existing DNS system/query is difficult to support the management of computing resource information dynamically (e.g., computing capability, computing site status). Also, the Edge Computing lacks the information of network status, so it can’t select an optimal EAS that ensures the E2E performance.

To fulfil the joint control, the computing resource information of the EAS needs to be known at the Computing Management Function or the like entity to enable an integral consideration, and an E2E point of view should be taken as a starting point.

Gap 2: The EAS discovery mechanism

In Edge Computing, the EAS discovery mechanism (based on EASDF) does not consider the input like service requirements from the service consumer, e.g., the APP client in the UE and the APP server, that may contain different kind of required service types, latency requirement, detailed computing resources, e.g., GPU, CPU, FLOPS etc. Therefore, it is impossible to reuse the EAS discovery procedure to find the appropriate EAS for the computing task, let alone other types of computing sites in the CN.

Gap 3: The awareness of Computing

In edge computing, the UE is not aware of the selective traffic routing to the local DN since it is done by the SMF instruction based on the AF guidance. While in 6G computing, as one of the main consumers, UE (APP client) will actively request the computing service provided by the network (computing as a service). Therefore, the Edge Computing clearly needs to be enhanced to allow UE to provide input as requirements for the computing service.

1.3 Way forward

To address the gap between the existing Edge Computing and the computing service in the 6G, it is necessary to investigate how to smoothly integrate the computing resources that are in the DN into the whole plate of available computing sites/resources that can be provided (together with the computing resources in the CN), and enable the computing as a service to the service consumers.

Based on the postponed Working Scope paper in the last SA2#170 meeting (S2-2508133 from Oppo), we further modify the key issue descriptions based on the wordings from the Working Scope (S2-2508133), with change marks on top.

**** First Change (based on the texts from postponed paper S2-2508133) ****

# Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

In order to support computing, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource(e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to improve the service experience/QoS(e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point) to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 3:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

**** Second Change (Based on the postponed S2-2508133) ****

5.X. Key Issue #X: Support of Computing

In order to support computing, the following Key Issues need to be studied:

Key Issue #6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

Key Issue #6.2: Coordination of communication and computing, service continuity and QoS/policy aspects:

  - Whether and how to coordinate communication and computing resource(e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect and/or monitor the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics to UE/AF.

  - Whether and how to configure the required computing service from UE/AF to the computing site (s)/Application Server (s).

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to improve the service experience/QoS for the computing service (e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to enable the policy control for the computing service.

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing among computing sites, AF influence, etc.).

NOTE 3: The definition of computing site and computing resource will be determined during the study phase.

Key Issue #6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point) to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 4:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 5: Potential gaps on 5G Edge Computing design will be identified in study phase.

**** End of Changes ****