3GPP TSG-SA WG2 Meeting #171	S2-2509080

13 – 17, October, 2025, Wuhan, CN	(revision of S2-2507234)

Source:	Xiaomi

Title:	KI#4: New Sol: Location Provision for IMS emergency call over NB-IoT(GEO)

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#4.

1. Introduction/Discussion

This solution addresses KI#4 to provide UE location for IMS emergency call over NB-IoT(GEO).

2. Proposal

It is proposed to capture the following text in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[21]	3GPP TS 23.271: "Functional stage 2 description of Location Service (LCS)".

* * * Second of Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x |  |  |  | x |

* * * Third of Change (all new text) * * * *

6.X	Solution #x: Location provision for the IMS emergency call over NB-IoT(GEO)

6.x.1	High level principles

High level solution principles are:

-	UE coarse location information defined in TS 23.401 is used for the IMS emergency call.

-	The EPC Network Induced Location Request procedure defined in TS 23.271 can be reused to acquire UE location for the IMS emergency call.

6.x.2	Description

As specified in TS 23.167, the purpose of UE location information is to enable the IMS network to determine a suitable PSAP and to get more accurate or updated location information during the emergency session required by local regulation. The UE or EPC NF (e.g., MME) provided location information or the location information through positioning can be used for the emergency session purpose.

This contribution proposes the UE provided coarse location information specified in TS 23.401 or the UE location information via EPC-NI-LR defined in TS 23.271 can be used for the IMS emergency session.

6.x.3	Procedures

Figure 6.x.3.1-1: Handling of location information in IMS emergency call over NB-IoT via GEO

The procdure defined in clause 7.6.1 of TS 23.167 can be reused to acquire the UE location information by IMS core with the following additions:

2.	The UE sends an INVITE with an emergency indication to the IMS core. The INVITE contains the UE coarse location information if UE provided.

4.	when the MME receives a location request from the LRF, if it has not the UE location information, it may request the UE to report its location or trigger the EPC-NI-LR procedure defined in TS 23.271 to acquire the UE location information.

6.x.4	Impacts on services, entities and interfaces

UE:

-	Sends UE coarse location information to the IMS core if available

MME:

-	Provides UE coarse location information to the IMS core if required

* * * End of Change * * * *