3GPP TSG-WG SA2 Meeting #172	S2-2510258

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[New Sol] Short Message Service over IP to Emergency Centre leveraging IMS emergency framework

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: this paper proposes a new solution to support routing to the emergency centre/PSAP of an SMS2EC over IP.

# Introduction

In TS 23.167 clause 7.1.1 defines a high-level procedure for UE detectable PS emergency session which enables the UE roaming outside its home PLMN to request access to the local IMS which is also beneficial to successfully route an SMS2EC when the user is roaming in a VPLMN. This procedure is specific to emergency call therefore it is not entirely applicable to sending an SMS2EC. For example, step7 “Establish Emergency Session (and Bearer resources)” is not needed for SMS delivery as the SMS over IP is transported in a SIP MESSAGE..

According to TS23.167 clause 7.1 and 7.4, for the UE which does not have sufficient credentials to authenticate with the IMS network (the default situation for a UE attempting to access an IMS other than its home IMS), the UE does not initiate an IMS emergency registration, but instead immediately establishes an emergency session towards the local P-CSCF with "anonymous user" and "emergency service" indications. It is proposed that when the UE attempts to send an SMS2EC over IMS while roaming, the UE follows the same procedure, that is it establishes an emergency session towards the local P-CSCF with “anonymous user” and “emergency service” indication.

This ensures that the SMS2EC is handled in the local IMS.

This document proposes a procedure for UE detectable SMS2EC which relies on the re-use of the IMS emergency calls framework to ensure local handling.

# Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-65.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| X |  |  | X |

# * * * * Second change (all new) * * * *

## 6.X	Solution #X: Short Message Service over IP to Emergency Centre Leveraging IMS Emergency Framework

### 6.x.0	High-level solution Principles

This solution addressed Key Issue #3: SMS over IP Routing to Appropriate PSAP.

The principle proposed is to re-use the IMS emergency call framework when the UE attempts to send an SMS2EC so that the SMS is routed to the IMS of the serving network rather than by the IMS of the user’s home PLMN.

The UE includes in the SIP MESSAGE to the local P-CSCF an indication that the SIP MESSAGE contains an SMS2EC and the requested emergency type.

The solution is applicable to UE detectable SMS2EC. In case of non-UE detectable, the home network may instruct the UE to send the SMS again using the SMS2EC over IP described in this solution.

### 6.X.1	Description

#### 6.x.1.1	UE in VPLMN

The description of this solution is as follows:

-	A UE that supports SMS2EC starts the procedure to send an SMS2EC when the user requests to send an SMS to a known emergency number (stored in the USIM, stored in the UE, received from the serving network) and the serving network has indicated support for SMS2EC.

-	The UE prepares a SIP MESSAGE that includes the SMS2EC payload, an SMS2EC indication and the requested emergency type.

-	For the UE does not have sufficient credentials to authenticate with the IMS network or registered on the home IMS whilst not in the home PLMN (i.e. the UE is roaming), the UE does send the SIP MESSAGE to the home P-CSCF, but instead initiates MO SMS submit towards the P-CSCF of the serving PLMN with "anonymous user" in the MESSAGE request.

-	Based on "SMS2EC" indication, the P-CSCF inserts UE identifiers and UE location in SIP MESSAGE, then forwards SIP MESSAGE request to an appropriate E-CSCF.

-	the E-CSCF replace the “Request-URI” and “to” header with emergency SMSC address, then forwards SIP MESSAGE to IP-SM-GW.

-	IP-SW-GW extracts SMS payload, the UE location and emergency type and forwards it to emergency SMSC.

-	Based on emergency type and UE location, the SMS2EC is routed by the SMSC to the correct PSAP.

#### 6.x.1.2	Other scenarios

If the UE is not roaming the same procedure as described in 6.x.2.1 and 6.x.2.2 can be reused with the difference that the UE can use the existing IMS registration and submit the SIP MESSAGE with the SMS2EC to the P-CSCF.

### 6.X.2	Procedures

#### 6.X.2.1	High Level Procedure for SMS2EC over IP

The following flow describes how an SMS2EC over IP is delivered to the appropriate PSAP when the UE is roaming  and does not have sufficient credentials to access the IMS of the serving PLMN.

Figure 6.x.2.1-1: high level procedure for UE Detected SMS2EC over IP in VPLMN

The steps follow the UE Detected Emergency Session as defined in TS 23.167 [x] clause 7.1.1 with the following differences.

Step 1. The UE detects the request for the delivery of an SMS2EC.

Step2. In the case that the UE has insufficient resources or capabilities to deliver an SMS2EC due to other ongoing sessions then the UE should terminate the ongoing communication and release reserved bearer resources.

Step6. If the UE does not have sufficient credentials to authenticate with the IMS network or the UE is registered in the Home IMS but roaming, the UE shall not initiate an IMS emergency registration request, but instead immediately initiate step7 (see clause 6.x.2.2 for details).

Step7. The UE initiates MO SMS2EC submit using the SIP MESSAGE procedures which is described in clause 6.x.2.2. If the UE has performed emergency registration, the UE shall use an emergency registered Public User Identifier.

#### 6.X.2.2	SMS2EC over IP in VPLMN

The following flow describes SMS2EC over IP delivery procedure when the UE is in VPLMN .

Figure 6.x.2.2-1: Emergency SMS delivery procedure

0.	The UE performs DNS procedure to obtain the address of the P-CSCF of the serving network

1.	the UE encapsulates the emergency SMS in SIP MESSAGE request and send it to P-CSCF. The MESSAGE includes SMS2EC indication, emergency type and SMS payload.

The UE includes "anonymous user" in the SIP MESSAGE.

2.	If "anonymous user" is included in SIP MESSAGE, the P-CSCF retrieves subscriber related identifier(s) (e.g. IMSI, IMEI(SV),MSISDN) from the IP-CAN. The P-CSCF also retrieves UE location from IP-CAN in this step.

3.	The P-CSCF inserts UE identifiers and UE location in SIP MESSAGE, then forwards SIP MESSAGE request to an appropriate E-CSCF based on SMS2EC indication.

4.	The E-CSCF, if required (e.g. because it has not received the UE location) may query the LRF for location information.

The E-CSCF replace the “Request-URI” and “to” header in SIP MESSAGE with a suitable SMSC address (based on location and support of SMS2EC in the SMSC), then forwards the SIP MESSAGE to the IP-SM-GW.

5-7. The network acknowledges the SIP MESSAGE reception.

8.	the IP-SM-GW extracts and forwards SMS, emergency type and UE location to the SMSC.

9.	Based on the emergency type and UE location, SMSC forwards SMS to a correct EC/PSAP.

10-13.	Acknowledgement of message submit report.

14-16.	UE acknowledges receipt of the submit report.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following entities are impacted based on the solution above:

UE:

-	Support to detect the request for the delivery of an emergency SMS.

-	Support to include SMS2EC indication in SIP MESSAGE.

-	Support to include "anonymous user" in SIP MESSAGE.

P-CSCF:

-	Support to accept “anonymous user" for emergency SMS delivery

-	Inserts UE identifiers and UE location in SIP MESSAGE and forwards SIP MESSAGE request to an appropriate E-CSCF

E-CSCF:

- 	Support to forward SIP MESSAGE request to IP-SW-GW

-	Support to replace the “Request-URI” and “to” header in SIP MESSAGE and RP-DA in RP-DATA with emergency SMSC address if emergency SMSC is deployed

SMSC:

- 	Support to forward SMS to EC/PSAP together with UE location based on destination number

# * * * * End of changes * * * *