3GPP TSG-WG SA2 Meeting #172	S2-2510259

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Update of Interim Agreement of KI#1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Updates to the KI#1 interim conclusions based on RAN feedback and SA2 discussion regarding visibility and controllability requirements.

# 1. Introduction/Discussion

For Option 2-UP, the UE establishes a User Plane connection with a 5GC entity. To support this functionality, we would need a protocol layer (possibly implemented as an app client) for data transfer to be deployed in the UE. As discussed in S2-2508594 proposed in SA2#171, most of companies believe that SA2 need to discuss the ownership of the application client. Currently, no study has been conducted regarding the ownership and governance of such an application client.

If the transport layer client is not owned or managed by the operator, e.g., deployed in the MT, it raises concerns regarding the ability of the Core Network (CN) function to exercise full control over the client. Specifically, mechanisms are needed to ensure that the client does not transmit data to servers other than the intended CN Network Function (NF).

Proposal 1: Add “Whether and how to achieve controllability of UE data collection client” in Topics for further consideration for KI#1.

For the traffic differentiations, no matter whether it is in PDU Session granularity or QoS Flow granularity, the 5GC can recognize the specific traffic based on SDF template, e.g., IP-5-Tuple, which is existing mechanism. Thus, there is no need to further discuss “Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).”

Proposal 2: Remove “Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter)” in Topics for further consideration for KI#1. Legacy mechanism should be reused.

Regarding data visibility requirement, several solutions propose that a check is to be performed to ensure conformity of the collected data with the expected standardized data. There are at least two aspects that are related to this:

- How the check is actually performed?

- How to deal with collected data that does not conform with the standardized format or content?

Regarding the second point, according to some of the solutions, the network should consider such data as invalid, and at least one solution considers that the network may perform processing on the data. However, since the data is supposed to be standardized, and since the focus of SA2 is on the standardized transfer of the data, the network should indeed ensure that collected data is standardized but should not go beyond this to “correct” or “process” the data if the collected data is not conforming to the expectation. In this case the network should indeed discard the data.

This aspect should be captured in the interim agreement. It should also be noted that this does not have dependency on how the network makes this determination or check.

Proposal 3: Add conclusion that 5GC NF shall discard the data if it does not conform with the standardized format or content.

RAN2 sends LS R2-2507928, which provides feedback about if UE needs to be page UE due to data collection:

RAN2 concluded that there is no need to support paging of UEs in IDLE or INACTIVE mode to provide them with data reporting configuration or to trigger data collection procedure.

Proposal 4: Add conclusion that when a UE is in IDLE or INACTIVE mode, it should not be selected for data collection.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * *

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. Legacy mechanism to use SDF Template to differentiate a specific traffic can be reused. Thus, no new standard impact is required.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	When a UE is in IDLE or INACTIVE mode, it should not be selected for data collection.

-	The 5GC NF only exposes data to the UE model training entity server if the data collected from the UE conforms with the standardized format or content. If the 5GC NF detects that data collected from the UE does not conform with the standardized format or content, the 5GC NF discards the data and does not perform any processing on it.

# * * * * Second change * * * *

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether and how to select UEs for UE data collection.

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

-	Whether and how to achieve controllability of UE data collection client.

# * * * * End of changes * * * *