3GPP TSG-SA WG2 Meeting #172	S2-2510396

17 - 21 November, 2025, Dallas, USA

Source: 	vivo, China Mobile, MediaTek Inc., Qualcomm Incorporated, ZTE, China Unicom, CATT, Ericsson, Nokia, Samsung, Spreadtrum, Xiaomi, OPPO, SHARP, Thales, EchoStar, CSCN

Title: 	Interim conclusion for KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an updated interim conclusion for KI#2 of FS_5GSAT_Ph4_ARC on IMS signaling transport.

1	Discussion

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-  The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

In SA2#171 meeting, the interim conclusion has been agreed as following (see S2-2509293):

Based on above key interim conclusion for IMS voice data and the solutions on the table, further interim conclusions need to be provided for following key points related to KI#2:

- IMS Signaling over UP vs CP

- IMS Signaling over IP vs non-IP

The discussion paper S2-2510395 has provided a detailed technical analysis and evaluation on above two key points with following observations:

- Observation 1: Based on the reply form RAN2, DRBs are enough for a UE to support both IMS PDN (EPS bearers for IMS Signaling and data) and internet PDN connection.

- Observation 2: Considering that it was already decided for KI#1 to use UP for the EPS bearer that will be used to transport voice packets, for IMS PDN connection the UE CANNOT use CP and UP simultaneously.

- Observation 3: Based on the reply form SA3, the NAS header CANNOT be reduced to small enough.

- Observation 4: There is no end-to-end security mechanism for IMS signaling transported over Non-IP which lowered the security level comparing to the end-to-end security as each hop nodes (e.g. eNB, S/P-GW) can see IMS signaling body.

Based on above observations, following proposals were provided:

- Proposal 1: The IMS Signaling shall be transported over User Plane (UP)

- Proposal 2: The IMS Signaling shall be transported over IP

2 Proposal

It is proposed to capture the following contents into TR 23.700-19 v1.1.0.

# * * * * First Change* * *

8.2	Interim conclusions for Key Issue #2

Editor's note:	Other interim conclusions for Key Issue#2 are FFS.

The following interim conclusions for Key Issue #2 IMS enhancement for GEO NB-IoT NTN access are made:

-	The IMS signalling shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The IMS signalling shall be transported using IP packets.

# * * * * End of change * * * *