SA WG2 Meeting #171	S2-2510506

17th – 21th November, 2025, Dallas, USA

Source:	ETRI

Title:	KI#1 New Sol: Bounce-Back SMS for Unsupported Routing to PSAP

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1 of TR 23.700-65.

1. Introduction

The key issue #1 of the TR 23.700-65 is:

This key issue aims to study solutions to:

-	identify whether an SMS should be routed to a PSAP;

-	identify the type of emergency service (police, ambulance, fire brigade, etc.) required;

-	address both the case of the UE detecting and the UE not detecting an SMS that is to be routed to Emergency Response Centre;

-	consider the scenario where the serving PLMN does not support routing an SMS to an Emergency Response Centre.

This solution specifically addresses the aspect of concerning the scenario where the serving PLMN does not support routing an SMS to an Emergency Response Centre.

2. Discussion

During the EPS/5GS Attach or Registration procedure, the UE and the network may exchange their capability to support SMS to Emergency Centre (SMS2EC) over NAS and/or IP.

Even with such capability exchange, it remains essential that the UE be explicitly informed when the serving PLMN does not support routing of SMS to an Emergency Response Centre (ERC) for the following reasons:

-	Emergency situations typically occur after PLMN selection. Without prior notice, the user may attempt to send an emergency SMS while camped on a network that cannot route it to a PSAP.

-	If only networks supporting SMS2EC are to be selected, users' PLMN selection is limited and accessibility is reduced—not only for those who fundamentally rely on text-based communication, such as Deaf and Hard of Hearing individuals and Speech Impaired persons, but also for all of us, as we all may potentially encounter emergency situations where voice calls are impossible or dangerous (e.g., home invasions, domestic violence situations, medical conditions affecting speech, or hostage scenarios).

Therefore, even if the capability exchange occurs during registration, the serving PLMN should provide an explicit indication to the UE when it does not support routing of an SMS to an Emergency Response Centre.

3. Proposal

It is proposed to agree the following updates to TR 23.700-65.

*** Start of Changes ***

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X |  |  |
| 3 | X |  |  |
| 4 | X | X |  |
| 5 | X | X |  |
| 6 | X | X |  |
| 7 | X | X |  |
| 8 |  | X |  |
| 9 |  | X |  |
| 10 |  | X |  |
| 11 |  | X |  |
| X | X |  |  |

*** Next Change (All new text)***

6.X	Solution #X: Bounce-Back SMS for Unsupported Routing to PSAP

6.X.0	High level principles

This solution addresses KI#1 and KI#2.

The principles of this solution are listed below:

-	When the serving PLMN does not support routing an SMS2EC to an Emergency Response Centre (ERC), the SMSC shall send a bounce-back short message (e.g., “Text-to-911 is not available. Please call 911 by voice for emergency help.”) to the UE, informing the user that the emergency SMS service is unavailable.

NOTE:	If the SMSC is not used for SMS2EC routing to an ERC, IP-SM-GW or SMSF may act as the bounce-back sender.

-	The serving PLMN selects the same transfer type for the MT-SMS as for the MO-SMS of SMS2EC to achieve reduced delivery latency through the reuse of the network context established by the MO-SMS; if that transfer type is unavailable, it selects an alternative.

6.X.1	Description

This solution defines a bounce-back mechanism to notify the user when the serving PLMN does not support routing of SMS to an Emergency Response Centre (ERC).

When an Emergency SMS (SMS2EC) fails to be routed to a Public Safety Answering Point (PSAP), the SMSC generates a bounce-back short message containing a predefined alert text (e.g., “Text-to-911 is not available. Please call 911 by voice for emergency help.”).

The bounce-back message is delivered to the originating UE using the same SMS delivery type (SMS over NAS or SMS over IP) as the original SMS2EC submission. Upon reception, the UE prominently displays the alert to inform the user that emergency SMS service is unavailable in the current PLMN.

6.X.2	Procedures

#### 6.x.2.1 Bounce-Back SMS Procedure for Unsupported Routing to PSAP

Figure 6.x.2.1-1: SMS2EC bounce-back procedure when routing to PSAP is not supported by serving PLMN

1.	The UE initiates the transmission of an SMS2EC using either SMS over NAS or SMS over IP.

2.	The IP-SM-GW (or equivalent entity) forwards the Mobile-Originated Short Message (MO-Forward-SM) request to the SMSC for further routing.

3.	If the SMSC detects that the serving PLMN does not support routing of SMS to an ERC, it generates a bounce-back short message. This message includes a user notification text such as: “Text-to-911 is not available. Please make a voice call to 911 for emergency help.”
In the bounce-back SMS, TP-DA and RP-DA are set to the UE address, RP-OA is set to the SMSC address, and TP-OA is set to an appropriate sender identity such as “Emergency Alert” or “911.”

4.	The SMSC sends the generated bounce-back message back to the IP-SM-GW (or SMSF), which treats it as a Mobile-Terminated (MT) SMS toward the same UE.

5.	The IP-SM-GW requests from the HSS/UDM the registration and routing information of the UE.

6.	The IP-SM-GW performs the action in step 5 of clause 6.4 of TS 23.204. The IP-SM-GW verifies the subscriber's authorization for SMS and IMS delivery, and performs domain selection to route the message. The IP-SM-GW selects the same SMS delivery type (SMS over NAS or SMS over IP) as used in the original SMS2EC submission, only selects an alternative if the the same SMS delivery type is unavailable.

7.	The bounce-back short message is delivered to the UE using the same transport method originally used (NAS or IP). Upon reception, the UE prominently displays the bounce-back alert message to the user.

6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Supports prominently displaying the bounce-back alert message to the user

SMSC:

-	Supports generating and sending a bounce-back SMS when routing to a PSAP is not supported

IP-SM-GW:

-	Supports selecting the same SMS delivery type (SMS over NAS or SMS over IP) as used in the original SMS2EC submission.

*** End of Change ***