SA WG2 Meeting #172	S2-2509953

17 – 21 November, 2025, Dallas , USA

Source:	CATT, vivo

Title:	[WT#1.2] Discussion on 6G User Plane Protocol

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the motivation for 6G User Plane Protocol of WT#1.2 and analyses the importance of SRv6. It is proposed that the user plane protocol of 6G should utilize the SRv6 protocol .

- Introduction

At the SA2#171 meeting in Wuhan, China, where 20 companies had submitted papers on the user plane architecture of WT # 1.2, majority of  people mention user plane protocols in their documents, e.g., SRv6. In the stage of the discussion, the merged version S2-259709 mentioned the content of the user plane protocol, as follows:

-		Study whether there are any architectural requirements that could influence the design or choice of user plane protocols in the 6G network.

NOTE: SA WG2 will not design or select user plane protocol(s). However, an output of this work task may be architectural requirements that stage 3 working groups may consider when making protocol related decisions.

- Justifications

2.1	5G User Plane Protocol

In Rel-16, CT4 has studied the User Plane Protocol for 5GC (WI: FS_UPPS, TR: 29. 892)  and two candidate protocols (i.e., GTP-U and SRv6) were discussed and evaluated. The conclusion was to use GTP-U and SRv6 without GTP-U was taken out after technical vote. The study was conducted between 2017 and 2019 when the SRv6 protocols were not mature in IETF, and draft versions (e.g., IETF draft-ietf-6man-segment-routing-header-18) were referred to in the TR.

Till now, besides the RFC 8402(Segment Routing Architecture), RFC 8754 (IPv6 Segment Routing Header (SRH)) has been standardized at Mar. 2020, also RFC 8786 (Segment Routing over IPv6 (SRv6) Network Programming) was released at Feb. 2021. Moreover, IETF also studied the Segment Routing over IPv6 for the Mobile User Plane and RFC 9433 (Informational) was released at July, 2023. These RFCs can be further inspected by CT4 group to re-consider to select SRv6 as user plane protocol.

2.2	6G User Plane Protocol

At the CT4 meeting in Sophia, France, where some companies had submitted papers for the New SID on User Plane Protocol in 6G. One paper stated that CT4 study aims to identify the key issues and propose solutions for improvements to user plane management. The key issues to be studied may include(see C4-254342 for details):

-    WT#1: investigate the limitations of PFCP/GTP-U protocol and study the possible enhancements or potential replacements (based on SA2 requirements on the user plane architecture) to meet the future need of 6G network.

Another paper indicated that the objective of this work is to study the protocol aspects for the user plane, and the expected objectives include(see C4-254404 for details):

Identify the requirements of user plane for 6G from Stage 2, e.g., supporting flexible user plane (i.e., UPF (re)selection) for different service.

-	Study how to enhance the GTP-U protocol, leverage the SRv6 with/without GTP-U or design other possible candidate protocols for 6G user plane.

-	Study the impacts to the CP procedures to support enhanced GTP-U or other new protocols.

-	Study the interworking with existing GTP-U entities.

-	Study the support of roaming and failure handling for new protocols.

NOTE:	RAN3 may be involved to coordinate the user plane selection for the interface between RAN and Core network.

2.3	Motivation on user plane protocol

In order to meet the requirements of multi types data, sudden traffic transmission, new services and new capabilities, 6G user plane needs to enhance to meet the requirements of user plane path selection, diverse set of applications and traffic patterns.

In order to transmit data more efficiently and flexibly, the user plane function should enhance the protocol and select the protocol as needed based on the data processing requirements, e.g., SRv6. Compared to GTP-U, SRv6 has more advantages, as follows：

- Simplification of architecture and unification of protocols

GTP-U: In mobile networks, it needs to be used in conjunction with other protocols such as IPsec, forming a "protocol stack" (Eth-IP-UDP-GTP-U-IP-payload). This increases message overhead and device processing complexity.

SRv6: Native based on IPv6, it is a part of the IP network. It can replace multiple overlapping network layers (such as MPLS, L2VPN, L3VPN, GTP-U, etc.) with a unified SRv6 user plane, achieving end-to-end protocol unification from the core network to the bearer network and to the data network. This greatly simplifies network operation and management.

- Native programmability and services awareness

- GTP-U: It's a "mute pipeline". It only knows the starting and ending points of the tunnel, and has no knowledge or control over the network behavior along the path. If we want to implement traffic engineering or path optimization, we need to rely on additional and complex control systems (e.g., SDN controllers)

- SRv6: It is an "intelligent pipeline". Source nodes (e.g., controllers or user terminals) can directly encode services intentions into data packets.

- Simplified End-to-End operations and visualization

- GTP-U: The tunnel is point-to-point, and the middle path is not visible to the operator. Fault localization requires segmented troubleshooting and complex operation and maintenance.

- SRv6: Since the path is pre-defined by the source, the network can easily implement native flow detection. Operations personnel can accurately know which nodes each services flow has passed through, and performance indicators such as latency and packet loss are clear at a glance, achieving true end-to-end services visualization and fast fault demarcation.

- Conclusions and Proposals

Based on the analysis above, CATT would like to provide the following proposals as way forward to be considered during 6G UP discussion:

Proposal1: SA2 should study and identify architectural requirements that could influence the design or choice of user plane protocols(e.g., SRv6) in the 6G network.

Proposal2: SA2 and CT4 should collaborate on the study of 6G user plane protocol.