3GPP TSG-WG SA2 Meeting #172	S2-2510035

Dallas, USA, Oct 17 - 21, 2025

Source:	China Mobile

Title:	Update on Interim Agreements for KI#1

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes an update on Interim agreements for KI#1 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

There are two potential connections between Sensing Function and Sensing Entity (i.e., gNB) in the current release:

The direct connection is a better solution in the current release:

From the perspective of quick and flexible deployment, it is preferable to establish a direct connection between the SE and the Sensing Function. In this case, operators don’t need to upgrade the AMF or deploy the new AMF to support the routing.

From the security perspective, the sensing service should not impact the stability of the existing mobile network serving UE’s communications. Without the AMF in the solution, the entire sensing service solution can be an independent network, sharing only the gNB(s).

From a market investigation perspective, sensing services are more suited for localized applications rather than public sensing services at the beginning of initiating the sensing service. Therefore, there is no need for the AMF in the solution for the sensing service to increase the deployment complexity.

From a performance perspective, if the AMF is in the path, it will introduce additional latency.

From the perspective of the sensing service solution, it simply utilizes the routing function in the AMF.

However, if the Sensing Entity includes the UE, the AMF may be considered in a future release.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.x.0.

* * * * Start of Changes * * * *

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

Principle 4: Sensing control signalling is exchanged between SE and SF(SCF) directly, and AMF is not involved to support Sensing data delivery.

NOTE 4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

* * * * End of Changes * * * *