3GPP TSG SA2 Meeting #172	S2-2510205

17 – 21 Nov, 2025, Dallas, USA

Source:	Ericsson

Title:	(KI#1) Interim agreement

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes some updates to the interim agreements.

# 1. Discussion

This PCR proposes some interim agreements.

Since a solution shall make a CN be in control of the data transfer and have full visibility of the standardized data CN needs to be involved in both data collection and data transfer. The measurement/data collection is related to an Event ID and must be known by the data collection NF in Core. To allow a UE to perform measurement/data collection the gNB must allow data collection. Data collection should only be done for the UEs that are expected to transfer the data to the Core data collection NF. According to paragraph below some information originated from the UE training centre, via CN is expected to allow data collection. For full control and visibility, CN should trigger both data collection and data transfer.

To make it possible for the UE to set-up a UP path to the Core data collection NF, the FQDN or IP address of the Core NF is sent from CN to UE. This can be done either by the Core data collection NF directly sends this info or using the PCF to send a URSP rule including this info. Both methods use an Namf_communication_N1N2MessageTransfer request. This document does not describe which method to use, only describe that the solution will contain an Namf_communication_N1N2MessageTransfer request is sent to AMF which is forwarded to the UE over N1 (including IP address or FQDN of the 5GC data collection NF).

The LS reply from RAN 2 R2-2507902 makes it clear that RAN has no requirement on paging a UE in IDLE nor any requirement against doing so, leaving it to SA2 to decide. Since the request is to let the CN to be in control it is suggested to allow CN to page a UE to set-up a path to the NF receiving the data from the UE. In this way the UE is in ACTIVE when starting data collection.

Also in the LS reply it is mentioned that RAN has not studied how the UEs can be selected for data collection nor which criteria to be used, including the specific criteria geographical area. It is proposed that from the training center only a list of UEs is sent.

RAN 2 makes it clear that UE cannot initiate data collection without first being enabled by gNB. RAN2 also states that “if the RAN receives some information from the UE-side server via the CN or OAM there may be some dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN”. Since SA2 do a core solution, it is proposed information from CN is sent to initiate the data collection.

Finally, in the LS RAN mention that possible other information may be sent from CN to support gNB. This is left for RAN to decide upon and come back to SA2 with an LS on the proposals.

When the CN receives the list of UEs it further checks user consent to make the final UE selection. For each such UE an authorization needs to be sent to the gNB handling the UE. This is to make it possible for the gNB to enable/authorize the UE before the UE requests measurements.

An N2 container is either forwarded together with the N1 container in an N2 message containing both N1 (NAS) and N2 container or if solution requires an N2 container is sent in a separate Namf_communication_N1N2MessageTransfer request.

The N2 container includes the Event ID and indication that UE measurements for UE model is authorized for this NGAP ID. RAN is configured with the measurements related to the Event ID. When gNB receives the authorization from AMF it can enable/authorize the UE before it starts the UE data collection.

To further support coordination with the data collection entity regarding downlink measurements for data collection, RAN may decide to let the gNB at reception of the authorization request respond to AMF whether it will authorize the UE or not. Since there exists an Editor’s Note for the solution proposing this SA2 can only suggest this and clearly make it visible that this is a suggestion from SA2 but RAN takes their own decision.

Principle 1: Selection of UEs is done in Core .

Principle 2: An Namf_communication_N1N2MessageTransfer is sent to AMF which is forward to the UE over N1 (NAS), to initiate the UE to act upon information included in the N1 (NAS) container.

Principle 3: The same or another Namf_communication_N1N2MessageTransfer includes the Event ID and indication that UE measurements for UE model is authorized for this NGAP ID. This information is forwarded to the gNB in an N2 container in an N2 message.

Principle 4: It is suggested to add a NOTE: It is for RAN WGs to decide on any possible response to the N2 message.

Principle 5: Visibility is achieved when the data collection NF receives the standardized data and it verifies this against the Event ID it sent to the UE.

*** Start of changes ***

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	Selection of UEs for data collection and data transfer is done in Core.

-	Selection is done by using a list of UEs, received from the UE training center and a user consent check.

-	An Event ID is sent from the UE training center to the Data collection NF in Core. This Event ID represents the measurements and data collection needed to be done by the selected UE.

NOTE 1: RAN will define the Event ID and the related measurements and standardized data to be sent to Core NF.

-	Visibility is achieved by letting the CN data collection NF know the Event ID and also be configured with the standardized data related to the Event ID.

-	An Namf_communication_N1N2MessageTransfer is sent to AMF which is forwarded to the UE over N1 (including IP address or FQDN of the 5GC data collection NF), to initiate the UE to act upon information included in the N1 (NAS) container.

-	The same or a another Namf_communication_N1N2MessageTransfer includes the Event ID and indication that UE measurements for UE model is authorized for this NGAP ID. This information is forwarded to the gNB in an N2 container in an N2 message.

NOTE 2: It is for RAN WGs to decide on any possible response to the N2 message.

*** Next change***

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

*** End of changes ***