3GPP SA WG2 Meeting #172	S2-2510267

17-21 November 2025, Dallas, USA	(was S2-2508912)

Source:	Qualcomm

Title:	Need for "voice fallback"

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / Rel.20

Abstract of the contribution: This contribution proposes the KI of "voice services for 6G" based to include voice fallback.

# 1.	Discussion

In SA2#170, SA2#171 and the subsequent conference call it was discussed whether "voice fallback" to 5GS/EPS for normal and emergency voice calls should be handled as part of WT#1.4 (and the related KI) or instead as part of the interworking and migration WT/KI.

## 2. 	Need for "voice fallback"

In SA2#171 there was some discussion that confused standards procedures for "voice fallback" (to 5GS or EPS) and commercial availability of native voice over 6GR.

What did we learn from 5G? In 5G the majority of features related to the support of voice over NR was available in Rel.15 specification (Table 1):

| Feature Name​ | Protocol Layer​ | Purpose​ | R15 spec?​ |
| --- | --- | --- | --- |
| Voice, video, emergency indication in RRC Connection Request​ | RRC​ | Call setup success rate optimization​ | Yes​ |
| SSAC​ | RRC​ | Call setup success rate optimization​ | Yes​ |
| IMS voice support indication | NAS | Indication for support of IMS voice in TA | Yes |
| Radio Capability check for voice | NGAP | Check support for IMS voice related radio capabilities by AMF | Yes |
| Paging priority​ | NGAP​ | Call setup success rate optimization​ | Yes​ |
| Flow based QoS, GBR, PBR, LCP, DRB combinations​ | MAC​ | Basic function​ | Yes​ |
| SM signaling​ | NGAP​ | Basic function​ | Yes​ |
| UE capability check​ | NGAP​ | Basic function​ | Yes​ |
| SPS ​ | MAC​ | Power/overhead optimization​ | Yes​ |
| C-DRX​ | MAC​ | Power optimization​ | Yes​ |
| TTI Bundling (slot aggregation)​ | PHY​ | Coverage optimization​ | Yes​ |
| PUSCH enhancement​ | PHY​ | Coverage optimization​ | No​ |
| Short RLC SN​ | RLC​ | Coverage/overhead optimization​ | Yes​ |
| RoHC ​ | PDCP​ | Coverage/overhead optimization​ | Yes​ |
| Short PDCP SN​ | PDCP​ | Coverage optimization​ | No ​ |
| ECN​ | UP and IMS​ | Reliability and spectral efficiency optimization​ | Yes​ |
| MAC CE based rate adaption​ | MAC​ | Reliability and spectral efficiency optimization​ | Yes​ |
| UE reporting air interface delay budget adjustment​ | RRC​ | Power, voice quality and coverage optimization​ | Yes​ |
| SRVCC to UTRAN​ | System​ | Mobility optimization​ | No​ |
| MDT for MMTel​ | RRC​ | Network optimization​ | No​ |

Table 1: VoNR related features

Nevertheless in Nov. 2025, a very significant number of operators (primarily in Europe, NA and Japan) have not yet fully commercially deployed Voice over NR (VoNR). Moreover, we are seeing in general around a 1 year lag between 5G standalone deployments and VoNR deployments, even in limited scale deployments. At the same time all MNOs that have EPS have deployed EPS fallback.

There are various reasons for this e.g. related initially to testing availability, handset availability etc. But most importantly, most of the feedback we’ve received is that in general operators have strong requirement to guarantee at least same performance for voice services on the new G to migrate that particular service, and it takes time to test and adopt all needed voice related features. None of these delaying factors though have to do with standards. In other words, if "EPS fallback" had not been standardised in Rel.15, it would not have been possible to have deployment of IMS voice service in 5G SA or some "last minute patch" would have been required.

We therefore argue, based on previous experience on commercial availability of voice support in a new G, that  availability of "voice fallback" to 5GS or EPS or both from standards point of view is an essential safety net, in case the maturity of the functionality for voice over 6GR, including implementation at both UE and network, and required Interoperability Development Testing (IODT), for all needed features in UE, RAN, CN (NAS and IMS), and across those functions, does not become available in time. This does not mean that functionalities related to support of IMS voice over 6GR would be available in standards on "day 1".

In this paper, we further argue that voice fallback should be covered as part of WT#1.4 because the solutions for voice fallback may diverge from normal interworking scenarios, which are triggered due to UE mobility.

The fundamental difference is that mobility procedures are usually governed by radio condition measurements performed by the source RAN node as a basis for triggering a handover or an RRC redirection. This means for the mobility scenario, even if the handover fails, it is reasonable to expect that in most cases the UE will "stay" at the target cell, e.g., in IDLE mode, since the radio conditions are typically better or at least sufficient at the target cell.

For EPS fallback as it was designed for 5G, the trigger to initiate a handover or an RRC redirection works differently: handover or RRC redirection are triggered based on the existence of a QoS Flow with 5QI-1 that is used for voice. In case of EPS fallback, if the handover fails, the target cell may however not be the optimal cell from radio conditions’ point view. This is because the UE at AS or NAS layers is not aware of why the handover was triggered. From our implementation and testing experience, the error handling of the 5G EPS fallback solution therefore resulted in unnecessarily complex "cross-layer" interactions.

For 6G, if voice fallback is pursued either to EPS or 5GS, we propose to allow the option to study solutions where the UE is aware of the fallback, e.g., by triggering the voice fallback through a NAS procedure. This can be similar to the 5G procedure for emergency fallback, where the UE is aware at least at NAS layer that a fallback is triggered.

Solutions for voice fallback may differ from general mobility procedures. To allow for such targeted solutions to be discussed and studied, we prefer that voice fallback is handled as part of WT 1.4.

# 2.	Proposal

WT#1.4 and KI "voice services for 6G" to study the need and potential solutions for voice fallback for normal and emergency calls.