3GPP TSG-SA2 Meeting #172	S2-250abcd

Dallas, USA, November 17th -21st 2025

Source:	Intel

Title:	KI#1, Update interim agreement

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Interim conclusion for KI#1 – System architecture to support sensing is proposed.

# 1. Introduction

It is proposed to include the functionality supported by Sensing Function (SCF and SPF) in the conclusion for KI#1.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all text new)* * * *

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

Principle 1a: The function of SCF may include:

- receiving the sensing service request and authorization of the sensing service request,

- discovery and selection of the sensing entities (SE) and sensing processing functions corresponding to the sensing service request,

- sending the configuration parameters to the SE required for sensing data collection

Principle 1b: The function of SPF may include:

- sensing data collection from the SE including setup of the data connection required for sensing data collection

- sensing data processing of the data collection from the SE as configured by the SCF

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.