3GPP TSG-WG SA2 Meeting #172	S2-2510389

Dallas, USA, 17 – 21 November 2025	(revision of S2-250abcd)

Source:	vivo

Title:	Conclusions on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: this paper tries to provide the conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection.

1. Introduction

This paper provides the conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection:

- add more detailed conclusions according to the agreed principles in clause 7 for Sensing Entity and Sensing Function Discovery and (Re-)Selection

- add more filter info (e.g., sensing service type) for SF selection

- specify/includes the differences for Sensing Function Discovery and (Re-)Selection

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14 v1.1.0.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

8.X Conclusions for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

To resolve the KI#3 for Sensing Entity and Sensing Function Discovery and (Re-)Selection, the following conclusions are adopted for normative work.

-	The NEF selects the Sensing Function based on NEF's local configuration or NRF query using the requirements in the sensing service request. For NRF query, the NEF provides the requirements in the sensing service request to the NRF based on which the NRF returns the matched Sensing Function instance to the NEF. The requirements in the sensing service request for Sensing Function selection include:

-	Target sensing service area;

-	Optional, the sensing service type.

-	The Sensing Function registers itself to the NRF, with its NF profile. The NF profile of Sensing Function includes:

-	The supported sensing service area of the Sensing Function;

-	Optional, the supported sensing service type of the Sensing Function.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request (e.g., Target sensing service area) and the sensing capability (e.g., supported sensing service area) of the Sensing Entity.

-	In case of indirect architecture (via AMF):

-	The Sensing Entities, i.e. gNBs, provide its sensing capability (e.g., supported sensing service area) to the AMF during the NG setup.

-	The Sensing Function selects the AMF by querying the NRF using the Target sensing service area.

-	The Sensing Function requests the Sensing Entity, i.e. gNBs, from the AMF by providing the Target sensing service area. The AMF returns the matched Sensing Entity to the Sensing Function.

-	In case of direct architecture (no AMF):

-	The Sensing Entities, i.e. gNBs, provide its sensing capability (e.g., supported sensing service area) to the Sensing Function using during NG setup.

-	The Sensing Function registers/updates its NF profile (e.g., the supported sensing service area) based on the sensing capability (e.g., supported sensing service area) of the Sensing Entities, i.e. gNBs.

NOTE 1:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 2:	The mechanisms of what and how the Sensing Entities, i.e. gNBs, provide sensing capability to the AMF or the Sensing Function need coordination with RAN3.

# * * * * End of changes * * * *