SA WG2 Meeting #172	S2-2510467

17 – 21 November, 2025, Dallas, US

Source:	Samsung

Title:	KI#1 Interim Agreements

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes Interim Agreements for KI#1.

1. Discussion

Rel-20 Architecture and precoedures for AmbientIoT Topology 2 can be derived based on Rel-19 partial conclusions and Rel-20 Architecture assumptions and KI agreements.

Based on interim version in SA2#171, many editor’s notes will be resolved.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of first Change (all new text) * * * *

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2:  AIoT System Architecture for Topology 2 in reference point representation

NOTE: Whether the interface between AIOTF and UDM is needed will be checked in later phase.

Message and protocol stack

-	Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-1.

Figure 7.1.1-1: Protocol Stack for the RRC option

UE reader authorization and revocation part:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	Validity information: valid time duration and area that allowed for UE to act as an AIoT Reader.

-	AF ID information indicating that the UE reader belongs to the designated AF

-	UE Reader subscription information is available to AMF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, considering “the UE reader capability” signaled from UE to AMF, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

AIOTF Discovery and Selection for UE reader ID:

Editor’s note: How to discover and select AIOTF when AF provides UE reader ID is FFS.

-	During the UE registration procedure, the AIOTF is selected and invoked by the AMF, after the AMF has authorized the UE as UE Reader.

-	The AMF performs the AIOTF selection.

-	When the UE ID are provided by the AF, the NEF determines its serving AIOTF which covers the AIoT Areas of the serving AMF according to information queried from the UDM and local configuration.

UE reader selection part:

Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader.

For AF providing UE reader ID case:

-	If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN.

For AF providing Area case:

-	For Fixed UE reader:

-	the AIOTF configures the UE Reader configuration information (UE reader ID and AIoT Area ID), AIOTF selects the UE readers (e.g. candidate or final UE readers) based on the received AIoT service operation request and UE Reader configuration information, and provides the selected UE Reader list or AIoT Area ID list to the NG-RAN.

UE reader ID allocation:

Editor’s note: Whether and how to allocate the UE reader ID is FFS.

* * * End of Change * * * *