SA WG2 Meeting #172	S2-2510504

17 – 21 November, 2025, Dallas, US	(revision of S2-25xxxxx)

Source:	vivo, NTT DOCOMO, China Telecom, LG Electronics, Toyota

Title:	KI#1: update of the agreed principles

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update the principles for KI#1 for FS_EnergySys_Ph2.

1.	Introduction

There are two solutions captured in the TR to enhance the energy consumption calculation formulars in order to minimize the impact of the idle energy consumption of the node.

In solution 10, it proposes to enhance the mathematical approach of energy consumption information for the required granularity defined in Rel-19. For the time interval T, the EIF calculates the per bit energy consumption at the network node by using the ratio of the energy consumption difference to the total data volume difference between the two time intervals. Then, the EIF applies the per bit energy consumption as the coefficient to calculate the energy consumption of the required granularity at the node (i.e., UPF and gNB) over the time interval T. It aims to reduce the impact of the idle energy consumption of the node.

In solution 11, it is proposed that the EIF collects the idle energy consumption of the node based on operator’s configuration. This idle energy consumption of the node can be configured within the EIF. And then the EIF excludes (deducts) the idle energy consumption from the total energy consumption of the node to calculate the energy consumption at the required granularities.

These two calculation examples aim to reduce the impact of the idle energy consumption of the node and improve the robustness of the calculation.

Proposal: It is proposed to consider the new formulas taking idle energy consumption into account as the alternative EC estimation formulas for the required granularity.

2. Text Proposal

It is proposed to add the following changes for KI#1 in TR 23.700-67.

# * * * * First change * * * *

7.1.1	Agreed Principles for KI#1

The following principles are made based on the solutions in clause 6 and agreed for KI#1.

1)	The following additional energy related information is exposed by EIF:

A)	The exposure of additional information in association to the exposure of energy consumption information at a desired granularity, specifically,

a)	This additional information, if requested by the consumers of the exposure services of the EIF, is provided together with the Energy Consumption information at the desired granularity and is the result of postprocessing of the information the EIF already has collected (i.e. no new information is required beyond what the EIF already has collected from OAM and SMF).

b)	energy consumption per bit: this is defined as the calculated energy consumption of the desired granularity divided by the measured data volume of the granularity used to calculate the energy consumption.

c)	“Energy consumption category” that is a scalar with multiple values that are determined by the EIF using PLMN-operator defined rules (e.g. 5 categories of percentile) and the rules and values associated to each category are not in scope of 3GPP. This category information is applicable to the same scope (e.g. regional, PLMN-wide) as the scope of the information that the EIF is collecting for the same granularity for other UEs that are e.g. associated to ongoing exposure subscriptions. The scope of the information an EIF collects is implementation and deployment specific.

B)	Per UE application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session with the following restriction:

a)	The application ranking related information exposed to the AF is only about the applications that the AF has permission/authorization with the applications and for which has requested the ranking. This permission/authorization is performed by the NEF. The EIF performs the ranking only of the ongoing applications subject to exposure.

NOTE 1:	The application ranking related information is exposed only to AF(s) related to those applications.

NOTE 2:	It is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF depending on SA5 feedback.

3)

The EIF may support the following new formulas for taking idle energy consumption of the node into account as other examples of EC estimation formulas for the required granularity. Whether to adopt the new formulas is left to implementation and operator policy.

-     If the idle energy consumption information of the node is known by the EIF (e.g., via local configuration), this information may be excluded from the total energy consumption of the node to calculate the energy consumption at the required granularities.

-   In order to minimize the impact of the idle energy consumption of the node, the per bit energy consumption information of the network node may be calculated by using the ratio of the energy consumption information difference to the data volume information difference between two time intervals. This per bit energy consumption information may be further used to calculate the energy consumption at the required granularities.

# * * * End of change * * * *