3GPP TSG|WG-SA2 Meeting #172	S2-2510642

Dallas, USA, November 17 – 21, 2025	(revision of S2-20xxxx)

Source:	Samsung

Title:	Discussion on the approach of keeping NSSAA status in UDM

Agenda Item:	20.18

Document for:	Information

# 1.	Background

For the TEI20_NEPS feature, there is an EN which is basically to decide the need for keeping NSSAA status in UDM to skip the NSSAA procedures in different scenarios explained in clause 2 Discussion.

Editor's note:	a UDM based approach to keep status of NSSAA outcome will be discussed (subject to contributions)

# 2.	Discussion

For the feature NSSAA over EPS, if both the UE and SMF+PGW-C supports the feature and the S-NSSAI associated with the requested PDN connection is subjected to NSSAA, NSSAA procedure is executed as per the agreed paper    S2-2509818. But there are few scenarios mentioned below need to be addressed whether there is a need for NSSAA execution or can be avoided from duplicate execution to save UE, RAN and CN resources and become energy efficient.

1st use case (when PDN connections are handled by different SMF+PGW-Cs)

When the UE triggers the 1st PDN connection with APN 1 and one SMF+PGW-C handles the PDN connection, assuming the associated S-NSSAI is subjected NSSAA, SMF+PGW-C triggers NSSAA and if the NSSAA gets successful, SMF+PGW-C register its address to UDM. For the subsequent PDN connection from the same UE with APN 2, if a different SMF+PGW-C handles the PDN connection with the same associated S-NSSAI as previously established PDN connection, then without any knowledge of past NSSAA successful execution, the 2nd SMF+PGW-C will again trigger unnecessary NSSAA which leads to wastage of UE, RAN and CN resources and not energy efficient.

Observation 1: The SMF+PGW-C which handles the subsequent PDN connections unnecessarily execute NSSAA even though the same UE has already gone through successful NSSAA procedure for the same S-NSSAI during establishment of 1st PDN connection. This consequence is further multiplied if UE does multiple PDN connections and will happen for all the UEs, all the time.

Proposal 1: If the NSSAA gets successful, SMF+PGW-C shall store the NSSAA status as “success/pass” in UDM and SMF+PGW-C fetch from NSSAA status if any from UDM before deciding to trigger NSSAA. If find NSSAA status as success/pass from UDM, SMF+PGW-C can skip NSSAA.

2nd use case (UE has already registered over non-3GPP access and NSSAA is successful for S-NSSAI in 5GS. UE triggers PDN connection over 3GPP access and same S-NSSAI is associated in EPS)

In the case when UE registers over non-3GPP access and NSSAA for the S-NSSAI is successful, the same UE when triggers PDN connection over 3GPP access and same S-NSSAI is associated, then SMF+PGW-C without any knowledge of past NSSAA successful execution by AMF in 5GS for the same UE and same S-NSSAI, again trigger unnecessary NSSAA which leads to wastage of UE, RAN and CN resources and not energy efficient.

Observation 2: The SMF+PGW-C which handles the PDN connections unnecessarily execute NSSAA even though the same UE has already gone through successful NSSAA procedure for the same S-NSSAI in 5GS.

Proposal 2: If the NSSAA gets successful, AMF shall store the NSSAA status as “success/pass” in UDM and SMF+PGW-C fetch NSSAA status if any from UDM before deciding to trigger NSSAA. If find NSSAA status as success/pass from UDM, SMF+PGW-C can skip NSSAA.

3rd use case (UE has established PDN connection over 3GPP access and NSSAA is successful for S-NSSAI in EPS. The same UE is doing initial registration over non-3GPP access and same S-NSSAI is sent in Requested NSSAI)

In the case when UE has established PDN connection over 3GPP access and NSSAA for the S-NSSAI is successful, the same UE when does initial registration over 3GPP access in 5GS and send the same S-NSSAI in Requested NSSAI, then AMF without any knowledge of past NSSAA successful execution by SMF+PGW-C in 5GS for the same UE and same S-NSSAI, again trigger unnecessary NSSAA which leads to wastage of UE, RAN and CN resources and not energy efficient.

Observation 3: The AMF unnecessarily execute NSSAA even though the same UE has already gone through successful NSSAA procedure for the same S-NSSAI in EPS during PDN connection establishment.

Proposal 3: As part of observation 2/proposal 2, SMF+PGW-C is already storing the NSSAA status as success/pass in UDM. AMF can fetch NSSAA status if any from UDM before deciding to trigger NSSAA. If find NSSAA status as success/pass from UDM, AMF can skip NSSAA.

# 3.	Proposal.

Proposal: By looking at the all the three observations, it is proposed to consider for the AMF and SMF+PGW-C to store the NSSAA status in UDM. During PDN connections in EPS, the SMF+PGW-C or registration procedure in 5GS, the AMF fetch NSSAA status if any from UDM and skip NSSAA procedure if finds success/pass.