SA WG2 Meeting #172	S2-2510645

17 – 21 November, 2025, Dallas, TX, USA	(S2-250xxxx)

Source:	NTT DOCOMO

Title:	[WT#2, Migration and Interworking] WT scope and Key Issues

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes WT scope update and Key Issues related to Migration and Interworking under WT#2. The WT scope in the postponed document S2-2509802 is copied here and used as a baseline for the Key Issues.

**** First Change (all text new) ****

# Annex A.X. Migration and Interworking Scope

Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 2a: Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

NOTE 3:	It is assumed that interworking for roaming is within scope of this task.

NOTE 4:	WT2 focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

**** Next Change (all text new) ****

## 5.X	Key Issue #X: 6GS interworking and migration

Study migration and interworking, including

1.	How to support migration to 6GS

a)	Identify possible migration path(s) from 5GS to 6GS, considering the co-existence of EPS.

b)	How to ensure co-existence of NFs that support 6G features and NFs that do not.

2.	How to support interworking with 5GS

a)	How to support inter-RAT handovers and idle-mode mobility between 5G NR and 6G NR.

b)	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility.

c)	Whether and how to make the data throughput for the UE equal to or greater than that of 5G, even with the limited frequency allocation of 6G. For example, whether and how to support simultaneous use of 5G NR and 6G NR for a single UE.

3.	Whether and how to support interworking with EPS

a)	Whether and how to support inter-system handovers and idle-mode mobility between 6GS and EPS, for example whether to support direct mobility between 6G NR and E-UTRA or mobility is provided via 5G NR.

b)	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility.

4.	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

a)	Whether and how to support interworking during mobility of satellites across different systems.

b)	Whether and how to ensure IP address preservation in inter-system handovers for the identified scenarios.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 2a: Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

NOTE 3:	It is assumed that interworking for roaming is within scope of this task.

NOTE 4:	WT2 focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

**** End of Changes ****