3GPP TSG-SA WG2 Meeting #172	S2-2510719

# October 17-21, Dallas, US

Source:	Xiaomi

Title:	Conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes the conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection in clause 8 of TR 23.700-14.

1. Discussion

TS 23.700-14 v1.1.0 has documented the interim agreements for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection:

7.1.3 Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

Proposal 1: copy all the interim agreements for KI#3 in clause 7.1.3 into clause 8.3 conclusion for KI#3.

Observation 2: the supported sensing area of SF is different from the supported sensing area of SE. so it is confusing to use the same wording.

Proposal 2: to use different terms, the sensing service area is used for Sensing Function. And the sensing area is used for Sensing Entity.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

8.3	Conclusion for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing service area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

* * * End of Changes * * * *