SA WG2 Meeting #172	S2-2510750

17 – 21 Nov, 2025, Dallas, TX, USA

Source:	IIT Bombay

Title:	[WT#1.1, all topics] WT#1.1 Support for Control Signalling for 6G

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the Scope and KI for WT#1.1

# 1. Justifications

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

This contribution proposes Scope and KI in line with the pre-SA2#172 CC and follow-up email discussions with some minor revisions.

With respect to the issue of NAS definition that came up during the email discussion, we would like to state that the following definition does not clearly explain the term NAS functionality that is used in the WT#1.1 scope. Also, the term highest stratum is misleading and the phrase control plane between the UE and the 6G CN is not clear.

Non-Access Stratum: The Non-Access Stratum (NAS) forms the highest stratum of the control plane between the UE and the 6G CN

Hence, we propose definitions for the terms Non-Access Stratum Protocols and Non-Access Stratum functionality below.

**** First Change ****

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Access Stratum Protocols: Protocols between UE and the 6G CN for control signalling that are not terminated in the RAN.

Non-Access Stratum functionality: Functionality that is supported by the NAS Protocols to enable an operator service.

NOTE: NAS functionality is not necessary to enable all operator services.

# Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1. Study the support for control signalling for 6G System, including whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1: 	It is assumed that this WT1.1 bullet 1 covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet 1, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. Study the support for control signalling for 6G System, including whether and how to support generic mechanisms (e.g., service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2 can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2 can have dependency on other work tasks e.g., WT1.1 bullet 1, WT3, WT4, WT5, WT6.

NOTE 4: This WT1.1 bullet 2 does not cover services that are not operator services.

**** Second Change (New Text) ****

# 5.X. Key Issue #1.1: Study the support for control signalling for 6G System

Key Issue#1.1.1 Control Signalling for Non-access stratum functionalities

1. Study the support for control signalling for 6G System, including whether and how to enable the introduction of new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1: 	It is assumed that this key issue covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet 1, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

NOTE 3: 6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signaling interaction between RAN-CN need coordination and alignment with RAN3.

Key Issue#1.1.2 Generic mechanisms for UE to Core Network interaction

2. Study the support for control signalling for 6G System, including whether and how to support generic mechanisms (e.g., service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 1: This KI1.1 bullet 2 can include any transport mechanism such as using NAS or UP or new plane. KI1.1 bullet 2 can have dependency on other key issues defined for e.g., WT 1.1 bullet 1, WT3, WT4, WT5, WT6.

NOTE 2: Minimizing interdependency of NFs within 6G CN to support 6G System procedures is to be considered, while considering also the signalling efficiency of e2e communication.

NOTE 3: 6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signaling interaction between RAN-CN need coordination and alignment with RAN3.

NOTE 4: Interworking and migration aspects are covered by key issues for WT#2. Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2.

NOTE 5: This KI 1.1.2 bullet 2 does not cover services that are not operator services.

**** End of Changes ****