| TSG SA WG3 #9, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | S3-99513 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Helsinki, 07- 09 December 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 33.102 |  |  |  |  |  | CR |  |  | XXXr1 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA#9 |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  | x | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 1999-12-03 |  |  |  |  |  |
| Subject: |  |  | Sequence Counter (SEQ) Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | Release 99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | An implementation proposal for a mechanism intended to be implemented internally in a network entity is not needed from an standardisation point of view since global interoperability should be achieved with external requirements/specification. Proposal for internal implementation can limit manufacturers and operators for improvement in its products. The requirements on SQN generation mechanism should be clearly defined in the TS in order to ensure the interoperability. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 6.3.2, 6.3.3, 6.3.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

### 6.3.2	Distribution of authentication data from HE to SN

The purpose of this procedure is to provide the SN/VLR with an array of fresh authentication vectors from the user’s HE to perform a number of user authentications.

Figure 5: Distribution of authentication data from HE to SN/VLR

The SN/VLR invokes the procedures by requesting authentication vectors to the HE/AuC.

The authentication data request shall include a user identity. If the user is known in the SN/VLR by means of the IMUI, the authentication data request shall include the IMUI. However, if the user is identified by means of an encrypted permanent identity (see 6.2), the HLR-message from which the HE can derive the IMUI is included instead. In that case, this procedure and the procedure user identity request to the HLR are integrated.

Upon the receipt of the authentication data request from the SN/VLR, the HE may have pre-computed the required number of authentication vectors and retrieve them from the HLR database or may compute them on demand. The HE/AuC sends an authentication response back to the SN/VLR that contains an ordered array of n authentication vectors AV(1..n).

Figure 6 shows the generation of an authentication vector AV by the HE/AuC.

Figure 6: Generation of an authentication vector

The HE/AuC starts with generating a fresh sequence number SQN and an unpredictable challenge RAND.

For each user the HE/AuC keeps track of a counter: SQNHE

To generate a fresh sequence number, the counter is incremented and subsequently the SQN is set to the new counter value.

The HE has some flexibility in the management of sequence numbers, but some requirements need to be fulfilled by the mechanism used:

- The generation mechanism shall allow a re-synchronisation procedure in the HE described in section 6.3.5

- The SQN should be generated in such way that it does not expose the identity and location of the user.

- In case the SQN may expose the identity and location of the user, the AK may be used as an anonymity key to conceal it.

- The generation mechanism shall allow protection against wrap around the counter in the USIM.

An authentication and key management field AMF is included in the authentication token of each authentication vector. Example uses of this field are included in Annex F.

Subsequently the following values are computed:

-	a message authentication code MAC = f1K(SQN || RAND || AMF) where f1 is a message authentication function;

-	an expected response XRES = f2K (RAND) where f2 is a (possibly truncated) message authentication function;

-	a cipher key CK = f3K (RAND) where f3 is a key generating function;

-	an integrity key IK = f4K (RAND) where f4 is a key generating function;

-	an anonymity key AK = f5K (RAND)  where f5 is a key generating function.

Finally the authentication token AUTN =  SQN Å AK || AMF || MAC is constructed.

.

Note 1:	The need for f5 to use a long-term key different from K is ffs.

Note 2:	The requirements on f3, f4 and f5 are ffs.

Note 3:	It is also ffs in how far the functions f1, ..., f5 need to differ and how they may be suitably combined.

### 6.3.3	Authentication and key agreement

The purpose of this procedure is to authenticate the user and establish a new pair of cipher and integrity keys between the SN/VLR and the MS. During the authentication, the user verifies the freshness of the authentication vector that is used.

Figure 7: Authentication and key establishment

The SN/VLR invokes the procedure by selecting the next unused authentication vector from the ordered array of authentication vectors in the VLR database.  The SN/VLR sends to the user the random challenge RAND and an authentication token for network authentication AUTN from the selected authentication vector.

Upon receipt the user proceeds as shown in Figure 8.

Figure 8: User authentication function in the USIM

Upon receipt of RAND and AUTN the user first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Next the user computes XMAC = f1K (SQN || RAND || AMF) and compares this with MAC which is included in AUTN. If they are different, the user sends user authentication reject back to the SN/VLR with an indication of the cause and the user abandons the procedure.

Next the user verifies that the received sequence number SQN is in the correct range.

The USIM keeps track of a counter: SQNMS.

To verify that the sequence number SQN is in the correct range, the USIM compares SQN with SQNMS. If SQN > SQNMS the MS considers the sequence number to be in the correct range and subsequently sets SQNMS to SQN.

The MS have some flexibility in the management of sequence numbers.

If the user considers the sequence number to be not in the correct range, he sends synchronisation failure back to the SN/VLR including an appropriate parameter, and abandons the procedure.

The synchronisation failure message contains the parameter RANDMS || AUTS. 
Here RANDMS is the random value stored on the MS which was received in user authentication request causing the last update of SQNMS . 
It is AUTS  = Conc(SQNMS ) || MACS.
Conc(SQNMS) = SQNMS  f5K(RANDMS) is the concealed value of the counter SQNMS in the MS, and. 
MACS = f1*K(SQNMS || RAND || AMF) where RAND is the random value received in the current user authentication request. 
f1* is a message authentication code (MAC) function with the property that no valuable information can be inferred from the function values of f1* about those of f1, ... , f5 and vice versa.

The AMF used to calculate MACS assumes a dummy value of all zeros so that it does not need to be transmitted in the clear in the re-synch message.

The construction of the parameter AUTS in shown in the following Figure 9:

Figure 9: Construction of the parameter AUTS

If the sequence number is considered to be in the correct range however, the user computes RES = f2K (RAND) and includes this parameter in a user authentication response back to the SN/VLR. Finally the user computes the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND). Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND. The MS stores RAND for re-synchronisation purposes.

Upon receipt of user authentication response the SN/VLR compares RES with the expected response XRES from the selected authentication vector. If XRES equals RES then the authentication of the user has passed. The SN/VLR also selects the appropriate cipher key CK and integrity key IK from the selected authentication vector.

Conditions on the use of authentication information by the SN/VLR: Using the procedures described in subsections 6.3.1, 6.3.2 and 6.3.4, authentication vectors will have to be used in the specific order in which they were generated, otherwise the user will reject the authentication attempt. The SN/VLR shall use an authentication vector only once and, hence, shall send out each user authentication request RAND || AUTN only once no matter whether the authentication attempt was successful or not. A consequence is that authentication vectors cannot be reused. When a user changes from one VLR to another one and the new VLR requests remaining authentication vectors from the old VLR (cf. subsection 6.3.4) then the old VLR shall not retain any copies of these authentication vectors. When a VLR receives a “cancel location” request for a certain user it shall delete all authentication vectors relating to that user.  When a VLR receives a location update request from a user and the VLR notices that authentication vectors relating to that user are still stored in the VLR it will delete this information and request fresh authentication vectors from the HE/AuC.

## 6.3.3.1	Cipher key selection

Because of the separate mobility management for CS and PS services, the USIM establishes cipher keys with both the CS and the PS core network service domains. The conditions on the use of these cipher keys in the user and control planes are given below.

##### 6.3.3.1.1	User plane

The CS user data connections are ciphered with the cipher key CKCS established between the user and the 3G CS core network service domain and identified in the security mode setting procedure. The PS user data connections are ciphered with the cipher key CKPS established between the user and the 3G PS core network service domain and identified in the security mode setting procedure.

##### 6.3.3.1.2	Control plane

When a security mode setting procedure is performed, the cipher/integrity key set by this procedure is applied to the signalling plane, what ever core network service domain is specified in the procedure. This may require that the cipher/integrity key of an (already ciphered/integrity protected) ongoing signalling connection is changed. This change should be completed within five seconds.

### Re-synchronisation procedure

An SN/VLR may send two types of authentication data requests to the HE/AuC, the (regular) one described in subsection 6.3.2 and one used in case of synchronisation failures, described in this subsection.

Upon receiving a synchronisation failure message from the user, the SN/VLR sends an authentication data request with a “synchronisation failure indication” to the HE/AuC, together with the parameters

-	RAND sent to the MS in the preceding user authentication request and

-	RANDMS || AUTS received by the SN/VLR in the response to that request, as described in subsection 6.3.3.

An SN/VLR will not react to unsolicited “ synchronisation failure indication” messages from the MS.

The SN/VLR does not send new user authentication requests to the user before having received the response to its authentication data request from the HE/AuC (or before it is timed out).

When the HE/AuC receives an authentication data request with a “synchronisation failure indication” it acts as follows: The HE/AuC verifies AUTS by computing f5K(RANDMS), retrieving SQNMS from Conc(SQNMS) and verifying MACS (cf. subsection 6.3.3.). If the verification is successful, but SQNMS is such that SQNHE is not in the correct range then the HE/AuC adapts the value of the counter SQNHE to SQNMS so that the next SQN generated is accepted by the MS. 
Otherwise, the HE/AuC leaves SQNHE unchanged.

In all cases the HE/AuC sends an authentication data response with a new batch of authentication vectors to the SN/VLR. If the counter SQNHE was not adapted then these authentication vectors can be taken from storage, otherwise they are newly generated after adapting SQNHE. In order to reduce the real-time computation burden on the HE/AuC, the HE/AuC may also send only a single authentication vector in the latter case.

Whenever the SN/VLR receives a new batch of authentication vectors from the HE/AuC in an authentication data response it deletes the old ones for that user in the VLR.

The user may now be authenticated based on a new authentication vector from the HE/AuC.

Optionally, in order to minimise extra effort by the HE/AuC, in an authentication data request with synchronisation failure indication the SN/VLR may also send the concealed sequence number Conc(SQNSN) corresponding to the last authentication vector received which the SN/VLR has in storage, i.e. it may send Conc(SQNSN) = RANDSN || SQNSNf5K(RANDMS).

On receipt the HE/AuC retrieves SQNSN from Conc(SQNSN). If the counter in the HE/AuC did not have to be reset and if SQNSN = SQNHE the HE/AuC informs the SN/VLR accordingly and does not send fresh authentication vectors. (In this way, a synchronisation failure does not cause the HE/AuC to produce extra authentication vectors when they are not needed.)

Figure 12 shows how re-synchronisation is achieved by combining a user authentication request answered by a synchronisation failure message (as described in subclause 6.3.3) with an authentication data request with synchronisation failure indication answered by an authentication data response (as described in this subclause).

MS

SN

HE

User auth request (RAND || AUTN)

synch failure (RANDMS || AUTS)

Auth data  request (id), 
 synch failure ind (RAND || RANDMS || AUTS)

Auth data response (auth vectors)

Figure 12: Re-synchronisation procedure

# Annex C: Management of sequence numbers

This annex is devoted to the management of sequence numbers for the authentication and key agreement protocol.

# C.5	A mechanism using two individual counters on each side offering protection against wrap around of counters

The basic idea of the alternative sequence number handling is that the MS will not accept arbitrary jumps in sequence numbers. The sequence number SQN is accepted by the MS if and only if the following holds for some :

SQN > SQNMS (as for alternative C.1) and SQN - SQNMS <  .

This means that SQNMS can reach its maximum value only after a minimum of SQNmax/ successful authentications have taken place.

Conditions on   :

(1)	 shall be sufficiently large so that the MS will not receive any  SQN with SQN - SQNMS   if the HE/AuC functions correctly.

(2)	SQNmax / shall be sufficiently large to prevent that SQNMS ever reaches SQNmax during the lifetime of the USIM.