3GPP TSG-SA3 Meeting #100e 	S3-201551

- e-meeting, 17 -28 August 2020

- Source:	KPN

- Title:	New solution for KI2: Encrypted storage of LTK in UDR

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is requested that SA3 approves this solution based on encryption for protecting the storage of the long term key in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

A new solution is proposed for the protection of the long term key during storage in the UDR. The proposal is to use a an approved symmetric-key algorithm where the secret encryption/decryption key is shared between the UDM/ARPF and the source of the long term key (e.g. the party provisioning USIMs). This would result a strong enough protection mechanism that most likely corresponds to current practices for many operators. The absence of a decryption key in the UDR provides additional protection against unauthorized retrieval of long term keys from the UDR.

The solution allows the possibility of having multiple decryption/encryption keys by using a key identifier for identifying the appropriate key.

# 4	Detailed proposal

START OF CHANGE 1

## 7.x	Solution #<x>: Encrypted storage of the long-term key in the UDR

### 7.x.1	Introduction

In order to protect the long-term key during storage in the UDR, the long-term key is provisioned and stored in encrypted form in the UDR. During primary authentication, the UDM/ARPF retrieves the authentication subscription data for the UE which includes the encrypted long-term key as provisioned and stored in UDR. That is, the long-term key is never provided by the UDR in clear text and there is no need for the UDR to decrypt the long-term key.

At generation of a long-term key an encryption key is used that is shared with the UDM/ARPF where it is decrypted during primary authentication. The generation of a long-term key can be performed in the network of an operator or it can be performed at the facility where USIMs are being provisioned.

### 7.x.2	Solution details

The long-term key is provisioned and stored in encrypted form if stored in the UDR. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

Editor’s note: the encryption algorithm used and the method for the handling of the encryption/decryption key(s) is FFS.

7.x.3	Evaluation

Editor's note: to be provided.

END OF CHANGE 1