- 3GPP TSG-SA3 Meeting #100-e 	draft_S3-201717-r2

- e-meeting, 17 – 28 August 2020	Revision of S3-20xxxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0009 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on error response handling in AKMA process |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2020-07-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 6.2, it defines that the AAnF provides AKMA keys to the AF. However, the scenario for error response is not given in the context. The case that KAKMA is not available in AAnF is because the Kakma and A-TID is deleted due to run a fresh primary authentication. If KAKMA is not available, the AAnF shall continue with step 4 and send an error response. How to handle the error response is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the error case and add subsequent description for error response handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The case that the KAKMA is not available is not clear. The handling of the error response is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE***

### 6. 2	Deriving AKMA Application Key for a specific AF

…

2.	If the AF does not have an active context associated with the A-KID, then the AF sends a Naanf_AKMA_AFKey request to AAnF with the A-KID to request the AKMA Application Key for the UE. The AF also includes its identity (AF Id) in the request. The AAnF shall authorize AF. The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information or policy provided by the NEF/NRF using the AF Id. If succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF can check whether the subscriber is authorized to use AKMA by the presence of the AKMA anchor key K_AKMA that has been received from the AUSF.

If the AAnF is in possession of the AKMA Application Key (KAF), it responds to the AF with the KAF. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the A-KID.

If KAKMA is available in AAnF, the AAnF shall continue with step3.

If KAKMA is not available, the AAnF shall continue with step 4 and send an error response.

3.	The AAnF derives the AKMA Application Key (KAF) from KAKMA.

The key derivation of KAF shall be performed using the key derivation function (KDF) specified in TS 33.220 [4]. KAF is computed (as per clause A.4) as KAF=KDF (KAKMA, AF_ID), where the AF_ID is constructed as follows: AF_ID = FQDN of the AF || Ua* security protocol identifier. The Ua* security protocol identifier is specified as Ua security protocol identifier in Annex H of TS 33.220 [4]. The key used for the derivation of KAF is KAKMA.

4.	The AAnF sends Naanf_AKMA_AFKey response to the AF with KAF and lifetime.

5.	The AF response the Application Session Establishment request to the UE. If the information in step 4 indicates failure of AKMA key request, the AF shall reject the Application Session Establishment by including a failure cause. Afterwards, UE may trigger a new Application Session Establishment request with the latest A-KID to the AKMA AF.

*** END OF CHANGES***