3GPP TSG-SA3 Meeting #102-e 	S3-210071

- e-meeting, 18th - 29th January 2021

- Source:	KPN

- Title:	Updated Solution #22: Representation of identities during broadcast

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested to approve this update to Solution #22: Representation of identities during broadcast

# 2	References

[1]	3GPP TR 33.847 Study on security aspects of enhancement for proximity based services in the 5G System (5GS), version 0.3.0

# 3	Rationale

This contribution updates Solution #22 (previously called: 'Masked representation of identities') in order to resolve the editor's notes in the introduction and the solution details. As a consequence of the resolution of the editor's note on the need for having a Temporary Broadcast Identifier, the title of the solution has been revised to 'Representation of identities during broadcast'.

The following changes have been made:

- The Tempoary Broadcast Identifier has been removed from the solution.

- Instead of Masked representation (of the TBI), the solution now refers to a Representation of the UE identity.

- The applicability and working of the solution in out-of-coverage scenarios has been described.

- Examples/explanations are given (in the form of Notes) about the assignment/construction of Codes and Filters.

# 4	Detailed proposal

START OF CHANGE 1

## 6.22	Solution #22: Representation of identities during broadcast

### 6.22.1	Introduction

This solution addresses Key Issue #11, UE identity protection during ProSe discovery.

The solution is based on the idea to use representations of the UE identity during announcing (i.e. broadcasting) and monitoring (i.e. filtering). Using a representation ensures that the UE cannot be traced and it cannot be impersonated.

The solution is described using the framework and terminology of ProSe Direct Discovery as described in TS 23.303 [5], i.e. for Model A discovery, the terms Announcing UE and Monitoring UE are used, and for Model B discovery, the terms Discoverer UE and Discoveree UE are used.

ProSE Direct Discovery can work in out-of-coverage scenarios provided the appropriate codes and filters have been provided before the start of the actual discovery process through a connection with the 5GC. The connection with the 5GC can be direct or via a relay.

6.22.2	Solution details

#### 6.22.2.1 	Solution for Model A

The solution for Model A consists of the following components:

- 	The use of a Discovery Code (DC), representing the UE identity, that is broadcast by the Announcing UE for announcing its presence and its identity;

-	The use of a Discovery Filter (DF), that can be used by Monitoring UEs for recognizing and identifying the Announcing UE.

The corresponding information flows are described in Figure 6.22.2.1-1 for the Announcing UE and in Figure 6.22.2.1-2 for the Monitoring UE.

Figure 6.22.2.1-1 Direct discovery flows for Announcing UE

The steps performed by the Announcing UE during direct discovery are as follows, see also Figure 6.22.2.1-1.

1.	At the start of direct discovery the Announcing UE requests and receives from the 5GC a Discovery Code (DC) which is a representation of the UE identity;

NOTE 1:	It is implementation dependent how the 5GC assigns a DC to a particular UE; in its simplest form it can be (pseudo) random number; it also can be a hash applied to a UE identitier. If two distinct UEs are assigned the same DC, they will be indistinguishable during ProSe discovery.

2.	The Announcing UE broadcasts its DC so that it can be discovered by other UEs; in case of restricted discovery only authorized UEs should be able to use the DC they receive.

Figure 6.22.2.1-2 Direct discovery flows for Monitoring UE

The steps performed by the Montoring UE during direct discovery are as follows, see also Figure 6.22.2.1-2.

1.	At the start of direct discovery the Monitoring UE requests and receives from the 5GC a Discovery Filter (DF) which is a representation of the Discovery Code broadcast by Announcing UEs that can be used to check for a match;

NOTE 2:	The Discovery Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

2.	The Monitoring UE uses the DF in order to try to match it against any received DCs; in case of a match it has discovered the Announcing UE that it looked for.

#### 6.22.2.2 Solution for Model B

The solution for Model B consists of the following components:

- 	The use of a Query Code (QC), that is broadcast by the Discoverer UE for announcing its query in search for (one or more) Discoveree UEs; the Query Code both represents the (properties of the) Discoveree UE and the Discoverer UE;

-	The use of a Query Filter (QF), that can be used by Discoveree UEs for recognizing the query sent by the Discoverer UE; it also can be used to identity the Discoverer UE (and based on that authorize the discovery of the Discoveree UE);

-	The use of a Response Code (RC), that is broadcast by Discoveree UE for announcing its response to the query received from the Discoverer UE; the Response Code also represents the Discoveree UE;

-	The use of a Response Filter (RF), that can be used by Discoverer UEs for recognizing the response sent by the Discoveree UE; it also can be used to identify the Discoveree UE.

The corresponding information flows are described in Figure 6.22.2.2-1 for the Discoverer UE and in Figure 6.22.2.2-2 for the Discoveree UE.

Figure 6.22.2.2-1 Direct discovery flows for Discoverer UE

The steps performed by the Discoverer UE during direct discovery are as follows, see also Figure 6.22.2.2-1.

1.	At the start of direct discovery the Discoverer UE requests and receives from the 5GC a Query Code (QC) and a Response Filter (RF); the Query Code is a representation of the identity of the Discoverer UE and of the identity of the Discoveree UEs that it wants to discover; the Response Filter is a representation of the Response Code broadcast by Discoveree UEs that can be used to check for a match;

NOTE 1:	The Response Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

2.	The Discoverer UE broadcasts its QC so that it can be received by the Discoveree UEs that it looks for;

3.	The Discoverer UE uses the RF in order to try to match it against any received RCs; in case of a match it has discovered a Discoveree UE that it looked for.

Figure 6.22.2.2-2 Direct discovery flows for Discoveree UE

The steps performed by the Discoveree UE during direct discovery are as follows, see also Figure 6.22.2.2-2.

1.	At the start of direct discovery the Discoveree UE request and receives from the 5GC a Query Filter (QF) and a Response Code (RC); the Query Filter is representation of the Query Code broadcast by Discoverer UEs that can be used to check for a match; the Response Code is a representation of the identity of the Discoveree UE;

NOTE 2:	The Query Filter is constructed in the same way as has been described for LTE based ProSe (e.g. in clause 4.6.4.2a of TS 23.303 [5]).

2.	The Discoveree UE uses the QF in order to try to match it against any received QCs; in case of a match it has received a query from an interested Discoverer UE;

3.	The Discoveree UE broadcasts its RC so that it can be received by the interested Discoverer UE.

### 6.22.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

END OF CHANGE 1